| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 |  |  | Document: JCTVC-U0064 |
| --- | --- | --- | --- |
| Title: | CE1-related: Palette coding with inter-prediction |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu1, Kai Zhang1, Xianguo Zhang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Weijia Zhu +86-10-5690-0888 ext. 60409 {weijia.zhu, kai.zhang, xianguo.zhang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a new palette mode named inter-palette, which combines the palette mode and the inter mode directly. When a CU is coded as the inter-palette mode, the motion information is signalled in the same say as the inter mode with SIZE2Nx2N. Thus a prediction block can be generated by motion compensation. And samples in the CU are coded in the same way as the palette mode, except that an additional copy method ‘COPY_INTER’ is appended besides existing INDEX and COPY_ABOVE. If a sample chooses COPY_INTER, it will copy the value of the corresponding sample in the prediction block directly. Experimental results reportedly show that the segmental prediction method can achieve 2.6%, 1.8% and 0.9% BD-rate savings under AI, RA, and LB conditions, respectively with constrained intra block copy searching; and 2.0%, 1.3% and 0.7% BD-rate savings under AI, RA, and LB conditions, respectively with full-frame intra block copy searching for “RGB, text & graphics with motion, 1080p & 720p sequences”.

# 1. Introduction

In the current HEVC screen content coding extensions draft specification [1], palette mode was adopted. As illustrated in Fig. 1, samples in a CU coded as the palette mode are represented by indices refereeing to a palette. A palette index can be coded with two modes: 'INDEX' and 'COPY_ABOVE', indicated by a flag. With the 'INDEX' mode, the palette index is explicitly signalled. With the 'COPY_ABOVE' mode, the palette index of the sample in the row above is copied. For both 'INDEX' and 'COPY_ABOVE' modes, a run value is signalled to specify the number of subsequent samples that are coded using the same mode.

# 2. The proposed method

Although the current palette index coding method can reduce redundancy in a block well, it cannot exploit non-local duplicated contents in screen pictures as depicted in Fig.2. To address this problem, a new palette mode named inter-palette is proposed.

When a CU is coded as the inter-palette mode, the motion information is signalled in the same say as the inter mode with SIZE2Nx2N. Thus a prediction block can be generated by motion compensation. And samples in the CU are coded in the same way as the palette mode, except that an additional copy method ‘COPY_INTER’ is appended besides existing INDEX and COPY_ABOVE. If a sample chooses COPY_INTER, it will copy the value of the corresponding sample in the prediction block directly. As in INDEX and COPY_ABOVE modes, a run value is also signalled to specify the number of subsequent samples that are coded with COPY_INTER. An illustration of the proposed scheme is shown in Fig.3. In addition, index redundancy removal is handled by the methods in CE1 A.1 and CE1 A.2.

At encoder, the inter-palette mode is checked in the rate-distortion optimization (RDO) process.  The motion information used by inter-palette can be gathered during the existing inter mode decision process. So no additional motion estimation burden is required.

| Figure 1 Coding of palette indices |  |
| --- | --- |

Figure 2 Non-local repeated patterns in screen contents

Figure 3 An illustration of the proposed scheme

# 3. Simulation results

The proposed method is implemented on SCM 4.0 [2] and simulations are carried out under the SCC common test condition [3]. Table 1 - Table 9 show the results for lossy and lossless coding under different conditions. The complete test results are attached in the proposal package. The runtime complexity in table 6-9 is not accurate since the anchor and test are performed by different clusters.

Table 1.  Lossy coding efficiency (SCM4.0 vs. Inter-PLT IBC 1x4)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.6% | -2.6% | -2.6% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.7% | -0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.4% | -2.4% | -2.7% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -0.9% | -1.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.8% | -1.7% | -1.7% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.5% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.6% | -1.6% | -1.6% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.6% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.9% | -0.9% | -0.8% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.1% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.7% | 0.0% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.3% | -0.5% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.5% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2.  Lossy coding efficiency (SCM4.0 vs. Inter-PLT IBC full)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.0% | -1.9% | -2.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.8% | -1.8% | -2.0% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.3% | -1.2% | -1.3% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.2% | -1.1% | -1.2% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.5% | -0.8% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.7% | -0.6% | -0.6% |
| RGB, mixed content, 1440p & 1080p | 0.2% | -0.1% | 0.1% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.6% | -0.2% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.4% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 3.  Lossless coding efficiency (SCM4.0 vs. Inter-PLT)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.9% | -3.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.9% | -1.3% | -3.9% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.6% | -2.3% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -1.0% | -2.8% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.5% | -1.8% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.8% | -2.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 4.  Lossy coding efficiency for YUV420 sequences(SCM4.0 vs. Inter-PLT)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.2% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p & 768p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.2% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | 0.7% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 5.  Lossless coding efficiency for YUV420 sequences (SCM4.0 vs. Inter-PLT)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.4% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.2% | 0.1% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.2% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 6.  Lossy coding efficiency (SCM4.0 vs. CE1 A.1 + Inter-PLT IBC full)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.1% | -3.2% | -3.2% |
| RGB, mixed content, 1440p & 1080p | -0.7% | -1.0% | -1.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -3.6% | -3.6% | -3.8% |
| YUV, mixed content, 1440p & 1080p | -0.9% | -1.5% | -1.7% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.0% | -2.0% | -2.1% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.8% | -0.7% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -2.0% | -2.1% | -2.4% |
| YUV, mixed content, 1440p & 1080p | -0.7% | -1.2% | -1.6% |
| YUV, Animation, 720p | -0.1% | -0.4% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.5% | -1.3% | -1.3% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.4% | -0.3% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -1.5% | -1.1% | -1.1% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.7% | -0.9% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.3% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |

Table 7.  Lossy coding efficiency (SCM4.0 vs. CE1 A.2 + Inter-PLT IBC full)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.0% | -3.0% | -3.1% |
| RGB, mixed content, 1440p & 1080p | -0.7% | -1.0% | -1.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -3.3% | -3.3% | -3.6% |
| YUV, mixed content, 1440p & 1080p | -1.0% | -1.6% | -1.8% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.0% | -2.0% | -2.1% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.7% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -2.0% | -2.0% | -2.4% |
| YUV, mixed content, 1440p & 1080p | -0.7% | -1.1% | -1.6% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | -0.2% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.5% | -1.4% | -1.3% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -1.3% | -1.0% | -0.8% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.5% | -0.9% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.2% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 8.  Lossless coding efficiency (SCM4.0 vs. CE1 A.1 + Inter-PLT)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.9% | -1.3% | -3.9% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.3% | -1.7% | -5.0% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.8% | -2.8% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -1.2% | -3.5% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.6% | -2.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -1.0% | -2.7% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 9.  Lossless coding efficiency (SCM4.0 vs. CE1 A.2 + Inter-PLT)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.9% | -1.2% | -3.6% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.2% | -1.6% | -4.6% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.7% | -2.5% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -1.1% | -3.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.6% | -1.7% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.9% | -2.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

# 4. References

- R. Joshi, S. Liu, J. Xu, Y. Ye “HEVC Screen Content Coding Draft Text 3,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1005, Feb. 2015.

- SCM-4.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu “Common test conditions for screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1015, Feb. 2015.

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).