| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0065 |
| --- | --- |

| Title: | CE2-related: Intra block copy searching constraints for reducing worst case bandwidth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Tzu-Der Chuang +886-3-5670766 ext. 24602 { peter.chuang, ching-yeh.chen, yuwen.huang, shawmin.lei }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HEVC-SCC, the IntraBC predictor comes from the unfiltered reconstructed pixels of the current picture. The unfiltered reconstructed pixels are required to be outputted which increases the worst case memory bandwidth for HEVC-SCC. In this contribution, the worst case bandwidth of HEVC and HEVC-SCC is analyzed. Two IntraBC encoder searching constraints are proposed to reduce the worst case bandwidth. In Method-1, a current picture is divided into 8×8 blocks. An 8×8 block cannot be referenced by IntraBC blocks if this 8×8 block is coded as an 8×8 bi-predicted block, two 4×8/8×4 uni-predicted blocks, or it is belongs to a 16×16 CU which is coded by 4×16+12×16 bi-predicted blocks or 16×4+16×12 bi-predicted blocks, and this 8×8 block is not IntraBC coded block and the use_integer_mv_flag is 0. In method-2, an additional constraint is added that if one MV in this 8×8 block is integer MV, this 8×8 block is allowed to be referenced by IntraBC blocks. With the proposed methods, the worst case bandwidth of the HEVC-SCC is the same as the worst case bandwidth of the HEVC. Experimental results reportedly show that for method-1, no BD-rate losses are shown under AI configuration, 0.5% to 0.6% BD-rate losses are shown under RA configuration, and 0.7% to 1.0% BD-rate losses are shown under LB configuration. For method-2, no BD-rate losses are shown under AI configuration, 0.3% to 0.4% BD-rate losses are shown under RA configuration, and 0.2% to 0.6% BD-rate losses are shown under LB configuration.

# Problem statement

In HEVC Screen Content Coding extension, the Intra Block Copy (IntraBC) mode is introduced to improve the coding efficiency. In IntraBC, the unfiltered reconstructed pixels (before deblocking) of the current picture are used to predict the current PU. The unfiltered reconstructed pixels of the current picture are usually stored in the off-chip memory. The additional write data bandwidth is required for IntraBC mode. In JCTVC-R0188 [1], the worst case memory bandwidth of HEVC and HEVC-SCC is evaluated. The bandwidth per pixel is calculated by the eq. (1). It assumes that memory are accessed (read and write) based on the memory blocks. Only integer number of blocks can be fetched. M and N are the PU width and height. m and n are the memory block width and height. L is the interpolation filter length. The L is 8 for inter prediction, and is 1 for IntraBC and outputting the filtered and unfiltered reconstructed pixels. For an M×N block, the average number of pixels fetched from the memory per pixel can be calculated by eq. (1). Since the outputted block is aligned with the memory block grid, the P of outputting the filtered or the unfiltered reconstructed pixels is 1.

(1)

Table 1 shows the worst case bandwidth requirements of different PU in different size of memory configuration for inter motion compensation. The worst case bandwidth of inter prediction compensation is the 8×8 bi-predicted blocks. However, in some memory configurations, the bandwidth of 4×8 uni-predicted blocks is also the worst case bandwidth.

Table 1. Worst case memory bandwidth of different PU in different memory configuration

| Memory configuration | 8x8 bi-pred. | 8x4 uni-pred. | 4x8 uni-pred. | 16x16 bi-pred. | 16x4 bi-pred. + 16x12 bi-pred. | 4x16 bi-pred. + 12x16 bi-pred. |
| --- | --- | --- | --- | --- | --- | --- |
| 4×1 | 9.375 | 6.875 | 7.5 | 5.03125 | 6.5625 | 7.1875 |
| 4×2 | 10 | 7.5 | 8 | 5.25 | 7 | 7.5 |
| 8×1 | 11.25 | 8.25 | 11.25 | 5.75 | 7.5 | 10.0625 |
| 8×2 | 12 | 9 | 12 | 6 | 8 | 10.5 |
| 16×1 | 15 | 11 | 15 | 8.625 | 11.25 | 14.375 |

Table 2 shows the bandwidth requirements of different PU in different size of memory configuration for inter motion compensation with outputting filtered and unfiltered pixels. Both memory read and write are taken into account in the bandwidth calculation. The worst case bandwidth of HEVC is shown in the category of “With outputting filtered pixels”. The worst case bandwidth of HEVC-SCC is shown in the category of “With outputting filtered and unfiltered pixels”. In HEVC-SCC, the bandwidth requirements that larger than the worst case bandwidth of HEVC is shown in red number. The worst case bandwidth of the HEVC-SCC is 6.3%-9.6% larger than the worst case bandwidth of the HEVC in different memory configuration.

Table 2. Worst case memory memory bandwidth of different PU in different memory configuration, with outputting the filtered and unfiltered pixels

|  | With outputting filtered pixels (bandwidth in HEVC) |  |  | With outputting filtered and unfiltered pixels (bandwidth in HEVC-SCC) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Memory configuration | 8×8 bi-pred. | 4×8 uni-pred. | 4×16 bi-pred. + 12×16 bi-pred. | 8×8 bi-pred. | 4×8 uni-pred. | 4×16 bi-pred. + 12×16 bi-pred. |
| 4×1 | 10.375 | 8.5 | 9.1875 | 11.375 | 9.5 | 10.1875 |
| 4×2 | 11 | 9 | 8.5 | 12 | 10 | 9.5 |
| 8×1 | 12.25 | 12.25 | 11.0625 | 13.25 | 13.25 | 12.0625 |
| 8×2 | 13 | 13 | 11.5 | 14 | 14 | 12.5 |
| 16×1 | 16 | 16 | 15.375 | 17 | 17 | 16.375 |

In JCTVC-R0188 method-1, it is restricted that the any reference pixel for Intra BC block cannot be from bi-predicted blocks. Thus, the unfiltered reconstructed pixels are not needed to be written to the memory, and the worst-case memory bandwidth can be reduced. However, in Table 2, we found that the 4×8 uni-predicted blocks use the same amount of memory bandwidth as 8×8 bi-predicted blocks. If the unfiltered reconstructed pixels of 4×8 uni-predicted blocks are outputted, the worst case bandwidth of the HEVC-SCC will be larger than the worst case bandwidth of the HEVC.

# Proposed method

From the Table 2, it shows that if the the unfiltered reconstructed pixels of 8×8 bi-predicted blocks, 4×8 uni-predicted blocks and 4×16 bi-predicted blocks + 12×16 bi-predicted blocks are not outputted, the worst-case memory bandwidth of HEVC-SCC will be the same as HEVC. Considering sometimes the 1×4, 2×4, 1×8, or 2×8 memory configuration will be used, the the unfiltered reconstructed pixels of 8×4 uni-predicted blocks and 16×4 bi-predicted blocks + 16×12 bi-predicted blocks should not be outputted as well. Moreover, for a inter mode PU, if the MV is integer MV, e.g. an IntraBC coded for the use_integer_mv_flag is 1, the bandwidth with outputting the filtered and reconstructed pixels for all kinds of PU is smaller than the worst case bandwidth of HEVC. Therefore, for the inter PU with integer MV, its unfiltered reconstructed pixels are outputted. In this contribution, two IntraBC encoder searching constraints are proposed to reduce the worst case bandwidth of HEVC-SCC.

## Method 1

In method-1, it is restricted that the any reference pixel for Intra BC block cannot be from 8×8 bi-predicted blocks, 4×8 uni-predicted blocks, 8×4 uni-predicted blocks, 4×16 bi-predicted blocks + 12×16 bi-predicted blocks, and 16×4 bi-predicted blocks + 16×12 bi-predicted blocks. In encoder, a current picture is divided into 8×8 blocks. For each 8×8 block, if it is coded as an 8×8 bi-predicted block, two 4×8 uni-predicted blocks, two 8×4 uni-predicted blocks, or it is belongs to a 16×16 CU which is coded by 4×16 bi-predicted block + 12×16 bi-predicted block or 16×4 bi-predicted block + 16×12 bi-predicted block, it cannot be referenced by IntraBC blocks. However, if one PU in this 8×8 block is IntraBC block or the use_integer_mv_flag is 1, this 8×8 block is allowed to be referenced by IntraBC blocks. if an 8×8 block is not allowed to be referenced by IntraBC blocks, its unfiltered reconstructed pixels are not needed to be outputted. In addition, the current CTB and the previous CTU (the left CTU) are always considered as available for IBC prediction.

## Method 2

In method-2, for each 8×8 block in current picture, the same constraint in method-1 is applied. In addition, if one MV (both MVx and MVy) in this 8×8 block is integer MV, this 8×8 block is allowed to be referenced by IntraBC blocks.

# Experimental results

The proposed method is integrated on SCM-4.0 [2]. All tests were conducted under the common test condition (CTC) [3] for SCC with the full picture IBC search range. Table 1 and Table 2 show the lossy coding results of the proposed method-1 for full-frame IntraBC search and 4-CTU IntraBC search in 444 format, respectively. No BD-rate losses are shown under AI configuration, 0.5% to 0.6% BD-rate losses are shown under RA configuration, and 0.7% to 1.0% BD-rate losses are shown under LB configuration. The lossless results of the proposed method-1 in 444 format are shown in Table 3. The lossy and lossless results of the proposed method-1 in 420 format are shown in Table 4 and Table 5, respectively. Table 6 and Table 7 show the lossy coding results of the proposed method-2 for full-frame IntraBC search and 4-CTU IntraBC search in 444 format, respectively. No BD-rate losses are shown under AI configuration, 0.3% to 0.4% BD-rate losses are shown under RA configuration, and 0.2% to 0.6% BD-rate losses are shown under LB configuration. The lossless results of the proposed method-2 in 444 format are shown in Table 8. The lossy and lossless results of the proposed method-2 in 420 format are shown in Table 9 and Table 10, respectively. The runtimes are not accurate.

Table 11 and Table 12 summarize the memory bandwidth measurements of the proposed method-1 and method-2 repectively. The measurements were carried out as described in the code experiment definition document [4].

# Conclusions

This contribution proposes two IntraBC encoder searching constraints to reduce the worst case bandwidth. With the proposed methods, the worst case bandwidth of the HEVC-SCC will be the same as the worst case bandwidth of the HEVC. Experimental results reportedly show that for method-1, no BD-rate losses are shown under AI configuration, 0.5% to 0.6% BD-rate losses are shown under RA configuration, and 0.7% to 1.0% BD-rate losses are shown under LB configuration. For method-2, no BD-rate losses are shown under AI configuration, 0.3% to 0.4% BD-rate losses are shown under RA configuration, and 0.2% to 0.6% BD-rate losses are shown under LB configuration.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C. Pang, K. Rapaka, J. Sole, X. Li, T. Hsieh, M. Karczewicz, “Non-SCCE1: Memory bandwidth reduction for Intra block copy,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0188, June 2014.

- SCM-4.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1015, Feb. 2015.

- K. Rapaka, C. Gisquet, J. Lainema, X. Xu, “Description of Core Experiment 2 (CE2): Intra block copy memory access,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1102, Feb. 2015.

Table 1.  Lossy results of the proposed method-1 in 444 format and full frame Intra Block Copy search

Table 2. Lossy results of the proposed method-1 in 444 format and constrained Intra Block Copy seaerch (1x4 CTUs)

Table 3.  Lossless results of the proposed method-1 in 444 format and full frame Intra Block Copy search

Table 4.  Lossy results of the proposed method-1 in 420 format and full frame Intra Block Copy search

Table 5. Lossless results of the proposed method-1 in 420 format and full frame Intra Block Copy search

Table 6.  Lossy results of the proposed method-2 in 444 format and full frame Intra Block Copy search

Table 7. Lossy results of the proposed method-2 in 444 format and constrained Intra Block Copy seaerch (1x4 CTUs)

Table 8.  Lossless results of the proposed method-2 in 444 format and full frame Intra Block Copy search

Table 9.  Lossy results of the proposed method-2 in 420 format and full frame Intra Block Copy search

Table 10. Lossless results of the proposed method-2 in 420 format and full frame Intra Block Copy search

Table 11. Average and maximum memory bandwidth of the proposed method-1 with respect to the anchor

Table 12. Average and maximum memory bandwidth of the proposed method-2 with respect to the anchor