| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0066 |
| --- | --- |

| Title: | CE1-related: Row-based copy pixel from neighbouring CU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang1, Yu-Chen Sun1, Jungsun Kim2, PoLin Lai2, Yi-Wen Chen1, Shan Liu2, Yu-Wen Huang1, Shawmin Lei1 1 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | Tzu-Der Chuang +886-3-5670766 ext. 24602 { peter.chuang, yc.sun, jungsun.kim, polin.lai, yiwen.chen, shan.liu, yuwen.huang, shawmin.lei }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a row-based copy pixel from neighbouring CU is proposed to reduce the complexity of CE1 Test A1 and A2 while maintaining the coding gains. A NumCopyPixelRow is first signalled to indicate the number of rows copied from the neighbouring CU. The rest of rows in the CU are coded by the original palette index map coding in SCM-4.0. Experiment results show that, compared with SCM-4.0 with full-frame IntraBC search, 1.4%, 0.9%, and 1.0% BD-rate savings are shown for “YUV, text & graphics with motion, 1080p & 720p sequences” under AI, RA, and LB, respectively; 1.8%, 1.3%, and 1.0 % BD-rate savings are shown for “RGB, text & graphics with motion, 1080p & 720p sequences” under AI, RA, and LB, respectively.

# Introduction

In CE1 Test A1 and A2 [1][2], it was proposed for palette mode to use the nearest row of the above CU or the nearest column of the left CU to predict colour indexes of the current CU. In palette index map coding, the copy-above mode is allowed in first row. The palette index parsing process is modified when the previous coded run mode is copy above mode and the above sample is copied from the neighbouring CU. In this proposal, a simplified version of CE1 Test A1 and A2 is proposed to reduce the complexity while maintaining the coding gains of CE1 Test A1 and A2.

# Proposed method

In this contribution, the extended copy above mode to the first row in CE1 Test A1 and A2 is processed in row-based. The extended copy above mode to the first row can be only used for the first sample. The number of samples whose values are copied from the neighbouring CU is restricted to be multiple of the CU width. A NumCopyPixelRow is first signalled to indicate the number of rows copied from the neighbouring CU. For the rest of samples, the palette index map coding in SCM-4.0 is used except the starting position is changed. For example, if the NumCopyPixelRow is 0, no sample is predicted from the neighbouring CU, as shown in Figure 1 (a). It is the same as the SCM-4.0 palette mode coding. The starting position is the upper-left sample (the dark-red sample). The copy-above mode is not allowed in the first row (the red row). If the NumCopyPixelRow is 3, the first three rows are predicted from the neighbouring CU, as shown in Figure 1 (b). The palette mode coding in SCM-4.0 is used for the rest rows, and the starting row is the 4th row. The traverse scan order is still applied so the starting position is the left most sample (the dark-red sample). The copy-above mode is not allowed in the first normal row, which is the 4th row (the red row). No sample is predicted from the neighboring CU after the 4th row.

In the proposed method, there is no sample that the previous coded run mode is copy above mode and the above sample is copied from the neighbouring CU. The palette index parsing process is the same as SCM-4.0. Compared with the CE1 Test A1 and A2, the parsing complexity is reduced.

The context formation and binarization for NumCopyPixelRow is the same as the paletteRun coding of the copy above mode. If the neighboring CU is not valid, e.g. the picture boundary, the pixel values of the first palette color (the palette index equal to 0) is used to replace the unavailable neighboring samples.

Figure 1. Examples of the proposed row-based copy pixel from neighbouring CU. 
(a) NumCopyPixelRow = 0. (b) NumCopyPixelRow = 3

# Experimental results

The proposed method is integrated on SCM-4.0 [3]. All tests were conducted under the common test condition (CTC) [4] for SCC with the full picture IBC search range. Table 1 and Table 2 show the lossy coding results of the proposed row-based copy pixel from neighbouring CU for full-frame IntraBC search and 4-CTU IntraBC search in 444 format respectively. For RGB and YUV text & graphics with motion sequences in full-frame IntraBC search, 1.4% and 1.8% BD-rate gains are shown under AI configuration, 0.9% and 1.3% BD-rate gains are shown under RA configuration, and 1.0% and 1.0% BD-rate gains are shown under LB configuration. The lossless results of the proposed method in 444 format are shown in Table 3. The lossy and lossless results of the proposed method in 420 format are shown in Table 4 and Table 5 respectively.

# Conclusions

This contribution proposes simplified version of version of CE1 Test A1 and A2. The row-based copy pixel from neighbouring CU is proposed. A NumCopyPixelRow is first signalled to indicate the number of rows copied from the neighbouring CU. The rest of rows in the CU are coded by the original palette index map coding in SCM-4.0. Experimental results reportedly show 1.4% and 1.8% gains for RGB and YUV text & graphics with motion sequences under AI configuration in lossy condition, 0.9% and 1.3% gains for RGB and YUV text & graphics with motion sequences under RA configuration in lossy condition, 1.0% and 1.0% gains for RGB and YUV text & graphics with motion sequences under LB configuration in lossy condition.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-C. Sun, J. Kim, T.-D. Chuang, S. Liu, Y.-W. Chen, Y.-W. Huang, S. Lei, V. Seregin, F. Zou, W. Pu, R. Joshi, M. Karczewicz, “CE1 : Test A.1 : Extended copy above mode to the first line with index adjustment bits,” JCTVC-U0061, June 2015.

- Y.-C. Sun, J. Kim, T.-D. Chuang, P. Lai, S. Liu, Y.-W. Chen, Y.-W. Huang, S. Lei, “CE1 : Test A.2 : Extended copy above mode to the first line with grouping the palette_run_type_flag in front,” JCTVC- U0062, June 2015.

- SCM-4.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1015, Feb. 2015.

Table 1.  Lossy results of the proposed method in 444 format and full frame Intra Block Copy search

Table 2. Lossy results of the proposed method in 444 format and constrained Intra Block Copy seaerch (1x4 CTUs)

Table 3.  Lossless results of the proposed method in 444 format and full frame Intra Block Copy search

Table 4.  Lossy results of the proposed method in 420 format and full frame Intra Block Copy search

Table 5. Lossless results of the proposed method in 420 format and full frame Intra Block Copy search