| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 Jun. 2015 | Document: JCTVC-U0076 |
| --- | --- |

| Title: | On unification of motion vector resolution for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Shan Liu Shawmin Lei | Email: | xiaozhong.xu@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In SCM-4.0, it is intended to unify the intra block copy operation with inter coding. However, a discrepancy exists in which the motion vectors pointing to the current reference picture are always integer-pel resolution while motion vectors pointing to temporal reference pictures can switch between integer-pel resolution and quarter-pel resolution at slice level. This contribution proposes to unify the resolutions of all motion vectors in the same slice. When the slice level use_integer_mv_flag is equal to 1, all motion vectors in this slice are integer-pel resolution; when this flag is equal to 0, all motion vectors in this slice are quarter-pel resolution. In particular, if the current picture is the only one reference picture available for the current slice, the use_integer_mv_flag for this slice is signalled or inferred to be 1. Experimental results report average 0.0 to 1.0% B-D rate increase for RA and LB 444 lossy configurations by applying the proposed unification method. The B-D rate change for AI is negligible.

# Introduction

In the current SCM, the intra block copy (IBC) operation has been mostly unified with inter mode [1]. Two differences between these two coding tools are: (1) the resolution of motion vector that points to current picture (referred as block vector) and motion vector that points to a temporal reference picture can be different; (2) the storage handling of decoded vector is different for these two types of vectors. In Figure 1, the diagrams for decoded motion vector that points to current picture and decoded motion vector that points to a temporal reference picture are shown. Detailed differences are listed as follows:

- For the IBC part, the vectors are always at integer-pel resolution. After decoding, they are stored at quarter-pel resolution (left shifted by two). When used for vector prediction, the predictor is at integer resolution (right shifted by two). When used for interpolation and deblocking, they are at quarter-pel resolution.

- For the temporal reference picture part, the resolution of decoded MVs in one slice are depending on the use_integer_mv_flag:

  - When use_integer_mv_flag is equal to 1, the decoded MVs are always at integer-pel resolution. After decoding, they are stored at integer-pel resolution. When used for MV prediction and deblocking, they are at integer-pel resolution. When used for interpolation, they are at quarter-pel resolution (left shifted by two).

  - When use_integer_mv_flag is equal to 0, the decoded MVs are always at quarter-pel resolution. After decoding, they are stored and used at quarter-pel resolution.

Figure 1: parsing process for block vector and motion vector

The diagrams of the parsing process for these two types of vectors are shown in Figure 1. In order to fully unify the operations of IBC and inter mode in this regard, the existing differences between the two need to be handled properly.

# Proposed methods

Two methods are proposed for the resolution unification between IBC and HEVC inter.

In the proposed methods, the IBC vector operations are unified with inter mode. To be specific, when IBC mode is enabled at SPS level, for a given slice, the resolution of motion vectors in this slice should be controlled by the use of integer motion vector flag (use_integer_mv_flag). If use_integer_mv_flag is equal to 1, the vectors in this slice use integer-pel resolution; otherwise, the vectors should be at quarter-pel resolution. The handling operations of decoded motion vector that points to current picture follow those in inter mode. In particular, when the current picture is the only reference picture for this slice, in method 1 the use_integer_mv_flag flag is inferred to be 1; or in method 2, this flag is enforced to be signalled as 1. The diagram of the proposed method is shown in Figure 2.

Figure 2: proposed parsing process for motion vector of current picture and motion vector of other temporal pictures

With the proposed methods, the decoded vector resolution and operations are fully unified for IBC reference picture and all other reference pictures.

# Experimental results

HM-16.2+ SCM-4.0 default coding structure is used as the anchor. The common test conditions specified in Error: Reference source not found are followed. The summarized experimental results of method 1 for 444 lossy and lossless coding are shown in Table 1 and 2, respectively. The summarized experimental results of method 2 for 444 lossy and lossless coding are shown in Table 3 and 4, respectively. The summarized experimental results of method 1 for 420 lossy and lossless coding are shown in Table 5 and 6, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for 444 lossy coding in method 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 132% |  |  |
| Dec Time[%] | 128% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.8% | 0.8% | 0.9% |
| RGB, mixed content, 1440p & 1080p | 0.4% | 0.4% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 1.0% | 1.0% | 1.0% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 0.4% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 126% |  |  |
| Dec Time[%] | 120% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.7% | 0.7% | 0.8% |
| RGB, mixed content, 1440p & 1080p | 0.5% | 0.5% | 0.5% |
| RGB, Animation, 720p | 0.0% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 1.0% | 0.8% | 0.9% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 0.3% | 0.5% |
| YUV, Animation, 720p | 0.0% | 0.3% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 125% |  |  |
| Dec Time[%] | 118% |  |  |

Table 2 Experimental results for 444 lossless coding in method 1

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.5% | 0.2% | 0.0% | 1.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.6% | 0.2% | 0.0% | 1.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.5% | 0.2% | 0.0% | 1.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.5% | 0.2% | 0.0% | 1.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 3 Experimental results for 444 lossy coding in method 2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.8% | 0.8% | 0.9% |
| RGB, mixed content, 1440p & 1080p | 0.4% | 0.4% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 1.0% | 1.0% | 1.0% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 0.4% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.7% | 0.7% | 0.8% |
| RGB, mixed content, 1440p & 1080p | 0.5% | 0.5% | 0.5% |
| RGB, Animation, 720p | 0.0% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 1.0% | 0.8% | 0.9% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 0.3% | 0.5% |
| YUV, Animation, 720p | 0.0% | 0.3% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 3 Experimental results for 444 lossless coding in method 2

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.5% | 0.2% | 0.0% | 1.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.6% | 0.2% | 0.0% | 1.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.5% | 0.2% | 0.0% | 1.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.5% | 0.2% | 0.0% | 1.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

Table 7 Experimental results for 420 lossy coding in method 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p & 768p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 1.0% | 0.9% | 1.0% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 0.4% | 0.3% |
| YUV, Animation, 720p | 0.2% | 0.2% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 80% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 1.0% | 0.7% | 1.0% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.1% | 0.4% | 0.3% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 95% |  |  |

Table 8 Experimental results for 420 lossless coding in method 1

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.3% | 0.2% | 0.0% | 0.6% |
| Mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 96% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.3% | 0.1% | 0.0% | 0.5% |
| Mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

# Conclusions

Unification of vector resolution and handling operations for decoded MV of current picture and MV of other reference picture is proposed in this contribution. With the proposed methods, The MV resolution and usage in IBC mode are unified with MV in inter mode.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C. Pang, Y.-K. Wang, V. Seregin, K. Rapaka, M. Karczewicz, X. Xu, S. Liu, S. Lei, B. Li, J. Xu, “Non-CE2: Intra block copy and inter signalling unification”, Document of Joint Collaborative Team on Video Coding, JCTVC- T0227, Geneva, CH, 10–18 Feb. 2015.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1015, Geneva, CH, 10–18 Feb. 2015.