# Text modifications

The text provided in this section is based on JCTVC-T1005.

Text modifications for the proposed escape pixel coding method are in yellow.

In method A.1,

#### 8.5.3.2Derivation process for motion vector components and reference indices

##### 8.5.3.2.1General

……

It is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >> 2 ) − 2 * ( subWidthC − 1 ) , ( yPb + mvLX[ 1 ]  >>  2 ) − 2 * ( subHeightC − 1) ) as inputs, the output shall be equal to TRUE.

- When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2) + nPbW – 1 + 2 * ( subWidthC − 1 ), yPb + (mvLX[ 1 ]  >>  2) + nPbH – 1 + 2 * ( subHeightC – 1 )  ) as inputs, the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ]  >>  2 ) + nPbW + xB1 + 2 * ( subWidthC – 1 )  is less than or equal to 0.

- The value of ( mvLX[ 1 ]  >>  2 ) + nPbH + yB1+ 2 * ( subHeightC – 1 )   is less than or equal to 0.

- The following condition shall be true:

( xPb + ( mvLX[ 0 ]  >>  2 ) + nPbSw − 1 + 2 * ( subWidthC – 1 )  ) / CtbSizeY − xCurr / CtbSizeY  <=  yCurr/CtbSizeY − ( yPb + ( mvLX[ 1 ]  >>  2 ) + nPbSh – 1 + 2 * ( subHeightC −1 ) ) / CtbSizeY	(8-xx)

……

##### 8.5.3.2.10Derivation process for chroma motion vectors

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

……

For the derivation of the chroma motion vector mvCLX, the following applies:

–	If the picture with index refIdx from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-1)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-2)

–	Otherwise,

mvCLX[ 0 ] = ( ( mvLX[ 0 ] >> ( 1 + SubWidthC ) ) * 8	(8-xxx)

mvCLX[ 1 ] = ( ( mvLX[ 1 ] >> ( 1 + SubHeightC ) ) * 8	(8-xxx)

In method A.2,

#### 8.5.3.2Derivation process for motion vector components and reference indices

##### 8.5.3.2.1General

……

It is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- The variables xRef and yRef are derived as follows:

xRef = ( mvLX[0] % ( 1 << (1+SubWidthC) ) == 0 | | xPb + (mvLX[ 0 ]  >>  2) < 2 * (SubWidthC - 1) ) ?  ( xPb + (mvLX[ 0 ]  >>  2) ) :

( xPb + (mvLX[ 0 ]  >>  2) - 2 )   	       (8-XX)

yRef = ( mvLX[1] % (1 << (1+SubHeightC) ) == 0 | | yPb + (mvLX[ 1 ]  >>  2) < 2 * (SubHeightC - 1) ) ?  ( yPb + (mvLX[ 1 ]  >>  2) ) :

( yPb + (mvLX[ 1 ]  >>  2) - 2 )   	       (8-XX)

When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2), (yPb + mvLX[ 1 ]  >>  2) )(xRef, yRef) as inputs, the output shall be equal to TRUE.

- The variables xRef and yRef are modified as follows:

xRef = ( mvLX[0] % ( 1 << (1+SubWidthC) ) == 0 | | xPb + (mvLX[ 0 ]  >>  2 + nPbW – 1 ) > pic_width_in_luma_samples – 1– 2 * (SubWidthC - 1) ) ?  ( xPb + (mvLX[ 0 ]  >>  2) + nPbW - 1 ) : ( xPb + (mvLX[ 0 ]  >>  2) + nPbW + 1 )   	       (8-XX)

yRef = ( mvLX[1] % (1 << (1+SubHeightC) ) == 0 | | yPb + (mvLX[ 1 ]  >>  2 + nPbH – 1 ) > pic_height_in_luma_samples – 1 – 2 * (SubHeightC - 1) ) ?  ( yPb + (mvLX[ 1 ]  >>  2) + nPbH - 1 ) : ( yPb + (mvLX[ 1 ]  >>  2) + nPbH + 1 )   	       (8-XX)

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2) + nPbW − 1, yPb + (mvLX[ 1 ]  >>  2) + nPbH – 1 )(xRef, yRef) as inputs, the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ]  >>  2 ) + nPbW + ( (mvLX[0] % (1 << (1+SubWidthC) ) == 0) ? 0 : 2 ) + xB1 is less than or equal to 0.

- The value of ( mvLX[ 1 ]  >>  2 ) + nPbH + ( (mvLX[1] % (1 << (1+SubHeightC) ) == 0) ? 0 : 2 ) + yB1 is less than or equal to 0.

- The following condition shall be true:

( xPb + ( mvLX[ 0 ]  >>  2 ) + nPbSw – 1)xRef  / CtbSizeY − xCurr / CtbSizeY  <=
		yCurr/CtbSizeY − ( yPb + ( mvLX[ 1 ]  >>  2 ) + nPbSh − 1 )yRef / CtbSizeY	(8-3)

……

##### 8.5.3.2.10Derivation process for chroma motion vectors

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

……

For the derivation of the chroma motion vector mvCLX, the following applies:

–	If the picture with index refIdx from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-4)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-5)

–	Otherwise,

mvCLX[ 0 ] = ( ( mvLX[ 0 ] >> ( 1 + SubWidthC ) ) * 8	(8-xxx)

mvCLX[ 1 ] = ( ( mvLX[ 1 ] >> ( 1 + SubHeightC ) ) * 8	(8-xxx)

In method B.1,

##### 8.5.3.2.10Derivation process for chroma motion vectors

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

–	a luma motion vector mvLX,

–	the reference indices refIdxL0 and refIdxL1.

Output of this process is a chroma motion vector mvCLX.

A chroma motion vector is derived from the corresponding luma motion vector.

For the derivation of the chroma motion vector mvCLX, the following applies:

–	If the picture with index refIdx from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-6)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-7)

–	Otherwise,

mvCLX[ 0 ] = ( ( mvLX[ 0 ] >> ( 1 + SubWidthC ) ) * 8	(8-xxx)

mvCLX[ 1 ] = ( ( mvLX[ 1 ] >> ( 1 + SubHeightC ) ) * 8	(8-xxx)

##### 8.5.3.3.3Fractional sample interpolation process

###### 8.5.3.3.3.1General

Inputs to this process are:

……

–	The prediction sample value predSamplesLXCb[ xC ][ yC ] is derived by invoking the process specified in subclause  with ( xIntC, yIntC ), ( xFracC, yFracC ), (xPb / SubWidthC,  yPb / SubHeightC ), and refPicLXCb as inputs.

–	The prediction sample value predSamplesLXCr[ xC ][ yC ] is derived by invoking the process specified in subclause  with ( xIntC, yIntC ), ( xFracC, yFracC ), (xPb / SubWidthC,  yPb / SubHeightC ), and refPicLXCr as inputs.

###### 8.5.3.3.3.3Chroma sample interpolation process

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in eighth fractional-sample units ( xFracC, yFracC ),

–	a chroma location in full-sample units ( xPbC, yPbC ),

–	the chroma reference sample array refPicLXC.

……

In Error: Reference source not found, the positions labelled with upper-case letters Bi, j within shaded blocks represent chroma samples at full-sample locations inside the given two-dimensional array refPicLXC of chroma samples. These samples may be used for generating the predicted chroma sample value predSampleLXC. The locations ( xBi, j, yBi, j ) for each of the corresponding chroma samples Bi, j inside the given array refPicLXC of chroma samples are derived as follows:

xBi, j = Clip3( 0, ( pic_width_in_luma_samples / SubWidthC ) − 1, xIntC + i )	(8-8)

yBi, j = Clip3( 0, ( pic_height_in_luma_samples / SubHeightC ) − 1, yIntC + j )	(8-9)

If the reference picture is current picture,

- If (xBi, j, yBi, j) is not available, xBi, j is set to be = xBi-1, j

- If (xBi, j, yBi, j) is not available, yBi, j is set to be yBi, j-1.

In method B.2,

# Decoding process

## General decoding process

….

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found specify the following decoding processes using syntax elements in the slice segment layer and above:

…

–	When curr_pic_as_ref_enabled_flag is equal to 1, a picture storage buffer in the DPB is allocated for the current picture, and the current picture is marked as "used for long-term reference". For each of the samples in the current picture, the sample value of luma component is initialized as 1<< ( bit_depth_luma_minus8 + 7 ); if chroma_format_idc is not equal to 0, the sample value of chroma component is initialized as 1<< ( bit_depth_chroma_minus8 + 7 ).

…

##### 8.5.3.2.10Derivation process for chroma motion vectors

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

–	a luma motion vector mvLX,

–	the reference indices refIdxL0 and refIdxL1.

Output of this process is a chroma motion vector mvCLX.

A chroma motion vector is derived from the corresponding luma motion vector.

For the derivation of the chroma motion vector mvCLX, the following applies:

–	If the picture with index refIdx from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-10)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-11)

–	Otherwise,

mvCLX[ 0 ] = ( ( mvLX[ 0 ] >> ( 1 + SubWidthC ) ) * 8	(8-xxx)

mvCLX[ 1 ] = ( ( mvLX[ 1 ] >> ( 1 + SubHeightC ) ) * 8	(8-xxx)

##### 8.5.3.3.3Fractional sample interpolation process

###### 8.5.3.3.3.1General

Inputs to this process are:

……

–	The prediction sample value predSamplesLXCb[ xC ][ yC ] is derived by invoking the process specified in subclause  with ( xIntC, yIntC ), ( xFracC, yFracC ), (xPb / SubWidthC,  yPb / SubHeightC ), and refPicLXCb as inputs.

–	The prediction sample value predSamplesLXCr[ xC ][ yC ] is derived by invoking the process specified in subclause  with ( xIntC, yIntC ), ( xFracC, yFracC ), (xPb / SubWidthC,  yPb / SubHeightC ), and refPicLXCr as inputs.

###### 8.5.3.3.3.3Chroma sample interpolation process

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in eighth fractional-sample units ( xFracC, yFracC ),

–	a chroma location in full-sample units ( xPbC, yPbC ),

–	the chroma reference sample array refPicLXC.

……

xBi, j = Clip3( 0, ( pic_width_in_luma_samples / SubWidthC ) − 1, xIntC + i )	(8-12)

yBi, j = Clip3( 0, ( pic_height_in_luma_samples / SubHeightC ) − 1, yIntC + j )	(8-13)

If the reference picture is current picture, and if (xBi, j, yBi, j) is not available, (xBi, j, yBi, j) is set to be ( xPbC, yPbC ).