| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 Jun. 2015 | Document: JCTVC-U0077 |
| --- | --- |

| Title: | On chroma motion vector derivation for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Shan Liu Shawmin Lei | Email: | xiaozhong.xu@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In SCM-4.0, it is intended to unify intra block copy (IntraBC) with HEVC inter coding. However, the resolution of block vectors (i.e. motion vectors pointing to current picture) is always integer-pel, for both luma and chroma components, while the resolution of motion vectors pointing to temporal reference pictures can switch between integer-pel and quarter-pel at slice level. With this difference, the derivation process of chroma motion vectors for IntraBC blocks is different from that for blocks coded with HEVC inter mode. This contribution proposes several methods to resolve this difference and unify the chroma motion vector derivation procedure. In method A, an extra few rows and columns of pixels are restricted from being used as reference when a motion vector points to current reference to make sure the surrounding pixels which may be used for interpolation are all available. In method B, the allowed range of block vectors remain the same as SCM-4.0, instead extra rows and columns beyond the right and bottom boundaries of the reference area are padded by boundary or prefixed pixel values. Experimental results show that:

- The proposed method A.1 reports no change to 444 coding; it reports average 2.3%, 1.2% and 0.8% BD-rate increase for RGB TGM 1080p & 720p in AI, RA and LB 420 lossy coding, respectively;

- The proposed method A.2 reports no change to 444 coding; it reports average 2.8%, 1.6% and 0.8% BD-rate saving for RGB TGM 1080p & 720p in AI, RA and LB 420 lossy coding, respectively;

- The proposed method B.1 reports no change to 444 coding; it reports average 3.2%, 1.9% and 1.0% BD-rate saving for RGB TGM 1080p & 720p in AI, RA and LB 420 lossy coding, respectively;

- The proposed method B.2 reports no change to 444 coding; it reports average 3.2%, 1.9% and 0.9% BD-rate saving for RGB TGM 1080p & 720p in AI, RA and LB 444 lossy coding, respectively;

# Introduction

In the current SCM, the intra block copy (IBC) operation has been mostly unified with inter mode [1]. One existing difference is that the block vector resolution is always integer-pel and no interpolation is needed. When chroma motion vector is derived from luma motion vector, possible scaling operation may be required due to resolution difference across colour components. In order to maintain the vector resolution at integer-pel for intra block copy in this case, the derivation process for chroma motion vector in intra block copy mode is different from that for inter mode. If this part is unified with inter mode, reference pixel interpolation may be enabled, which requires the availability of neighbouring pixels around the reference block. However, when a motion vector points to a fractional pixel (sub-pel) position inside current picture, the neighboring pixels around the reference block may not be fully available for interpolation, even if the bitstream conformance conditions (1) - (2) are met.

BV_x + nPbSw + xPbS – xCbs <= 0 or

BV_y + nPbSh + yPbS – yCbs <= 0					(1)

( xPb + BV_x + nPbSw − 1 ) / CtbSize – xCbs / CtbSize  <=

yCbs / CtbSize − ( yPb + BV_y + nPbSh − 1 ) / CtbSize				(2)

In intra block copy mode, if a reference block is in fractional-pel position, for each of the fractional pixels in the reference block, besides the nearest integer pixels surrounding it, some other neighbouring integer pixels are also required for the purpose of interpolation. In general, for a symmetric interpolation filter with 2L length of filter taps, at each side of a sub-pel position (either left or right side, top or bottom side), there should be L integer pixels available in both side to interpolate this fractional-pel position. In HEVC, L = 4 for luma pixels and L = 2 for chroma pixels. In Figure 1, an example of an intra block copy interpolation is shown, where the reference block is near the boundary of reference area, on the right and bottom two sides, there are only 1 line available for interpolating this reference block. Interpolation cannot be done for this reference block due to lack of enough available reconstructed pixels on the right and from the bottom sides.

Figure 1: Reference block of intra block copy near boundary location

In order to resolve this issue, two solutions are proposed in this contribution. Details of these solutions are discussed in the next section.

# Proposed methods

## Method A:

From the above analysis, if the filter length is 2L, at least L lines on the right and from the bottom need to be available for interpolation. Therefore, the right and bottom boundaries of the reference area should be move to left and top by (L-1) lines.

The effective constraints in method A is shown in Figure 2 in dotted red lines.

Figure 2: proposed search range constraint for intra block copy in method A

In Method A.1, no change applies to 4:4:4 colour format. For non-4:4:4 colour formats, if the resolutions of luma and chroma components are different in one dimension, the derived chroma vector component for this direction may be at fractional-pel resolution. In this case, the proposed encoder constraints are applied to the search area of this direction. . The effective number of chroma lines to be constrained in this direction is 1. The corresponding number of luma lines will be 2. In addition, if the derived chroma vector points to a fractional-pel position, interpolation applies to generate this reference block.

In Method A.2, the above new constraints apply to fractional chroma vectors only (odd integer luma vector in non-4:4:4 formats). For integer chroma vectors, original constraints apply.

## Method B:

To solve the reference pixel availability issue, if a reference block inside current reference picture is at or near the boundary of the reference area, the unavailable pixels across the boundary are padded.

In method B.1, the unavailable pixels are padded by extending the boundary column and boundary row of the reference area. An example of method B.1 is shown in Figure 3, in which two extra lines on the right and two extra lines at the bottom are extended.

Figure 3: padding integer pixels from available pixels in method B.1

In method B.2, a variation of padding unavailable pixels is that a constant value is always used. In particular, any unavailable pixel inside the current reference picture is set to be (1<< bit_depth ) >> 1 (the value is 128 when bit_depth = 8) for interpolation purpose. For both methods B.1 and B.2, when the reference area boundary is the picture boundary, the HEVC rules apply—pixels at picture boundary will be extended.

# Experimental results

HM-16.2+ SCM-4.0 default coding structure is used as the anchor. The common test conditions specified in Error: Reference source not found are followed. The summarized experimental results of method A.1 for 420 lossy and lossless coding are shown in Table 1 and 2, respectively. The summarized experimental results of method A.2 for 420 lossy and lossless coding are shown in Table 3 and 4, respectively. The summarized experimental results of method B.1 for 420 lossy and lossless coding are shown in Table 5 and 6, respectively. The summarized experimental results of method B.2 for 420 lossy and lossless coding are shown in Table 7 and 8, respectively. More detailed experimental results could be found in the attached spreadsheets. Note that the runtime shown in below tables is not reliable.

Table 1 Experimental results for 420 lossy coding in method A.1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 2.3% | 1.6% | 1.5% |
| YUV, mixed content, 1440p & 1080p | 2.1% | 1.6% | 1.2% |
| YUV, Animation, 720p & 768p | 0.2% | -0.2% | 0.2% |
| Enc Time[%] | 132% |  |  |
| Dec Time[%] | 136% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 1.2% | 0.7% | 0.6% |
| YUV, mixed content, 1440p & 1080p | 1.2% | 0.9% | 0.6% |
| YUV, Animation, 720p | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 135% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 0.8% | 0.2% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.6% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.1% | -0.2% | 0.0% |
| Enc Time[%] | 138% |  |  |
| Dec Time[%] | 119% |  |  |

Table 2 Experimental results for 420 lossless coding in method A.1

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 2.5% | 2.7% | 0.2% | 6.1% |
| Mixed content, 1440p & 1080p | 1.6% | 1.6% | 1.4% | 1.8% |
| Animation, 720p & 768p | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 132% |  |  |  |
| Dec Time[%] | 135% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.8% | 1.3% | 0.2% | 2.8% |
| Mixed content, 1440p & 1080p | 0.3% | 0.3% | 0.2% | 0.5% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 137% |  |  |  |
| Dec Time[%] | 133% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.7% | 0.8% | -0.2% | 1.9% |
| Mixed content, 1440p & 1080p | 0.2% | 0.2% | 0.0% | 0.4% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 137% |  |  |  |
| Dec Time[%] | 137% |  |  |  |

Table 3 Experimental results for 420 lossy coding in method A.2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -2.8% | -3.5% | -3.4% |
| YUV, mixed content, 1440p & 1080p | -0.8% | -1.5% | -1.6% |
| YUV, Animation, 720p & 768p | -0.2% | -0.6% | -0.3% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -1.6% | -2.2% | -2.2% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.9% | -1.1% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 81% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.8% | -1.7% | -1.7% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.7% | -0.4% |
| YUV, Animation, 720p | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 88% |  |  |

Table 4 Experimental results for 420 lossless coding in method A.2

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -0.6% | -0.7% | -2.0% | 0.2% |
| Mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.3% | -0.2% |
| Animation, 720p & 768p | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -1.0% | -0.5% | -2.0% | 0.0% |
| Mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -1.0% | -0.4% | -1.9% | 0.0% |
| Mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

Table 5 Experimental results for 420 lossy coding in method B.1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -3.2% | -3.8% | -3.7% |
| YUV, mixed content, 1440p & 1080p | -0.9% | -1.6% | -1.8% |
| YUV, Animation, 720p & 768p | -0.3% | -0.7% | -0.3% |
| Enc Time[%] | 138% |  |  |
| Dec Time[%] | 136% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -1.9% | -2.4% | -2.4% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.9% | -1.0% |
| YUV, Animation, 720p | -0.1% | -0.4% | -0.2% |
| Enc Time[%] | 123% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -1.0% | -1.6% | -1.6% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -1.1% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.6% | -0.1% |
| Enc Time[%] | 133% |  |  |
| Dec Time[%] | 111% |  |  |

Table 6 Experimental results for 420 lossless coding in method B.1

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -0.7% | -0.8% | -2.1% | 0.1% |
| Mixed content, 1440p & 1080p | -0.2% | -0.3% | -0.3% | -0.2% |
| Animation, 720p & 768p | -0.1% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 134% |  |  |  |
| Dec Time[%] | 131% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -1.0% | -0.6% | -2.0% | -0.1% |
| Mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 156% |  |  |  |
| Dec Time[%] | 138% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -1.1% | -0.5% | -2.0% | -0.1% |
| Mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 131% |  |  |  |
| Dec Time[%] | 131% |  |  |  |

Table 7 Experimental results for 420 lossy coding in method B.2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -3.2% | -3.8% | -3.7% |
| YUV, mixed content, 1440p & 1080p | -0.9% | -1.6% | -1.8% |
| YUV, Animation, 720p & 768p | -0.3% | -0.7% | -0.3% |
| Enc Time[%] | 134% |  |  |
| Dec Time[%] | 132% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -1.9% | -2.3% | -2.5% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -0.8% | -1.1% |
| YUV, Animation, 720p | -0.2% | -0.3% | -0.2% |
| Enc Time[%] | 129% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.9% | -1.5% | -1.7% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.9% | -0.5% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 131% |  |  |
| Dec Time[%] | 116% |  |  |

Table 8 Experimental results for 420 lossless coding in method B.2

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -0.7% | -0.8% | -2.1% | 0.1% |
| Mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.3% | -0.2% |
| Animation, 720p & 768p | -0.1% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 145% |  |  |  |
| Dec Time[%] | 142% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -1.0% | -0.5% | -2.0% | 0.0% |
| Mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 131% |  |  |  |
| Dec Time[%] | 132% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -1.0% | -0.5% | -2.0% | -0.1% |
| Mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 150% |  |  |  |
| Dec Time[%] | 141% |  |  |  |

# Conclusions

In this contribution, two solutions are proposed to unify the chroma motion vector derivation process between intra block copy mode and inter mode. In method A, extra constraints are put on the reference area so that the vector can only point to reference area with all surrounding pixels available; in method B, the unavailable pixels outside the reference area of current reference picture are padded for the interpolation purpose.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C. Pang, Y.-K. Wang, V. Seregin, K. Rapaka, M. Karczewicz, X. Xu, S. Liu, S. Lei, B. Li, J. Xu, “Non-CE2: Intra block copy and inter signalling unification”, Document of Joint Collaborative Team on Video Coding, JCTVC- T0227, Geneva, CH, 10–18 Feb. 2015.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1015, Geneva, CH, 10–18 Feb. 2015.

- X. Xiu, Y. Ye and Y. He, “Intra block copy chroma interpolation for Non-444”, Document of Joint Collaborative Team on Video Coding, JCTVC-U0080, Warsaw, PL, 19–26 Jun. 2015

- X. Xiu, Y. Ye and Y. He, “On chroma derivation of intra block copy for non-444 video”, Document of Joint Collaborative Team on Video Coding, JCTVC-U0103, Warsaw, PL, 19–26 Jun. 2015