| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0079r1 |
| --- | --- |

| Title: | HLS : On intra block copy signaling control |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Vadim Seregin Rajan Joshi Marta Karczewicz Ye-Kui Wang Qualcomm Incorporated Alexis Tourapis Yeping Su David Singer Apple Inc. | Tel: Email: | krapaka@qti.qualcomm.com atourapis@apple.com |
| Source: | Qualcomm Incorporated Apple Inc. |  |  |

_____________________________

# Abstract

During the 20th JCT-VC meeting held in Geneva, the alignment of intra block copy (IBC) with inter signaling, as proposed by JCTVC-T0227, was adopted. In the current working draft (WD), I-slices are disallowed when IBC is enabled (i.e., when curr_pic_as_ref_enabled_flag is equal to 1). On the other hand, support of the IBC mode in B-slices seems to be compromised, and may potentially be problematic, since the current picture is only added as a reference in List0. This contribution proposes to allow I-slices when the IBC mode is enabled, improve support of IBC in B slices, and to provide finer control of IBC at the picture and/or slice level.

# Problem statement

In the current WD, it appears that I-slices are disallowed when IBC is enabled (i.e., when curr_pic_as_ref_enabled_flag is equal to 1). This is essentially implied by constraints and notes introduced in the text that seem to impose the use of Inter slices when this flag is enabled.

Unfortunately, this limitation seems to be with little justification since there could be some complexity, implementation, and possibly other advantages of enabling the use of normal I slices when the IBC mode is enabled. Without conventionally intra coded slices there can also be additional signalling and potentially unnecessary overhead, such as the signaling of the cu_skip_flag and pred_mode_flag in the CU header, and the signaling and processing of the reference picture management/list modification information in a slice header.

The current draft seems to also limit the effectiveness of the IBC mode, in particular when using this in B slices. The current picture is only added as a reference in the List0 reference list, which creates inconsistencies with the processes defined in the reference picture list modification semantics, disallows the use of bi-prediction with or without weights using only the current picture as a reference, and does not appear to permit, implicitly, the use of B slices when the nal_unit_type has a value in the range of BLA_W_LP to RSV_IRAP_VCL23, inclusive.

Another issue relates to the scope of IBC enabling control. Currently, whether IBC is used is solely controlled by the SPS flag curr_pic_as_ref_enabled_flag. However, it is observed that the usage of IBC is content dependent, and there can be scenarios where within a coded video sequence sometimes it is beneficial to use IBC and sometimes it is beneficial to use conventional intra/inter modes. It is asserted that allowing a finer control of IBC, e.g., at the slice/picture level, can have the following benefits:

- The IBC mode requires additional storage of unfiltered samples for reference. With the knowledge of usage of IBC at a slice/picture level, a smart decoder may avoid storing these additional samples thereby reducing the average bandwidth and the average power usage.

- There are cases where the IBC mode might provide minimal to no benefits. For example, we may encounter a completely or quasi-static frame for which all CTUs would most likely utilize temporal prediction, e.g. skip. Penalizing the decoder with maintaining the current picture reference may then be undesirable. In addition, having knowledge of usage or not of the IBC mode at the slice level could potentially allow having more reference indices allocated to temporal reference pictures, without the requirement of signaling picture list modification instructions. This could be beneficial, for example, in signaling more weighted prediction parameters that could improve coding efficiency in certain scenarios, without the additional overhead and processing required through the use of list modification instructions.

# Proposal

## Enabling I-slices when IBC is enabled

It is proposed to enable I-slices when IBC is enabled, i.e., to allow the value the slice_type to be equal to 2 when curr_pic_as_ref_enabled_flag is equal to 1.

Specification changes

Section 7.4.3.2.3

NOTE – When curr_pic_as_ref_enabled_flag is equal to 1, there is no I slice in CVSs referring to the SPS

The proposed change to a constraint in Section 8.3.4 of the spec, provided below in Section 2.3 of this document, is also related.

Some checks to the definitions of intra prediction, I slices, and IRAP picture etc. should be made to make sure the concepts are clearly specified in a consistent manner.

## Picture level IBC control flag and improved B slice support

It is proposed to add a picture level IBC control flag, in the PPS. This flag specifies whether any blocks within the current picture may use the current picture as a reference. The IBC mode requires additional storage of unfiltered samples for reference. With the knowledge of usage of IBC at the picture level, a smart decoder may avoid storing these additional samples for the entire picture thereby reducing the average bandwidth and the average power usage. This parameter has the benefit that no additional parsing of the slices is needed to determine whether IBC will be used for any of the slices that are associated with the current picture.

Proposed semantics/specification changes:

curr_pic_as_ref_enabled_pps_flag equal to 1 specifies that a current picture referring to the picture parameter set may be included in a reference picture list of any slice of the current picture. curr_pic_as_ref_enabled_pps_flag equal to 0 specifies that a current picture referring to the picture parameter set is never included in the reference picture list of any slice of the current picture. When not present, the value of curr_pic_as_ref_enabled_pps_flag is inferred to be equal to 0.

The value of curr_pic_as_ref_enabled_pps_flag shall be equal to 0 when curr_pic_as_ref_enabled_flag is equal to 0.

Section 7.4.7.1

When nal_unit_type has a value in the range of BLA_W_LP to RSV_IRAP_VCL23, inclusive, i.e., the picture is an IRAP picture, nuh_layer_id is equal to 0, and curr_pic_as_ref_enabled_flag curr_pic_as_ref_enabled_pps_flag is equal to 0, slice_type shall be equal to 2.

Section 8.1.3

When curr_pic_as_ref_enabled_flag curr_pic_as_ref_enabled_pps_flag is equal to 1, a picture storage buffer in the DPB is allocated for the current picture, and the current picture is marked as "used for long-term reference".

Syntax change

| pps_scc_extension( ) { | Descriptor |
| --- | --- |
| curr_pic_as_ref_enabled_pps_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| …… |  |
| } |  |

## Slice level IBC control flag

It is proposed to add a slice level IBC control flag. This flag specifies whether blocks within the current slice may use the current picture as a reference. Similarly, with the knowledge of usage of IBC at the picture level, a smart decoder may avoid storing these additional samples for the current slice thereby reducing the average bandwidth and the average power usage, while providing finer control than the picture level parameters. This parameter is used to control processes such as list construction and reordering.

Proposed semantics/specification changes:

curr_pic_as_ref_enabled_slice_flag equal to 1 specifies that the current picture may be included in a reference picture list of the current slice. curr_pic_as_ref_enabled_slice_flag equal to 0 specifies that the current picture is never included in the reference picture list of the current slice. When not present, the value of curr_pic_as_ref_enabled_slice_flag is inferred to be equal to 0. The value of curr_pic_as_ref_enabled_slice_flag shall not be equal to 0 for P and B slices when the decoded picture buffer contains only one reference picture that is the current picture.

Section 7.4.7.2

if(curr_pic_as_ref_enabled_flag curr_pic_as_ref_enabled_pps_flag )
	NumPicTotalCurr++

NOTE – When curr_pic_as_ref_enabled_pps_flag is equal to 1, NumPicTotalCurr will be incremented by 1 even when the current picture has slices with curr_pic_as_ref_enabled_slice_flag equal to 0. However, for these slices with curr_pic_as_ref_enabled_slice_flag equal to 0, there will be no entry in list_entry_lX[ i ] that corresponds to the current picture.

Section 8.1.3

When curr_pic_as_ref_enabled_flag is equal to 1 any entry in the reference lists of the current slice refers to the current picture, the decoded sample values of the current slice before in-loop filtering are stored into the picture storage buffer allocated for the current picture.

Section 8.3.2

The variable NumPicTotalCurr is derived as specified in clause Error: Reference source not found. It is a requirement of bitstream conformance that the following applies to the value of NumPicTotalCurr:

- When the current picture is a BLA or CRA picture, the value of NumPicTotalCurr shall be equal to curr_pic_ref_enabled_flag.

- Otherwise, when the current picture contains a P or B slice, the value of NumPicTotalCurr shall not be equal to 0.

- If the current picture contains a P or B slice, the value of NumPicTotalCurr shall not be equal to 0.

- Otherwise, when the current picture is a BLA or CRA picture and does not contain P or B slices, the value of NumPicTotalCurr shall be equal to 0.

Section 8.3.4

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )	(2-1)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	if( curr_pic_as_ref_enabled_flag curr_pic_as_ref_enabled_slice_flag )
		RefPicListTemp0[ rIdx++ ] = currPic
}

…

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )	(2-2)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	if( curr_pic_as_ref_enabled_slice_flag )
		RefPicListTemp1[ rIdx++ ] = currPic
}

…

It is a requirement of bitstream conformance that when nal_unit_type has a value in the range of BLA_W_LP to RSV_IRAP_VCL23, inclusive (i.e. the picture is an IRAP picture), and curr_pic_as_ref_enabled_flag curr_pic_as_ref_enabled_slice_flag is equal to 1, and slice_type is not equal to 2, all entries in RefPicList0 and RefPicList1, if available, shall contain one or more entries that refer to the current picture only.

Syntax change

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| ……… |  |
| if( !dependent_slice_segment_flag ) { |  |
| …….. |  |
| if( motion_vector_resolution_control_idc = = 2 ) |  |
| use_integer_mv_flag | u(1) |
| if( curr_pic_as_ref_enabled_pps_flag && ( slice_type = = P ¦ ¦ slice_type = = B ) ) |  |
| curr_pic_as_ref_enabled_slice_flag | u(1) |
| ………. |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The parameter list_entry_l1[i] has the same definition, and thus the same range, as list_entry_l0[i]. This results in a conflict with the list construction process.