| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0080r1 |
| --- | --- |

| Title: | On Chroma MV derivation for intra block copy for non-444 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Vadim Seregin Rajan Joshi Marta Karczewicz | Tel: Email: | krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated. |  |  |

_____________________________

# Abstract

During the 20th JCTVC JCT-VC meeting held in Geneva, the alignment of Intra block copy (IBC) with inter signaling, as proposed by JCTVC-T0227, was adopted. In the unification framework, luma and chroma block vectors are restricted to be in integer precision thus the prediction samples are generated without interpolation. However, for non-444 chroma formats, chroma block vectors may point to a fractional samples. For these case, in the current working draft chroma block vectors are rounded to integer pel accuracy. In this contribution it is proposed to enable fractional chroma interpolation for IBC block. In the process the IBC search region is reduced by interpolation samples when interpolation is applied. Compared to SCM 4.0 anchor on 420 sequences the proposed method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI of {2.1%, 2.7%, 27%} for the category YUV text & graphics with motion, 1080p & 720p, respectively. Additional results with encoder optimization to compute SAD based on interpolated chroma reference samples are included. With this optimization. Compared to SCM 4.0 anchor on 420 sequences the proposed method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI of {2.5%, 3.1%, 3.0%} for the category YUV text & graphics with motion, 1080p & 720p, respectively. The spec text changes are highlighted in red and yellow text.

# Proposal

In this contribution, it is proposed to enable fractional chroma interpolation for IBC after removing the rounding of chroma MV as follows:

–	If the picture with index refIdx from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-1)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-2)

–	Otherwise

mvCLX[ 0 ] = ( ( mvLX[ 0 ] >> ( 1 + SubWidthC ) ) * 8	(8-3)

mvCLX[ 1 ] = ( ( mvLX[ 1 ] >> ( 1 + SubHeightC ) ) * 8	(8-4)

However, it can be a case when chroma samples, required for the interpolation, are located outside of current IBC valid region. To prevent such a case, it is proposed to reduce the IBC reference area in a way that all samples, including the ones required for the interpolation, are located within the valid IBC region.

8.5.3.2	Derivation process for motion vector components and reference indices

8.5.3.2.1	General

offsetX =0

offsetY= 0

When ChromaArrayType is not equal to 0, the variable offsetX and offsetY are derived as

offsetX = mvCLX[ 0 ] & 0x7 ? 2 : 0

offsetY = mvCLX[ 1 ] & 0x7 ? 2 : 0

It is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2)  −  offsetX, (yPb + mvLX[ 1 ]  >>  2)  −  offsetY ) as inputs, the output shall be equal to TRUE.

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2) + nPbW − 1  +  offsetX , yPb + (mvLX[ 1 ]  >>  2) + nPbH – 1 )  +  offsetY) as inputs, the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ]  >>  2 ) + nPbW + xB1  +  offsetX is less than or equal to 0.

- The value of ( mvLX[ 1 ]  >>  2 ) + nPbH + yB1  +  offsetY is less than or equal to 0.

- The following condition shall be true:

( xPb + ( mvLX[ 0 ]  >>  2 ) + nPbSw − 1 +  offsetX ) / CtbSizeY − xCurr / CtbSizeY  <=
		yCurr/CtbSizeY − ( yPb + ( mvLX[ 1 ]  >>  2 ) + nPbSh − 1 +  offsetY ) / CtbSizeY

# Test results

420 CTC - Lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -2.1% | -2.7% | -2.6% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -1.2% | -1.2% |
| YUV, Animation, 720p & 768p | -0.1% | -0.5% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -1.3% | -1.8% | -1.8% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.7% | -0.9% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.6% | -1.2% | -1.3% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.8% | 0.2% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.1% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 108% |  |  |

420 CTC – Lossy with encoder optimization

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -2.5% | -3.1% | -3.0% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -1.3% | -1.4% |
| YUV, Animation, 720p & 768p | -0.1% | -0.6% | -0.2% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -1.5% | -1.9% | -2.0% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.6% | -0.8% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.2% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 111% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.8% | -1.4% | -1.6% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -1.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.5% | -0.2% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 106% |  |  |

The lossless 420 results are included in the package. There is no change to 444 CTC.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).