| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0081r1 |
| --- | --- |

| Title: | On unification of adaptive motion vector resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Vadim Seregin Chao Pang Marta Karczewicz Rajan Joshi | Tel: Email: | krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated. |  |  |

_____________________________

# Abstract

During the 20th JCTVC JCT-VC meeting held in Geneva, the alignment of Intra block copy (IBC) with inter signaling, as proposed by JCTVC-T0227, was adopted. Also, in the current working draft (WD) adaptive motion vector resolution can be enabled using the syntax element use_integer_mv_flag. This contribution identifies several issues related motion vector (MV) derivation and storage when IBC and use_integer_mv_flag are enabled. Several solutions are proposed to unification design that resolve the identified problems. The performance impact of proposed solutions seems non-significant ranging from 0.0% to 0.1%. In this version, the text specification to proposed method including full test results are included.

# Problem Statement

In the current design, there are two cases when integer MV is used, IntraBC mode and inter mode with use_integer_mv_flag equal to 1. For these cases, we have the following different design approaches:

- MV left shifted by 2 for motion compensation when use_integer_mv_flag is equal to 1.

- Rounding of AMVP predictor for IntraBC mode.

- Different MV storage accuracy, which has impact on TMVP accuracy and deblocking threshold when use_integer_mv_flag is equal 1. During the deblocking process, MV is compared against the threshold in quarter luma samples.

- Clipping of integer MV to guarantee the HEVC MV range when use_integer_mv_flag is equal 1, and wrap-around of MV for IntraBC mode.

It is desired to have more consistent design to represent integer MV for these two cases.

# Proposal

## MV derivation for IBC mode and inter mode with use_integer_mv_flag equal to 1

It is proposed to make design consistent to represent integer motion vectors for both inter mode with use_integer_mv_flag equal to 1 and IntraBC modes.

### Method 1:

When use_integer_mv_flag is equal to 1, use IntraBC MV derivation process and MV storage accuracy for inter mode. That is, MV = (MVP >> 2 + MVD) << 2 and no rounding of MV (<<2) before MC. This solution will also solve inconsistent de-blocking threshold as discussed in section 2.2 and TMVP accuracy. Also, MV clipping, added for use_integer_mv_flag is equal to 1, to guarantee a valid MV range would not be needed and can be removed.

### Method 2:

When use_integer_mv_flag is equal to 1, use inter MV derivation process and MV storage for IntraBC. That is BV = (MVP + BVD) and upscale BV (<< 2) at MC stage. However, it may still have inconsistent deblocking threshold and TMVP accuracy.

### Method 3:

Regardless of use_integer_mv_flag value, use inter MV derivation process and MV storage of use_integer_mv_flag equal to 1 for IntraBC. That is BV = (MVP + BVD) and upscale BV (<< 2) at MC stage. However, it may still have inconsistent deblocking threshold and TMVP accuracy.

## Inconsistent MV accuracy for deblocking

In the current WD, MV storage accuracy is different for inter mode with use_integer_mv_flag equal to 1 compared to IBC mode and inter mode with use_integer_mv_flag equal to 0. However, in the deblocking process MVs are compared against the threshold which is represented in quarter luma samples.

It is proposed to upscale (<<2) MV for inter mode when use_integer_mv_flag is equal 1 for the deblocking process to match the specification text.

## MV accuracy for inter mode with use_integer_mv_flag equal to 1

In the current WD, the luma and chroma MVs are scaled (<<2) and clipped just before motion compensation when use_integer_mv_flag is equal to 1 as below

mvLX = Clip3( − 215, 215 − 1, mvLX << 2 )	(8-1)

mvCLX = Clip3( − 215, 215 − 1, mvCLX << 2 )		 (8-2)

It is asserted that the scaled motion vectors (luma and chroma) when they are clipped to 215 – 1 result in requiring interpolation. The interpolation is not required for any other cases when use_integer_mv_flag is equal to 1. This special case would add unnecessary complexity for the decoder which it would not have to do otherwise.

It is proposed to clip the motion vectors to nearest value that does not require interpolation.

When use_integer_mv_flag is equal to 1 and the reference index refIdxLX is not equal to currPic, mvLX and mvCLX (where X equals to 0 or 1) are modified as follows:

mvLX = Clip3( − 215, 215 – 4, mvLX << 2 )	(8-3)

mvCLX = Clip3( − 215, 215 − 4, mvCLX << 2 )		 (8-4)

There is not performance change observed under CTC from the above change.

# Test results

## Test results for 2.1.1

## 4:4:4 CTC

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | 0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.2% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

## Test results for 2.1.2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.2% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

## Test results for 2.1.3

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

## Test results for 2.2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.3% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 95% |  |  |

# Text specification

## Text changes for method 2.1.1

- Section 8.5.3.2.1

- When predFlagLX is equal to 1 and the reference picture is the current picture or use_integer_mv_flag is equal to 1, the luma motion vector mvLX is derived as follows:

uLX[ 0 ] = ( ( ( mvpLX[ 0 ] >> 2 ) + mvdLX[ 0 ] )  <<  2 + 216 ) % 216	(8-5)

mvLX[ 0 ] = ( uLX[ 0 ]  >=  215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ] 	(8-6)

uLX[ 1 ] = ( ( mvpLX[ 0 ] >> 2 ) + mvdLX[ 0 ] )  <<  2 + 216 ) % 216	(8-7)

mvLX[ 1 ] = ( uLX[ 1 ]  >=  215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(8-8)

Section 8.5.3.3.1

When use_integer_mv_flag is equal to 1 and the reference index refIdxLX is not equal to currPic, mvLX and mvCLX (where X equals to 0 or 1) are modified as follows:

mvLX = Clip3( − 215, 215 − 1, mvLX << 2 )	(8-9)

mvCLX = Clip3( − 215, 215 − 1, mvCLX << 2 ) 		(8-10)

## Text changes for method 2.1.2

Section 8.5.3.2.1

- When predFlagLX is equal to 1 and the picture with index refIdx from reference picture list LX of the slice is not the current picture or use_integer_mv_flag is equal to 1, the luma motion vector mvLX is derived as follows:

uLX[ 0 ] = ( mvpLX[ 0 ] + mvdLX[ 0 ] + 216 ) % 216	(8-11)

mvLX[ 0 ] = ( uLX[ 0 ]  >=  215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ]	(8-12)

uLX[ 1 ] = ( mvpLX[ 1 ] + mvdLX[ 1 ] + 216 ) % 216	(8-13)

mvLX[ 1 ] = ( uLX[ 1 ]  >=  215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(8-14)

When predFlagLX is equal to 1 and the reference picture is the current picture and use_integer_mv_flag is equal to 0, the luma motion vector mvLX is derived as follows:

uLX[ 0 ] = ( ( ( mvpLX[ 0 ] >> 2 ) + mvdLX[ 0 ] )  <<  2 + 216 ) % 216	(8-15)

mvLX[ 0 ] = ( uLX[ 0 ]  >=  215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ] 	(8-16)

uLX[ 1 ] = ( ( mvpLX[ 0 ] >> 2 ) + mvdLX[ 0 ] )  <<  2 + 216 ) % 216	(8-17)

mvLX[ 1 ] = ( uLX[ 1 ]  >=  215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(8-18)

## Text changes for method 2.1.3

Section 8.5.3.2.1

When predFlagLX is equal to 1 and the picture with index refIdx from reference picture list LX of the slice is not the current picture, the luma motion vector mvLX is derived as follows:

uLX[ 0 ] = ( mvpLX[ 0 ] + mvdLX[ 0 ] + 216 ) % 216	(8-19)

mvLX[ 0 ] = ( uLX[ 0 ]  >=  215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ]	(8-20)

uLX[ 1 ] = ( mvpLX[ 1 ] + mvdLX[ 1 ] + 216 ) % 216	(8-21)

mvLX[ 1 ] = ( uLX[ 1 ]  >=  215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(8-22)

When predFlagLX is equal to 1 and the reference picture is the current picture the luma motion vector mvLX is derived as follows:

uLX[ 0 ] = ( ( ( mvpLX[ 0 ] >> 2 ) + mvdLX[ 0 ] )  <<  2 + 216 ) % 216	(8-23)

mvLX[ 0 ] = ( uLX[ 0 ]  >=  215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ] 	(8-24)

uLX[ 1 ] = ( ( mvpLX[ 0 ] >> 2 ) + mvdLX[ 0 ] )  <<  2 + 216 ) % 216	(8-25)

mvLX[ 1 ] = ( uLX[ 1 ]  >=  215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(8-26)

Section 8.5.3.3.1

When use_integer_mv_flag is equal to 1 and the reference index refIdxLX is not equal to currPic or the reference index refIdxLX is equal to currPic, mvLX and mvCLX (where X equals to 0 or 1) are modified as follows:

mvLX = Clip3( − 215, 215 − 1, mvLX << 2 )	(8-27)

mvCLX = Clip3( − 215, 215 − 1, mvCLX << 2 ) 		(8-28)

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).