| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0083 |
| --- | --- |

| Title: | On indication of the use of current reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chenghao Liu, Shan Liu, Xiaozhong Xu, Shawmin Lei 2860 Junction Ave, San Jose, CA, USA | Email: | {chenghao.liu, shan.liu, xiaozhong.xu, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In working draft of High Efficiency Video Coding (HEVC) extension on Screen Content Coding (SCC), the use of current picture as a reference picture is signalled in sequence parameter set (SPS). This contribution presents methods of signalling the use of current reference picture in picture parameter set (PPS) or slice header.

# Introduction

In the High Efficiency Video Coding (HEVC) Screen Content Coding (SCC): Draft 3 [1], the intra block copy (IBC) prediction is an coding tool for screen content coding, wherein the current picture may be included in a reference picture list of the picture itself and could be used as a reference picture. To indicate whether or not the picture may be included in a reference picture list of the picture itself, a flag named curr_pic_as_ref_enabled_flag presented in the sps_scc_extension as a part of sequence parameter set (SPS) is specified in the current HEVC SCC: Draft 3.

In a scenario of scene changes, for example scene changes between screen content and natural video content, the encoder may want to change the flag of using current picture as reference picture upon detection of the scene changes between natural sequence and screen content sequence. One example application may be in distance teaching where it is expected to change natural scene and screen content scene frequently. In such a scenario, there is a requirement of indicating whether or not inserting current picture in a reference picture list in picture level.

First, it is typical to send a picture parameter set (PPS) to update the parameters but rarely send an SPS in occurrence of a new scene. Second, it wastes quantities of bits to indicate the curr_pic_as_ref_enabled_flag in SPS not only in sending the SPS message itself but also in inserting an IRAP picture as definition of SPS and CVS. The reason is that syntax elements presented in SPS apply to zero or more entire CVSs, each of which starts with an IRAP access unit with NoRaslOutputFlag equal to 1. It is not necessary to insert an IRAP picture in compression efficiency point of view each time scene change is detected because long-term reference picture could be deployed to store reference pictures acrorss different scenes in the scenario where scence changes happen frequently.

In another scenario of picture refresh-based error resilient video coding application, indication of inserting current picture in a reference picture list can be deployed to provide a refreshing functionality. In the picture refresh based error cotrol, the current picture is not included in a reference picture list in most access points, which are not targeted for refreshing, and the current picture is included in a reference picture list in the access points which are targeted for refreshing to stop temporal error propogation and avoid waist bits in Intra coding. For that it is required to provide a picture level of indication of inserting the current picture in a reference picture list of the picture itself.

JCTVC-S0145 [2] proposed to reduce average bandwidth when intra block copy (IBC) mode is used for prediction. The contribution is based on the observation that not all previously unfiltered decoded samples of the current picture are used for prediction in IBC mode. In JCTVC-S0145, it is proposed to indicate which of the previously decoded coded tree blocks (CTBs) are used for IBC prediction. It is proposed to send a flag for each block in a slice segment header to indicate which CTBs need to be stored for reducing average bandwidth when a PPS level flag indicating the presence of the former flags in slice header. In the 19th JCT-VC meeting, it has been questioned that it needs to know how many CTBs are in a slice and needs to send the flags in advance. Furthermore, curr_pic_as_ref_enabled_flag is used to indicate whether the current picture is inserted in a reference picture list and used for reference picture in JCTVC-S0145. JCTVC-T0051 [3] and JCTVC-T0045 [4] proposed to signal CTU level of flag to indicate whether CTB is used for IBC prediction so that either CTBs before in-loop filter or after in-loop filter are stored to avoid additional storage when curr_pic_as_ref_enabled_flag equal to one.

If the flag of indicating inserting the current picture in a reference picture list of the picture itself is present in PPS, then the flag present in the PPS is applied to zero or more entire coded pictures as determined by a syntax element found in each slice segment header as the definition of the PPS. Or alternatively, if the flag of indicating inserting the currrent picture in the picture itself is present in a slice segment header, then the flag present in slice header is applied to the current picture. However, the flag of indicating inserting the current picture in a reference picture list of current picture itself is not present in PPS and not in slice segment header in HEVC SCC: Draft 3.

# Proposed method

This contribution presents two modifications of indicating whether or not the current picture is included in a reference picture list of the picture itself in PPS or slice segment header, respectively.

In the first modification, a syntax element named curr_pic_as_ref_enabled_flag is added in picture parameter set screen content coding extension, which is part of PPS, and curr_pic_as_ref_enabled_flag is removed from sps_scc_extension as follows.

In following, all modifications are based on the latest HEVC SCC: Draft 3.

| sps_scc_extension( ) { | Descriptor |
| --- | --- |
| curr_pic_as_ref_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| … |  |
| } |  |

| pps_scc_extension( ) { | Descriptor |
| --- | --- |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| ... |  |
| curr_pic_ as_ref _enabled_flag | u(1) |
| } |  |

In the first modification, the semantics of curr_pic_as_ref_enabled_flag in pps_scc_extension is modified as compared to curr_pic_as_ref_enabled_flag previously present in sps_scc_extension as follows.

curr_pic_as_ref_enabled_flag equal to 1 specifies a picture referring to the PPSSPS is may be included in a reference picture list of the picture itself. curr_pic_as_ref_enabled_flag equal to 0 specifies that a picture referring to the PPSSPS is never included in any reference picture list of the picture itself. When not present, the value of curr_pic_as_ref_enabled_flag is inferred to be equal to 0.

In the second modification, a syntax element named curr_pic_in_ref_list_flag is added in slice segment header if the curr_pic_as_ref_enabled_flag present in the referred SPS is equal to 1. The curr_pic_in_ref_list_flag present in slice segment headers of a coded picture shall be an identical value either 0 or 1 for all the slice segment headers of the coded picture.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... | u(1) |
| if( !dependent_slice_segment_flag ) { |  |
| ... |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| ... |  |
| } |  |
| if(curr_pic_as_ref_enabled_flag ) |  |
| curr_pic_in_ref_list _flag | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ... |  |
| } |  |
| ... |  |
| } |  |
| ... |  |
| byte_alignment( ) |  |
| } |  |

In the second modification, the semantics of curr_pic_in_ref_list_flag in slice_segment_header is modified as compared to curr_pic_as_ref_enabled_flag previously present in sps_scc_extension as follows.

curr_pic_in_ref_list_flag equal to 1 specifies a the current picture referring to the SPS may be is included in a reference picture list of the current picture itself. curr_pic_in_ref_list_flag equal to 0 specifies that a the current picture referring to the SPS is not never included in any reference picture list of the current picture itself. When not present, the value of curr_pic_in_ref_list_flag is inferred to be equal to 0. The curr_pic_in_ref_list_flag present in slice segment headers of a coded picture shall be an identical value either 0 or 1 for all the slice segment headers of a coded picture.

In the proposed modification 2, decoding process for reference picture lists construction is modified in a way that curr_pic_in_ref_list_flag present in slice segment header is served as a flag of indicating whether or not current picture is included in the reference picture as follows.

### Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

- …

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )	(8-1)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	if( curr_pic_as_ref_enabled_flag curr_pic_in_ref_list_flag)
		RefPicListTemp0[ rIdx++ ] = currPic
}

It is a requirement of bistream conformance that when nal_unit_type has a value in the range of BLA_W_LP to RSV_IRAP_VCL23, inclusive (i.e. the picture is an IRAP picture) and curr_pic_as_ref_enabled_flag curr_pic_ in_ref_list_flag is equal to 1, RefPicList0 shall contain one or more entries that refer to the current picture only.

In the proposed modifications 2, reference picture list modification semantics is modified in a way that curr_pic_in_ref_list_flag present in slice segment header is served as a flag of indicating whether or not NumPicTotalCurr is incremented by one to count the current picture as a reference picture as follows.

#### Reference picture list modification semantics

The variable NumPicTotalCurr is derived as follows:

NumPicTotalCurr = 0
for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++ )
	if( UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
		NumPicTotalCurr++
for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++)	(7-2)
	if( UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		NumPicTotalCurr++
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
	if( UsedByCurrPicLt[ i ] )
		NumPicTotalCurr++
if( curr_pic_as_ref_enabled_flag curr_pic_in_ref_list_flag )
	NumPicTotalCurr++

# Conclusions

Among the two modifications, the authors do not have specific preference of selecting which of the modifications to enable picture level signaling of including the current picture in a reference picture list of the picture itself. No specific drawback is found in the two modifications in terms of harming parallel processing design of HEVC as it does not cause additional dependency between slices. Also it does not cause error resiliency issue.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCTVC-T1005, “High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 3,” 20th Meeting: Geneva, CH, 10 February – 17 February 2015.

- JCTVC-S0145, “Bandwidth reduction method for intra block copy,” 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014.

- JCTVC-T0051, “AHG10: On IBC memory reduction,” 20th Meeting: Geneva, CH, 10–18 Feb. 2015.

- JCTVC-T0045, “AHG10: Memory bandwidth reduction for intra block copy,” 20th Meeting: Geneva, CH, 10–18 Feb. 2015.