| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0084 |  |
| --- | --- | --- |

| Title: | On palette predictor initialization for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye Shan Liu Shawmin Lei 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | +1-408-526-1899 jing.ye@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to allow signalling a set of palette predictor initializers in SPS to reduce the overhead of signalling them in each PPS under some circumstances. When sps_palette_predictor_initializer_present_flag is equal to 1, a set of palette predictor initializers are signalled in SPS for the current CVS to use; otherwise, the entries in the palette predictor are initialized to 0. When pps_palette_predictor_initializer_present_flag is equal to 1, a set of palette predictor initializers are signalled in PPS; otherwise, the entries in the palette predictor are derived from SPS palette predictor initializers. Experimental results report similar B-D rate performance by using the proposed method and the PPS palette predictor initializer scheme in SCM4.0 for AI configuration. B-D rate savings are observed when the proposed method is used for inter configurations.

# Introduction

In the 20th JCTVC meeting, the method of signaling palette predictor initializers in picture parameter set (PPS) for initializing the palette predictor was adopted [1]. In a reference implementation in SCM4.0, PPS is signalled every 16 frames for AI configuration, every 1 second for RA configuration and every 5 seconds for LB configuration, and when the following conditions are met:

- PalettePredInPPSEnabled is set to be TRUE,

- the num_palette_predictor_initializer_minus1+1 is greater than 3,

It is observed that the major colours (or palette) for adjacent slices might be similar. In many sequences, it could be just sufficient to signal a set of palette predictor initializers at sequence level instead of repeatedly resetting them in PPS. In this contribution, a mechanism is proposed to elaborate this thought.

# Proposed method

This contribution proposes to allow signalling a set of palette predictor initializers in SPS. Modified syntax and semantics on top of JCTVC-T1005 [2] are illustrated below, with changes highlighted in yellow.

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| …… |  |
| palette_mode_enabled_flag | u(1) |
| if( palette_mode_enabled_flag ) { |  |
| …… |  |
| sps_palette_predictor_initializer_present_flag | u(1) |
| if( sps_palette_predictor_initializer_present_flag ) { |  |
| sps_num_palette_predictor_initializer_minus1 | ue(v) |
| numComps = (chroma_format_idc = = 0 ) ? 1 : 3 |  |
| for( i = 0; i <= sps_num_palette_predictor_initializer_minus1; i++ ) |  |
| for( comp = 0; comp < numComps; comp++ ) |  |
| sps_palette_predictor_initializers[ i ][ comp ] | u(v) |
| } |  |
| } |  |
| …… |  |
| } |  |

| pps_scc_extensions( ) { | Descriptor |
| --- | --- |
| …… |  |
| pps_palette_predictor_initializer_present_flag | u(1) |
| if( pps_palette_predictor_initializer_present_flag ) { |  |
| monochrome_palette_flag | u(1) |
| luma_bit_depth_entry_minus8 | ue(v) |
| if( !monochrome_palette_flag ) |  |
| chroma_bit_depth_entry_minus8 | ue(v) |
| pps_num_palette_predictor_initializer_minus1 | ue(v) |
| numComps = monochrome_palette_flag ? 1 : 3 |  |
| for( i = 0; i <= pps_num_palette_predictor_initializer_minus1; i++ ) |  |
| for( comp = 0; comp < numComps; comp++ ) |  |
| pps_palette_predictor_initializers[ i ][ comp ] | u(v) |
| } |  |
| } |  |

7.4.3.2.3	Sequence parameter set screen content coding extensions

sps_palette_predictor_initializer_present_flag equal to 1 specifies that the sequence palette predictors are initialized using the sps_palette_predictor_initializers specified in clause Error: Reference source not found. sps_palette_predictor_initializer_flag equal to 0 specifies that the entries in the sequence palette predictor are initialized to 0. When not present, the value of sps_palette_predictor_initializer_flag is inferred to be equal to 0.

sps_num_palette_predictor_initializer_minus1 specifies the number of entries in the sequence palette predictor initializer.

It is a requirement of bitstream conformance that the value of sps_num_palette_predictor_initializer_minus1 plus one shall be less than or equal to PaletteMaxPredictorSize.

sps_palette_predictor_initializers[ i ][ comp ] specifies the value of the comp-th component of the i-th palette entry that is used to initialize the array PredictorPaletteEntries. For values of i in the range of 0 to sps_num_palette_predictor_initializer_minus1, inclusive, the value of the sps_palette_predictor_initializers[ i ][ 0 ] shall be in the range of 0 to (1 << BitDepthY) − 1, inclusive, and the values of sps_palette_predictor_initializers[ i ][ 1 ] and sps_palette_predictor_initializers[ i ][ 2 ] shall be in the range of 0 to (1 << BitDepth-C) − 1, inclusive.

7.4.3.3.3	Picture parameter set screen content coding extensions

pps_palette_predictor_initializer_present_flag equal to 1 specifies that the picture palette predictors are initialized using the pps_palette_predictor_initializers specified in clause Error: Reference source not found. pps_palette_predictor_initializer_flag equal to 0 specifies that the entries in the picture palette predictor  referring to the PPS are inferred to be equal to those specified by the active SPS. When not present, the value of pps_palette_predictor_initializer_flag is inferred to be equal to 0.

……

pps_num_palette_predictor_initializer_minus1 specifies the number of entries in the pps palette predictor initializer.

……

pps_palette_predictor_initializers[ i ][ comp ] specifies the value of the comp-th component of the i-th palette entry that is used to initialize the array PredictorPaletteEntries. For values of i in the range of 0 to pps_num_palette_predictor_initializer_minus1, inclusive, the value of the pps_palette_predictor_initializers[ i ][ 0 ] shall be in the range of 0 to (1 << BitDepthEntryY) − 1, inclusive, and the values of pps_palette_predictor_initializers[ i ][ 1 ] and pps_palette_predictor_initializers[ i ][ 2 ] shall be in the range of 0 to (1 << BitDepthEntryC) − 1, inclusive.

9.3.2.3	Initialization process for palette predictor variables

Outputs of this process are the initialized palette predictor variables PredictorPaletteSize and PredictorPaletteEntries.

The PredictorPaletteSize and PredictorPaletteEntries are derived as follows:

–	If pps_palette_predictor_initializer_present_flag is equal to 1, PredictorPaletteSize is set equal to pps_num_palette_predictor_initializer_minus1 plus 1.

–	Otherwise (palette_predictor_initializer_present_flag is equal to 0 ), PredictorPaletteSize is set equal to 0

The array PredictorPaletteEntries is derived as follows:

–	If palette_predictor_initializer_present_flag is equal to 1,

for( i = 0; i < PredictorPaletteSize; i++ )

for( comp = 0; comp < 3; comp++ )

PredictorPaletteEntries[ i ][ comp ] = pps_palette_predictor_initializers[ i ][ comp ]

–	Otherwise (pps_palette_predictor_initializer_present_flag is equal to 0 ), PredictorPaletteEntries is set equal to 0.

–	If sps_palette_predictor_initializer_present_flag is equal to 1, PredictorPaletteSize is set equal to sps_num_palette_predictor_initializer_minus1 plus 1.

The array PredictorPaletteEntries is derived as follows:

for( i = 0; i < PredictorPaletteSize; i++ )

for( comp = 0; comp < 3; comp++ )

PredictorPaletteEntries[ i ][ comp ] = sps_palette_predictor_initializers[ i ][ comp ]

–	Otherwise, PredictorPaletteSize is set equal to 0 and PredictorPaletteEntries is set equal to 0.

# Simulation results

The proposed method is implemented on top of SCM4.0. Table 1 and Table 2 report the lossy and lossless results of the proposed method with sps_palette_predictor_initializer_present_flag set to be equal to 1 and pps_palette_predictor_initializer_present_flag set to be equal to 0. Table 3 reports the lossy results of PPS refreshed every 1 second. Table 4 and Table 5 report the lossy results of SCM4.0 with the PPS refreshed every 16 and 32 frames. Table 6 and Table 7 report the lossy and lossless results of SCM4.0 with PalettePredInPPSEnabled set to be equal to 1. All the results are compared with SCM4.0 under the common test condition defined in [3]. All the simulations are tested with full frame IBC and compared with SCM4.0 CTC. It is observed that the proposed scheme outperforms the anchor in B-D rate by saving the signalling overhead.

Table 1 Lossy 444, proposed method

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.3% | 0.1% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.4% |
| YUV, Animation, 720p | -0.1% | -0.4% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.3% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | 0.3% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.4% | -0.4% |
| YUV, Animation, 720p | -0.1% | -0.4% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.6% | -0.5% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.3% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.6% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.3% | -0.7% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2 Lossless 444, proposed method

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.2% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.3% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.3% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.3% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.5% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | -0.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 3 Lossy 444, PPS refreshed every 1 second

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.5% | -0.4% |
| YUV, Animation, 720p | -0.1% | -0.6% | -0.5% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.3% | 0.0% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.2% | 0.7% |
| YUV, mixed content, 1440p & 1080p | 0.1% | -0.2% | -0.3% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.2% | 0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.1% | -0.5% |
| YUV, Animation, 720p | 0.2% | 0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4 Lossy 444, PPS refreshed every 16 frames

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.6% | -0.5% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.6% | 0.3% | 0.5% |
| RGB, mixed content, 1440p & 1080p | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.2% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.7% | 0.6% | 1.1% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 0.2% | 0.1% |
| YUV, Animation, 720p | 0.4% | 0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.5% | 0.4% | 0.5% |
| RGB, mixed content, 1440p & 1080p | 0.5% | 0.5% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.7% | 0.6% | 0.6% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 0.6% | 0.2% |
| YUV, Animation, 720p | 0.4% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

Table 5 Lossy 444, PPS refreshed every 32 frames

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.4% |
| YUV, Animation, 720p | -0.1% | -0.5% | -0.5% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.3% | 0.0% | 0.2% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.3% | 0.3% | 0.8% |
| YUV, mixed content, 1440p & 1080p | 0.2% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.2% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.3% | 0.2% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.2% | -0.2% |
| YUV, Animation, 720p | 0.2% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

Table 6 Lossy 444, SCM 4.0 with PalettePredInPPSEnabled equal to 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.6% | -0.5% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.3% | 0.0% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.3% | 0.2% | 0.7% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.3% | -0.4% |
| YUV, Animation, 720p | 0.3% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | -0.5% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.3% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.5% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.3% | -0.7% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 7 Lossless 444, SCM 4.0 with PalettePredInPPSEnabled equal to 1

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.2% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.3% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.3% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.3% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.5% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

# Conclusion

This contribution proposes to allow signalling a set of palette predictor initializers in SPS to reduce the overhead of signalling them in PPS under some circumstances. Experimental results report similar B-D rate performance using the proposed method and the PPS palette predictor initializer scheme in SCM4.0 for AI and B-D rate savings are observed when the proposed method is used for inter configurations. It is recommended to include the proposed method to the Screen Content Coding working draft.

# References

- C. Gisquet, T. Poirier, G. Laroche, P. Onno (Canon), “Non-CE1: On palette prediction for slices”, JCTVC-T0048, 20th Meeting: Geneva, Feb 2015.

- R. Joshi, J. Xu, S. Liu, Y. Ye, “High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 3”, JCTVC-T1005, 20th Meeting: Geneva, Feb 2015.

- H. Yu, R. Cohen, K. Rapaka, J. Xu (editors), “Common Test Conditions for Screen Content Coding”, JCTVC-T1015, 20th Meeting: Geneva, Feb 2015.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).