| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0085r1 |
| --- | --- |

| Title: | Modulation-based HDR video coding with SDR backward compatibility |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Lasserre, F. Le Léannec, E. François, T. Poirier 975 avenue des champs blancs CS 17616, 35576, Cesson-Sévigné Cedex, France | Tel: Email: | (+33) 2.99.27.37.21 sebastien.lasserre@technicolor.com fabrice.leleannec@technicolor.com edouard.francois@technicolor.com tangi.poirier@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution presents a single layer HDR video coding solution targeting SDR backward compatibility. The solution is based on a pre-processing step, applied prior to the encoding, that generates from the input HDR content an SDR version and side modulation metadata of limited size. The SDR video can be coded/decoded using legacy codecs (e.g compliant with HEVC Main 10) and the decoded video can be directly viewable on SDR displays. A new SEI message is proposed to transmit the modulation metadata (coded per picture, GOP or scene). Modulation metadata are used to reconstruct the HDR signal from the decoded SDR video, using a post-processing that is the functional inverse of the pre-processing. Both HDR quality and visual rendering are reported to be preserved. Pre- and post-processing are applied independently per picture, do not involve any inter-sample dependency, and are codec independent.

# Introduction

As stated in MPEG documents N15083 and N15084 [1,2], SDR backward compatibility (with decoding and rendering devices) may be an important feature in some video distribution systems, such as broadcasting or multicasting systems. Dual-layer codec is one solution to support this feature. However due to its multi-layer design, this solution is not adapted to all distribution workflows.

An alternative to SDR backward compatibility is to transmit an HDR content and to apply at receiving device an HDR-to-SDR adaptation process (tone mapping). One issue in this scenario is that the tone mapped content may be out of control of the content provider or creator.

The proposed solution aims at addressing these issues. It can provide SDR backward compatibility feature, while guarantying the HDR content rendering intent.

The document is organized as follows. The proposal overview is presented in section 2. Section 3 describes the theoretical basics of the solution (at encoder side). The actual implementation is described in section 4. Section 5 presents some compression results. Conclusion is given in section 6.

# Proposal overview

## Main features

The main features of the proposal are as follows:

- Single layer with metadata: the solution is based on a single layer coding process, with side metadata that can be used at post-processing stage. The metadata payload corresponds to a few bytes per picture, GOP or scene.

- Codec agnostic: the solution does not impact the core codec technology and is codec independent. In particular it can be used with HEVC Main 10 or AVC High Profile. The solution is based on an encoding pre-processing applied to the HDR input, and on a corresponding decoding post-processing (functional inverse of the pre-processing) applied to the reconstructed video from decoding.

- Enable SDR backward compatibility: a decoded bitstream can be displayed as is on an SDR display. An additional post-processing is applied to convert the decoded SDR version to HDR, thanks to the metadata, with preservation of the HDR artistic intent.

- Enable preserved quality of HDR content: there is no penalty due to the SDR backward compatibility feature; coding performance compared to the MPEG CfE HEVC anchors are improved, in particular in terms of color impairments.

- Limited additional complexity: the pre- and post-processing steps are of limited added complexity; in particular the involved operations are only sample-based, without inter-sample dependency; actual implementations on proof-of-concept set-top-boxes have been made and demonstrated [3].

- Independent from the input OETF: the pre- and post-processing operate in linear-light domain, and are therefore independent from the input OETF.

## High-level description

The solution is schematically described in Figure 1. The input HDR video is first pre-processed to generate an SDR version and side metadata. This pre-processing applies for each picture and is similar to signal demodulation. Each HDR picture is split into one modulation value Ba and one SDR picture. The SDR video is coded using for instance HEVC Main 10 profile, with side modulation values metadata. The decoded video is SDR and can be used by SDR displays. For HDR displays, a post-processing is applied to each reconstructed SDR picture. The post-processing modulates each SDR reconstructed picture with its related modulation values metadata to reconstruct the HDR picture version. The post-processing is the functional inverse of the pre-processing step, which guarantees artistic intent preservation of the HDR content.

Figure 1. Schematic block diagram of the solution.

# Theoretical basics of the solution

The pre-processing at encoder side converts (before encoding) the linear-light HDR input (R,G,B) into an SDR signal (noted (Y,U,V)). The process is based on signal demodulation. In addition to the SDR signal, the process generates at most one single modulation value for each picture. The pre-processing applies in the following four main operations:

- Derivation of a modulation value Ba from the input HDR picture.

Then for each tristimulus sample value (R,G,B), the following applies:

- Conversion of the HDR linear input luminance L into an intermediate SDR luma value of limited bit-depth Yi (typically on 10 bits) using the modulation value Ba.

- Derivation of two intermediate SDR chroma values Ui,Vi of limited bit-depth from the linear input components (R,G,B) using the modulation value Ba and value Yi.

- Color gamut correction, by gamut mapping from the intermediate color space in which the sample (Yi,Ui,Vi) is represented, to the source color space. This results in the output (Y,U,V) SDR sample.

A final chroma downsampling is optionally applied to obtain a 4:2:0 signal.

The complete process provides a viewable SDR stream from which the HDR version can be reconstructed thanks to the revertible property of this process. The SDR stream is encoded by any standard 10-bit codec profile, preferably the HEVC Main10 profile.

Figure 2. Overview of the single layer HDR pre-processing.

The four operations are detailed below. The input is the linear-light RGB picture.

## Step 1 – modulation value Ba derivation

First a modulation value Ba is derived for each picture. The conversion of input linear-light HDR luminance signal into an SDR luma signal is based on a normalized log-shape function (as explained below in step 2, section 3.2). The modulation value is derived from the signal geometric mean gm (typically derived from the 1% and 99% percentiles of the picture luminance distribution, as illustrated in Figure 3). Ba is basically chosen so that around the geometric mean gm , the number of codewords after conversion to the non-linear SDR domain (cf step 2, section 3.2) is maximum. A temporal smoothing is added to control the potential fluctuations of Ba among successive pictures.

Figure 3. Illustration of Ba derivation based on picture geometric mean.

## Step 2 – intermediate SDR luma Yi derivation from luminance L and Ba

In a second step, the following steps apply.

(eq.1)

- with  being the conventional 3x3 R’G’B’-to-Y’CbCr conversion matrix (e.g. BT.2020 or BT.709 depending on the source HDR color space),  being 1x3 matrices, and P being the mastering display peak luminance.

(eq.2)

- where the transfer function F(.) is of shape   with ,  being fixed parameters, and N the number of bits to be used by the video compressor (N is typically equal to 10 if HEVC Main 10 is used).

P will be signaled to the decoder in the mastering display colour volume SEI.

## Step 3 – intermediate SDR chroma Ui,Vi derivation from R, G, B, L and Yi

In a third step, the following steps apply.

- Normalization of R,G,B by the ratio r = Yi / L

(eq.3)

- SDR-adapted OETF : application of square root to reproduce an OETF close to BT.709 OETF; the usage of a square root enables getting a completely invertible process as proved later.

(eq.4)

- Ui and Vi derivation using conventional R’G’B’-to-Y’CbCr conversion matrix A as decomposed above.

(eq.5)

with 	(eq.6)

- Note :  can be recovered from L,U0,V0 by solving eq.1 and eq.6 with  as unknown. It can be shown that the solution is exactly:

  - with

- and  being constant parameters only depending on the matrix A coefficients. This proves that the process is invertible and that inverse operations are simple.

- this property will be used at decoder side to recover (R,G,B) signal from the reconstructed Y’CbCr SDR signal

## Step 4 – color gamut correction

The previous steps cause a color shift that must be corrected. The approach consists in finding the mapping matrix M preserving the HDR source primaries and white point, as illustrated in Figure 3.

Figure 4. Illustration of the color gamut correction.

An input red HDR signal PR=[R,0,0]T must be transformed into an output red SDR signal Pr=[r,0,0]T, converted to Y’CbCr domain, after the steps 2 (section 3.2), 3 (section 3.3) and 4 (section 3.4).

Similarly the three following additional constraints have to be satisfied for green and blue primaries, and grey signal:

It can be demonstrated by algebraic computations that the matrix M can be approximated in the following shape:

with a and b being constant parameters, and  depending on Yi, Ba, and weakly on the peak luminance P. A demonstration is provided in annex.

At high brightness, pure saturated primaries cannot be attained anymore, since the R, G, and B values cannot be above the peak luminance P. For instance pure red cannot have a luminance above LmaxR = a11.P (for a tristimulus sample [P,0,0]T). Above this value, the tristimulus sample necessarily comprises non-zero G and/or B component values. This is illustrated in figure 5 showing that above a luminance of a11.P, pure red colors are not anymore possible. We call this problem color gamut closure. The function  is adjusted to tackle this issue, based on the same primaries and white point preservation principles as described above.

Figure 5. Gamut closure for the red component; at peak luminance P, only white exists.

For a given luminance L above a11.P, the tristimulus redish sample completed by G and B values to reach the luminance L, is considered:

with  being easily deduced so that .

The red preservation constraint is applied:

Similarly, green and blue preservation constraints are applied. The white point is also constrained to be preserved. Solving these constraints enables adjusting  for high luminance values.

The color gamut correction therefore results in two successive steps :

- Mapping Ui and Vi to the right (source) color gamut (by saturation scaling factor 1/)

(eq.7)

or equivalently

(eq.8)

with  =  being a correction parameter depending on Yi, Ba, and weakly on the peak luminance P.

- Yi correction to manage color saturation and color gamut closure at high luminance levels

(eq.9)

The max clipping in (eq.9) is introduced to avoid values above 2N-1 (when () is negative) that should be clipped and would lead to annoying visual effects such as flat white zones around the peak luminance. In practice, parameters a and b can be used to control SDR colors saturation. A large value of a and b will generate more saturated colors (as luma is more decreased, colors look more saturated) while a small value of a and b will generate less saturated colors (as luma is less decreased, colors look less saturated, in particular at high brightness, because of gamut closure). Parameters a and b can be transmitted as metadata.

# Reference pre- and post-processing implementation

The previous section described the theoretical basics of the invertible pre-processing, that generates an SDR version from the input HDR content. In this section, a reference implementation of the pre- and post-processing steps is provided.

In both cases, the process applies independently to each input picture, and to each sample, using the Ba value associated to the picture (Ba is computed at pre-processing side, and decoded at post-processing side).

The following different functions are implemented using Look-Up Tables (LUTs):

- in pre-processing

  - as a limited set of LUTs depending on a few discrete Ba and P values, the mastering display and HDR container gamuts, with Yi as entry

- in post-processing

  - as one LUT with Yi as entry, independent from Ba and P.

  - and      as two LUTs with x as entry

  - as a limited set of LUTs depending on a few discrete Ba and P values, the mastering display and HDR container gamuts, with Yi as entry

## Pre-processing

The following table summarizes the pre-processing successive steps. The input is a linear-light HDR tristimulus sample value (R,G,B). The outputs are a modulation value Ba and a SDR non-linear light tristimulus sample value (Y,U,V).

Table 1. Pre-processing description.

| Pre-processing | Comments |
| --- | --- |
|  | conventional 3x3 R’G’B’-to-Y’CbCr conversion matrix |
|  | , , fixed parameters |
|  | Reproduction of an OETF similar to BT.709 OETF |
|  | is implemented as a 1D LUT (cf fig.9). It is interpolated for each picture from 2 LUTs, identified from Ba, P, and the mastering display and container gamuts. |
|  | a,b fixed parameters or coded in SEI message |

## Post-processing

The following table summarizes the post-processing successive steps. The inputs are a decoded modulation value Ba and a reconstructed SDR tristimulus sample value (Y,U,V). The output is a reconstructed linear-light HDR tristimulus sample value (R,G,B).

Table 2. Post-processing description.

| Post-processing | Comments |
| --- | --- |
|  | only depends on Yi and is implemented as a 1D LUT (cf fig.7). Note : as |
|  | Note: T is positive by construction |
| If T<=1 Else | Note : In case T>1 (may happen because of quantization), Ur and Vr are rescaled by 1/ and S becomes 0. The rescaling guarantees hue preservation, while simply clipping Ur, Vr results in noticeable hue change. and functions are implemented using two 1D LUTs. |
|  | is implemented as a 1D LUT (cf fig.8). It is interpolated for each picture from 2 LUTs, identified from Ba, P, and the mastering display and container gamuts. |
|  | Final adaptation of the linear-light signal to the display EOTF can be combined with this step. |

The post-processing block diagram is depicted in Figure 6.

Figure 6. Block diagram of the post-processing.

Practically, the pre-processing LUTs  are optimized to get optimal color (hue and perceived saturation) matching between the original HDR and the obtained SDR pictures. For a fixed couple (Ba, Yi) an optimal value  is determined by minimizing the distance between an input HDR gamut and an output pre-processed SDR gamut. The minimization is performed in the two-dimensional sub-domain “ab” of the Lab color space, the sub-domain being corrected for taking into account the perceived saturation instead of the chroma amplitude. The distance is computed as the weighted sum on a meshing of the gamut, each mesh being weighted by its perceptual importance (i.e. volume) in the perceptual Lab color space.

It is to be noted that the LUT optimization is performed independently on any content as it is performed on the whole gamut supposed to contain all possible contents. By doing so, the LUT’s can be automatically computed for any peak luminance P and any gamut (like BT.709, BT.2020, a display gamut, or other gamut that one may invent).

The LUTs  are perfectly defined from the definition of Yi as a function of L and Ba. The LUT  can be simply deduced as LUT  divided by LUT . Different examples of LUTs are depicted in the three following figures.

Figure 7. LUTs ’ for different content and container gamuts. “2020_709” indicates BT.2020 gamut container with a BT.709 mastering display gamut.

Figure 8. LUTs  for different Ba values.

Figure 9. LUTs  for different Ba values.

## Proposed new SEI message

The post-process requires the following parameters to perform the decoded signal adaptation:

- mastering display peak luminance;

- mastering display color gamut;

- output signal color gamut container;

- modulation value Ba.

The mastering display peak luminance and mastering display color gamut are coded in the mastering display colour volume SEI.

The output signal color gamut container is actually expected to be given from the rendering device. Note that for the signal adaptation to the display, other display parameters such as the peak luminance can be useful.

A new SEI message is defined to transmit the modulation value Ba (syntax element modulation_value in next table). The SEI message can be transmitted per picture. Nevertheless, thanks to the high temporal stability of the modulation parameter, the SEI message can be transmitted once per GOP or scene.

Table 3. HDR adaptation information SEI message, option 1.

| high_dynamic_range_adaptation_information( payloadSize ) { | Descriptor |
| --- | --- |
| modulation_value | u(16) |
| } |  |

The two parameters a and b (introduced in section 3.4) can also be transmitted to enable controlling the colors saturation of the SDR content (syntax elements saturation_value in next table). The SEI message, option 2, comprises these parameters.

Table 4. HDR adaptation information SEI message, option 2.

| high_dynamic_range_adaptation _information( payloadSize ) { | Descriptor |
| --- | --- |
| modulation_value | u(16) |
| for( c = 0; c < 2; c++ ) |  |
| saturation_value[ c ] | u(16) |
| } |  |

# Compression performance

## Tuning focused on visual quality

The proposed solution has been evaluated based on the test conditions defined for the MPEG Call for Evidence (CfE) for HDR and WCG Video Coding [1]. The input sequences are HDR sequences represented in linear-light RGB. The content is either BT.709 or P3, and is represented in a BT.2020 container.

The reference (anchors) are generated using the process depicted in figure 10. PQ-EOTF corresponds to the EOTF specified in SMPTE ST 2084 specification. PQ-OETF is the inverse of the PQ-EOTF. After applying the PQ-OETF to the input linear-light signal, the R’G’B’ signal is converted to Y’CbCr using the conventional R’G’B’-to-Y’CbCr conversion matrix. The signal is then quantized to 10 bits, and the chroma is downsampled to get a 4:2:0 Y’CbCr signal. This signal is encoded by HM16.2, then decoded. The output is converted back to linear-light domain by chroma upsampling, dequantizing and PQ-EOTF application.

Figure 10. Reference (anchors) generation process.

The sequences are coded with four different bitrates as explained in [1]. The proposed solution is evaluated for the same sequences and four bitrates.

The issue of objective metrics for comparing quality of two HDR videos is still open. Using the usual PSNR metric on Y,’, Cb and Cr components does not look relevant as correlation with visual evaluations is small in the case of HDR. For the MPEG HDR CfE, three other metrics are considered. One of them seems to be more relevant than the others, the PSNR-deltaE metric [1]. The following table summarizes the BD-rate performance of the proposed solution compared to the anchors using the PSNR-deltaE metric. Compared to the anchors, the BD-rate average variations of the proposed solution is of -36.6% for PSNR-deltaE.

Table 5. PSNR-deltaE BD-rate results.

## Tuning focused on objective metric

Previous section has provided objective metrics figures obtained when the pre-processing step is configured based on visual evaluation only, and the HM encoder is left unchanged compared to the anchors. The goal of this section is to show that the proposed HDR coding scheme can be adapted from a rate distortion performance point-of-view, given that an HDR quality metric is admitted as relevant.

In the next table, the coding performance is reported with a tuning more focused on objective metric. The HM weighted prediction is enabled. The rate-distortion optimization process has also been modified to better take into account the objective metric. Also the Ba derivation, the b0 parameter of the log function (see eq.2) and the  LUT are tuned differently to improve the metrics. The corresponding results are depicted in the following table. The average BD-rate variation for the deltaE metric is of -49.1%, which demonstrates that the proposed solution can also be efficiently tuned for objective purposes.

|  |  | deltaE visual | deltaE objective |
| --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | -24.5% | -45.8% |
|  | Market3Clip4000r2 | -60.5% | -71.8% |
|  | Tibul2Clip4000r1 | -17.8% | -31.3% |
| class B | AutoWelding | -28.5% | -38.8% |
|  | BikeSparklers | -30.2% | -53.2% |
| class C | ShowGirl2Teaser | -49.3% | -56.8% |
| class D | StEM_MagicHour | -32.3% | -41.5% |
|  | StEM_WarmNight | -33.2% | -41.1% |
| class G | BalloonFestival | -53.1% | -61.3% |
|  | Overall | -36.6% | -49.1% |

Table 6. Comparison of PSNR-deltaE metrics with visual and objective tunings.

# Conclusion

The document described an HDR coding system using a single layer design, and independent from the core codec. It is based on a pre-processing step that can generate an SDR backward compatible video, with accompanying modulation metadata of very limited size. After coding-decoding, the SDR video can thus be rendered as is on an SDR display. The post-processing is the functional inverse of the pre-processing, which enables recovering an HDR video from the decoded SDR video and modulation metadata visually conform to the HDR input. The added complexity from the post-processing is limited. Support of SDR backward compatibility feature does not impact the coding efficiency.

# References

[1]	A. Luthra, E. François, W. Husak, “Call for Evidence (CfE) for HDR and WCG Video Coding,” document ISO/IEC JTC1/SC29/WG11 MPEG2015/N15083, 111th MPEG meeting, Geneva, CH, October 2015.

[2]	A. Luthra, E. François, W. Husak, “Requirements and Use Cases for HDR and WCG Content Coding,” document ISO/IEC JTC1/SC29/WG11 MPEG2015/N15084, 111th MPEG meeting, Geneva, CH, October 2015.

[3]	“Technicolor developing HDR box,” http://www.csimagazine.com/csi/Technicolor-developing-HDR-box.php, 2015.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex - construction of the mapping matrix M

Preservation white point

We consider a given luminance L. The white point is WP=[L,L,L]T in RGB linear-light space.

L is converted to Y (which also depends on Ba). White point WP is converted to [Y,0,0]T after steps 1, 2 and 3. Preservation of the white point involves:

Which leads to m11=1, m12=0, m13=0. M becomes

Primaries preservation

For the same luminance L and for the red primary, the input signal is PR = [L/a11,0,0]T

The luma signal Y is same as above (since L is also same).

  normalize by        sqrt      UV derivation  

Thus

Which leads to the following equations

Doing the same for G and B primaries leads to following equations

Combining eq.2 and eq.3, eq.5 and eq.6, and eq.8 and eq.9   leads to

Solving eq.10, eq.11 and eq.12 leads to

m22=m33 and m23=m32=0

Back to eq.2, eq.5, eq.8:

Combining eq.1 and eq.13, eq.4 and eq.14, and eq.7 and eq.15 leads to

Or similarly

Combining eq.19 and eq.20 and eq.19 and eq.21 to remove first term  leads to

Solving these two equations gives m12 = -a.m22 and m13 = -b.m22, with a and b depending only of coefficients of matrix A.

Then m22 is derived as a function of  from eq.19, 20 or 21.