| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0087r1 |
| --- | --- |

| Title: | Syntax Cleanup for Palette Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu, Rajan Joshi, Ted Hsieh, Marta Karczewicz, Feng Zou, Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 Wei.Pu@qti.qualcomm.com rajanj@qti.qualcomm.com thsieh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current HEVC SCC draft specification, new palette entries are signalled by grouping each color component while the escape pixels are signalled by interleaving the three color components. In this contribution, two unification methods are proposed to clean up the palette mode syntax.

# Introduction

In palette mode, if parts of the current block’s palette is not predictable by the previously coded palette entries, the new palette entries are signalled into the bitstream according to the following syntax table (See [1]. Section 7.3.8.8 Palette syntax):

| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| --- | --- |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| palette_entry | ae(v) |

Also in palette mode, if there are pixels which cannot be represented by the palette, these pixels are named as escape pixels and their values are signalled using the following syntax table (See [1]. Section 7.3.8.8 Palette syntax):

| while( sPos < nCbS * nCbS ) { |  |
| --- | --- |
| … |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| … |  |
| palette_escape_val | ae(v) |
| … |  |
| } |  |

According to the above two table excerpts, it is noticed that within Section 7.3.8.8, the new palette entries are signalled by grouping each color component while the escape pixel values are signalled by interleaving the three color components.

# Proposed Cleanup

This document proposes two cleanups.

## Unification by Interleaving Color Components

This approach changes the order of signaling new palette entries, so that the new palette entries are signalled with color components interleaving.

| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| --- | --- |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| palette_entry | ae(v) |

## Unification by Grouping Color Components

This approach changes the order of signaling escape pixel values, so that the escape pixel values are signalled with color component grouping.

| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| --- | --- |
| while( sPos < nCbS * nCbS ) { |  |
| … |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| … |  |
| palette_escape_val | ae(v) |
| … |  |
| } |  |

# Reference

- R. Joshi, S. Liu, J. Xu, and Y. Ye, “HEVC screen content coding draft text 3,” in JCTVC-T1005, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.