| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0089 |
| --- | --- |

| Title: | Constraints on palette syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai1, Jungsun Kim1, Shan Liu1, Jing Ye1, Tzu-Der Chuang2, and Shawmin Lei1 1 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, USA 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Email: | { polin.lai, jungsun.kim, shan.liu, jing.ye, peter.chuang, shawmin.lei } @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents constraints on palette syntax elements. Constraint on the SPS parameter palette_max_size is presented to align with the palette CU size constraint. Constraints on the palette_predictor_run and num_signalled_palette_entries are presented to ensure the palette operations do not violate the palette_max_size specified in SPS.

# Proposed constraints

## Constraints on the SPS parameter palette_max_size

In SCC Draft Text 3 [1], for CU size > max TU size, palette mode is prohibit, and the palette mode flag is not transmitted and is inferred as 0. In HEVC Spec, MaxTbLog2SizeY specifies the max TU size in log scale. Accordingly, it is proposed to set an upper limit of the SPS parameter palette_max_size as below:

palette_max_size specifies the maximum allowed palette size. When not present, the value of palette_max_size is inferred to be 0. The value of palette_max_size shall be in the range of 0 to 1<<(MaxTbLog2SizeY*2), inclusive.

With the constraint above, for example, if MaxTbLog2SizeY = 5, the max TU size (max palette CU size) is 32, and the limit of palette_max_size is 32x32 = 1024, which means each sample in a 32x32 CU is a palette colour.

## Constraints on num_palette_indices_idx and NumPaletteIndices

The syntax element num_palette_indices_idx is used to derive the number of palette indices in the current CU (NumPaletteIndices). This number should not be greater than the total number of samples in the current CU. The following constraints are proposed:

num_palette_indices_idc is an indication of the number of palette indices signalled for the current block. When num_palette_indices_idc is not present, it is inferred to be equal to 0. The variable NumPaletteIndices specifies the number of palette indices signalled for the current block and is derived as follows: if( num_palette_indices_idc >= ( MaxPaletteIndex – 1) * 32 ) NumPaletteIndices = num_palette_indices_idc + 1 else if(num_palette_indices_idc % 32 = = 31) NumPaletteIndices = MaxPaletteIndex – (num_palette_indices_idc + 1 ) / 32 (7-1) else NumPaletteIndices = ( num_palette_indices_idc / 32 ) * 31 ) + (num_palette_indices_idc % 32) + MaxPaletteIndex The value of NumPaletteIndices should be in the range of 1 to nCbS * nCbS.

## Constraints on palette_predictor_run

Fig. 1 illustrates the process of reconstructing the palette for the current block as specified in SCC Draft Text 3 [1]. From the palette predictor, some entries are reused for the current block, as those indicated by the palette reuse flags equal to 1. These flags are coded by run-length coding: The runs between two 1s are coded with syntax element palette_predictor_run (coded using Exponential Golomb order 0, EG-0).

Fig. 1 Prediction of palette entries from the palette predictor

When a palette_predictor_run is decoded, the palette reuse flag at the corresponding position is set to 1, the parsing and decoding process for palette_predictor_run and palette reuse flags is as table 1 below.

Table 1. Decoding of palette_predictor_run and reuse reuse flags in SCC Draft Text 3

| palettePredictionFinished = 0 |  |
| --- | --- |
| NumPredictedPaletteEntries = 0 |  |
| for( i = 0; i < PredictorPaletteSize && !palettePredictionFinished && NumPredictedPaletteEntries < palette_max_size; i++ ) { |  |
| palette_predictor_run | ue(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| i += palette_predictor_run − 1 |  |
| PalettePredictorEntryReuseFlag[ i ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |

However, when inside the loop, palette_predictor_run might produce invalid value “i” that is beyond PredictorPaletteSize. For example, PredictorPaletteSize is 17, current i position is 14, and coded / parsed palette_predictor_run is 10, then i += 10-1 becomes 23, which is over the PredictorPaletteSize.

The following methods are proposed to prevent accessing beyond the array size in such scenario:

- Method 1: Add conditional check on “i”, terminate loop

If it is detected that the value i, after adding the decoded run, is larger than PredictorPaletteSize, the loop simply terminates (no further access to PalettePredictorEntryReuseFlag[], no increase to the value NumPredictedPaletteEntries). This method can be summarized as Table 2. below:

Table 2. Method 1 for decoding process for palette_predictor_run and reuse reuse flags.

| palettePredictionFinished = 0 |  |
| --- | --- |
| NumPredictedPaletteEntries = 0 |  |
| for( i = 0; i < PredictorPaletteSize && !palettePredictionFinished && NumPredictedPaletteEntries < palette_max_size; i++ ) { |  |
| palette_predictor_run | ue(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| i += palette_predictor_run − 1 |  |
| if (i >= PredictorPaletteSize ) |  |
| palettePredictionFinished = 1 |  |
| else { |  |
| PalettePredictorEntryReuseFlag[ i ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |

- Method 2: Cap the value “i” to the last position of PalettePredictorEntryReuseFlag[]

In this method, the value i after adding the decoded run, will be checked against PredictorPaletteSize - 1, which is the last position of PalettePredictorEntryReuseFlag[]. The value i will be capped to this value palette predictor size - 1. This way, the last position will be written with reuse flag 1, and the value of NumPredictedPaletteEntries is increased by 1. This method can be summarized as Table 3. below:

Table 3. Method 2 for decoding process for palette_predictor_run and reuse reuse flags.

| palettePredictionFinished = 0 |  |
| --- | --- |
| NumPredictedPaletteEntries = 0 |  |
| for( i = 0; i < PredictorPaletteSize && !palettePredictionFinished && NumPredictedPaletteEntries < palette_max_size; i++ ) { |  |
| palette_predictor_run | ue(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| i += palette_predictor_run – 1 |  |
| i = Min( i, PredictorPaletteSize– 1) |  |
| PalettePredictorEntryReuseFlag[ i ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |

## Constraints on the num_signalled_palette_entries

The size of the current palette is equal to the number of predicted colours + number of signalled colours. This size should not be greater than the max palette size palette_max_size specified in SPS. As shown in Table 1, the current SCC Draft Text ensures that the number of predicted colours < palette_max_size. This contribution further proposes to constrain the number of signalled colours:

- Method 1:

num_signalled_palette_entries specifies the number of entries in the current palette that are explicitly signalled. When num_signalled_palette_entries is not present, it is inferred to be equal to 0. The value of num_signalled_palette_entries shall be in the range of 0 to (palette_max_size – NumPredictedPaletteEntries), inclusive. The variable CurrentPaletteSize specifies the size of the current palette and is derived as follows: CurrentPaletteSize = NumPredictedPaletteEntries + num_signalled_palette_entries (7-2) The value of CurrentPaletteSize shall be in the range of 0 to palette_max_size, inclusive

- Method 2

num_signalled_palette_entries specifies the number of entries in the current palette that are explicitly signalled. When num_signalled_palette_entries is not present, it is inferred to be equal to 0. CurrentCuMaxPaletteSize = min(nCbS * nCbS, palette_max_size) The value of num_signalled_palette_entries shall be in the range of 0 to (CurrentCuMaxPaletteSize – NumPredictedPaletteEntries), inclusive. The variable CurrentPaletteSize specifies the size of the current palette and is derived as follows: CurrentPaletteSize = NumPredictedPaletteEntries + num_signalled_palette_entries (7-3) The value of CurrentPaletteSize shall be in the range of 0 to palette_max_size, inclusive

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

- R. Joshi, S. Liu, J. Xu, and Y. Ye (Text editors), “HEVC Screen Content Coding Draft Text 3,” JCTVC-T1005, Geneva, Switzerland, Feb. 2015