| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19 – 26 June 2015 | Document: JCTVC-U0091 |
| --- | --- |

| Title: | CE1-related: On the mismatch of SCM4.0 and HEVC SCC working draft |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye Shan Liu PoLin Lai Shawmin Lei 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | +1-408-526-1899 jing.ye@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports a mismatch between the SCM 4.0 software and HEVC SCC working draft (WD). In specific, this mismatch is about how “maxPaletteRun” is derived when palette mode is used. Fixes on WD to match SCM 4.0 is provided, as well as fixes on SCM4.0 to match the WD. Simulation results report average up to -0.13% to 0.17% Luma B-D rate changes across all testing conditions and classes when the software is modified to align with the WD.

# Introduction

# In the SCC WD [1], the binarization of palette_run_msb_id_plus1 and palette_run_refinement_bits is constrained by maxPaletteRun. In the WD, maxPaletteRun is specified and highlighted as the following:

Table 1 palette syntax

| maxPaletteRun = nCbS * nCbS – PaletteScanPos – 1 |  |
| --- | --- |
| if( AdjustedMaxPaletteIndex > 0 && ( ( CurrNumIndices + readIndex ) < NumPaletteIndices ¦ ¦ palette_run_type_flag[ xC ][ yC ] != last_palette_run_type_flag ) ) |  |
| if( maxPaletteRun > 0 ) { |  |
| palette_run_msb_id_plus1 | ae(v) |
| if( palette_run_msb_id_plus1 > 1 ) |  |
| palette_run_refinement_bits | ae(v) |
| } |  |
| CurrNumIndices + = readIndex |  |

However, in SCM4.0, the actual maxPaletteRun is derived as following:

maxPaletteRun = nCbS * nCbS – PaletteScanPos –1 – ( NumPaletteIndices – CurrNumIndices )

Thus there is a mismatch between the WD and the software. It is proposed to align them.

# Proposed method

## Modify the software according to the WD

It is proposed to modify the SCM4.0, thus the maxPaletteRun should be derived as the current working draft:

maxPaletteRun = nCbS * nCbS – PaletteScanPos – 1.

## Modify the WD to align with the software

It is proposed to modify the WD according to SCM4.0, and the syntax table should be changed as below :

Table 2 syntax table when align with SCM 4.0

maxPaletteRun = nCbS * nCbS – PaletteScanPos – 1 – ( NumPaletteIndices – CurrNumIndices )

if( AdjustedMaxPaletteIndex > 0 && ( ( CurrNumIndices + readIndex ) < NumPaletteIndices ¦ ¦ palette_run_type_flag[ xC ][ yC ] != last_palette_run_type_flag ) )

if( maxPaletteRun > 0 ) {

palette_run_msb_id_plus1

if( palette_run_msb_id_plus1 > 1 )

palette_run_refinement_bits

}

CurrNumIndices + = readIndex

# Simulation results

The proposed method 2.1 is implemented on top of SCM4.0 and tested using the common test condition defined in [2]. Table 1 and Table 2 are the lossy and lossless results of modifying SCM4.0 according to WD. The anchor is SCM 4.0 software. All the simulations are tested with full frame IBC.

Table 1 Lossy results, 444

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.2% | 0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | 0.4% | 0.0% |
| YUV, Animation, 720p | -0.1% | 0.3% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2 Lossless results,  444

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

# Conclusion

This contribution demonstrates two approaches to fix the mismatch on derivation of “maxPaletteRun” between the SCM4.0 and the current WD. It is suggested to align the next version SCM and WD with one of these two approaches.

# References

- R. Joshi, S. Liu, J. Xu, Y. Ye (editors), “HEVC Screen Content Coding Draft Text 3”, JCTVC-T1005, 20th Meeting: Geneva, Feb 2015.

- H. Yu, R. Cohen, K. Rapaka, J. Xu (editors), “Common Test Conditions for Screen Content Coding”, JCTVC-T1015, 20th Meeting: Geneva, Feb 2015.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).