| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0092_r1 |
| --- | --- |

| Title: | Draft Text modifications on palette |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai, Jing Ye, Jungsun Kim, Xiaozhong Xu, Shan Liu, and Shawmin Lei 2860 Junction Ave. San Jose, CA 95134, USA | Email: | { polin.lai, jing.ye, jungsun.kim, xiaozhong.xu, shan.liu, shawmin.lei } @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents modifications to the HEVC SCC Draft Text related to palette. The modifications are asserted to: 1. Fix errors in the Draft Text. 2. Resolve mismatches between SCM software and Draft Text. 3. Remove not referred parts. In the 2-nd version of the document, two modifications are added to reflect the in meeting discussions.

# Proposed modifications

## Fixes related to AdjustedMaxPaletteIndex

When AdjustedMaxPaletteIndex is 0, palette index is not parsed, and is inferred as 0. Semantically, it is still regarded as there is a palette index, with value being 0. The current HEVC SCC Draft Text 3 [1] does not correctly capture this operation.

To fix this issue, the following modifications are proposed.

Note that the changes would also align the Text with the SCM software.

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| if( MaxPaletteIndex > 0) { |  |
| palette_transpose_flag | ae(v) |
| num_palette_indices_idc | ae(v) |
| for( i=0; i < NumPaletteIndices; i++ ) { |  |
| if(AdjustedMaxPaletteIndex > 0 ) |  |
| { |  |
| palette_index_idc | ae(v) |
| PaletteIndexIdc[ i ] = palette_index_idc |  |
| } |  |
| else |  |
| PaletteIndexIdc[ i ] = 0 |  |
| } |  |
| last_palette_run_type_flag | ae(v) |
| } |  |
| CurrNumIndices = 0 |  |
| PaletteScanPos = 0 |  |
| while( PaletteScanPos < nCbS * nCbS ) { |  |
| … |  |
| … |  |
| if( MaxPaletteIndex > 0 && CurrNumIndices < NumPaletteIndices ) { |  |
| if( PaletteScanPos >= nCbS && palette_run_type_flag[ xcPrev ][ ycPrev ] != COPY_ABOVE_MODE && PaletteScanPos < nCbS * nCbS – 1) { |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| } |  |
| readIndex = 0 |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE && AdjustedMaxPaletteIndex > 0) |  |
| readIndex = 1 |  |
| maxPaletteRun = nCbS * nCbS – PaletteScanPos – 1 |  |
| if( AdjustedMaxPaletteIndex > 0 && ( ( CurrNumIndices + readIndex ) < NumPaletteIndices ¦ ¦ palette_run_type_flag[ xC ][ yC ]!= last_palette_run_type_flag ) ) |  |
| if( maxPaletteRun > 0 ) { |  |
| palette_run_msb_id_plus1 | ae(v) |
| if( palette_run_msb_id_plus1 > 1 ) |  |
| palette_run_refinement_bits | ae(v) |
| } |  |
| CurrNumIndices + = readIndex |  |
| } |  |
| … |  |
| … |  |
| } |  |
| … |  |
| } |  |

## Fix of derived CurrPaletteIndex value when MaxPaletteIndex = 0

When MaxPaletteIndex = 0 (current CU’s palette has only 1 colour and no Escape, or, current CU all samples are Escape with empty palette), the palette indices for all samples are 0. The parsing of palette indices will be skipped in this situation, leading to empty PaletteIndexIdc[] array in HEVC SCC Draft Text 3 [1]. As a results, the Draft Text does not specify the palette index value when MaxPaletteIndex = 0.

To fix this, the following change is proposed.

Note that the changes would also align the Text with the SCM software.

- 7.4.9.6 Palette mode semantics … … When PaletteSampleMode[ xC ][ yC ] is not equal to COPY_ABOVE_MODE, the variable CurrPaletteIndex is derived as follows: if (MaxPaletteIndex > 0) CurrPaletteIndex = PaletteIndexIdc[CurrNumIndices] if( CurrPaletteIndex >= adjustedRefPaletteIndex ) CurrPaletteIndex + = 1 Otherwise, CurrPaletteIndex = 0 …

## Fixes of syntax palette_run_type_flag and array PaletteSamleMode

- In HEVC SCC Draft Text 3, array position palette_run_type_flag[ xcPrev ][ ycPrev ] is used to determine whether to parse palette_run_type_flag, and also to determine the value of adjustedRefPaletteIndex. However the positions palette_run_type_flag[ xcPrev ][ ycPrev ] are not populated with the correct palette run type values. Instead, the values are populated in the array PaletteSampleMode[][].

- Furthermore, the array palette_run_type_flag[][] uses the same name as the syntax element palette_run_type_flag.

- Also in Section 7.4.9.6. Palete mode semantics, the actual value of palette_run_type_flag is not specified.

To fixed the above issues, it is proposed to use one populated array PaletteSampleMode[][], and not mixing syntax name with array name.

The values of palette_run_type_flag is specified below according to the SCM software.

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| while( PaletteScanPos < nCbS * nCbS ) { |  |
| … |  |
| … |  |
| if( MaxPaletteIndex > 0 && CurrNumIndices < NumPaletteIndices ) { |  |
| if( PaletteScanPos >= nCbS && PaletteSampleModepalette_run_type_flag[ xcPrev ][ ycPrev ] != COPY_ABOVE_MODE && PaletteScanPos < nCbS * nCbS – 1) { |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| } |  |
| readIndex = 0 |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE ) |  |
| readIndex = 1 |  |
| maxPaletteRun = nCbS * nCbS – PaletteScanPos – 1 |  |
| if( ( ( CurrNumIndices + readIndex ) < NumPaletteIndices ¦ ¦ palette_run_type_flag[ xC ][ yC ]!= last_palette_run_type_flag ) ) |  |
| if( maxPaletteRun > 0 ) { |  |
| palette_run_msb_id_plus1 | ae(v) |
| if( palette_run_msb_id_plus1 > 1 ) |  |
| palette_run_refinement_bits | ae(v) |
| } |  |
| CurrNumIndices + = readIndex |  |
| } |  |
| runPos = 0 |  |
| while ( runPos < = paletteRun ) { |  |
| xR = x0 + travScan[ PaletteScanPos ][ 0 ] |  |
| yR = y0 + travScan[ PaletteScanPos ][ 1 ] |  |
| if(palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE ) { |  |
| PaletteSampleMode[ xR ][ yR ] = COPY_INDEX_MODE |  |
| PaletteIndexMap[ xR ][ yR ] = CurrPaletteIndex |  |
| } else { |  |
| PaletteSampleMode[ xR ][ yR ] = COPY_ABOVE_MODE |  |
| PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xR ][ yR − 1 ] |  |
| } |  |
| runPos++ |  |
| PaletteScanPos++ |  |
| } |  |
| } |  |
| … |  |
| … |  |
| } |  |

- 7.4.9.6 Palette mode semantics palette_run_type_flag[ xC ][ yC ] equal to 1 (COPY_ABOVE_MODE) specifies that the palette index is equal to the palette index at the same location in the row above. palette_run_type_flag[ xC ][ yC ] equal to 0 (COPY_INDEX_MODE) specifies that an indication of the palette index of the sample is coded in the bitstream. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the picture. The variable CurrNumIndices specifies the number of palette runs of type COPY_INDEX_MODE up to but not including the current scan position. When palette_run_type_flag is not present, it is inferred as follows: - If CurrNumIndices is equal to NumPaletteIndices and MaxPaletteIndex is greater than 0, palette_run_type_flag is inferred to be equal to 1 (COPY_ABOVE_MODE). - Otherwise ( CurrNumIndices is less than NumPaletteIndices or MaxPaletteIndex is equal to 0 ) palette_run_type_flag is inferred to be equal to 0 (COPY_INDEX_MODE). The variable AdjustedMaxPaletteIndex is derived as follows: AdjustedMaxPaletteIndex = MaxPaletteIndex if( PaletteScanPos > 0 ) AdjustedMaxPaletteIndex − = 1 The variable PaletteIndexMap[ xC ][ yC ] specifes a palette index, which is an index to the array represented by CurrentPaletteEntries. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the picture. The value of PaletteIndexMap[ xC ][ yC ] shall be in the range of 0 to MaxPaletteIndex, inclusive. The variable adjustedRefPaletteIndex is derived as follows: adjustedRefPaletteIndex = MaxPaletteIndex + 1 if( PaletteScanPos > 0 ) { xcPrev = xC + travScan[PaletteScanPos – 1][0] ycPrev = yC + travScan[PaletteScanPos – 1][1] if(PaletteSampleMode palette_run_type_flag [xcPrev][ycPrev] != COPY_ABOVE_MODE ) { xcPrev = xC + travScan[PaletteScanPos – 1][0] ycPrev = yC + travScan[PaletteScanPos – 1][1] adjustedRefPaletteIndex = PaletteIndexMap[xcPrev][ycPrev] (7-1) } else adjustedRefPaletteIndex = PaletteIndexMap[xC][yC − 1] … … The variable PaletteSampleMode[ xC ][ yC ] specifies whether the palette index is copied from the palette index in the row above or explicitly coded in the bitstream. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the picture. The value of PaletteSampleMode[ xC ][ yC ] shall be one of COPY_ABOVE_MODE and COPY_INDEX_MODE. If PaletteIndexMap[ xC ][ yC ] is equal to MaxPaletteIndex and palette_escape_val_present_flag is equal to 1, the sample is coded in escape mode and PaletteSampleMode[ xC ][ yC ] is set equal to ESCAPE_MODE. The variable paletteRun specifies the number of consecutive locations minus 1 with the same palette index as the position in the above row when palette_run_type_flag is equal to COPY_ABOVE_MODE or specifies the number of consecutive locations minus 1 with the same palette index when palette_run_type_flag is equal to COPY_INDEX_MODE.

## Other modifications

- Palette mode semantics last_palette_last_run_type_flag specifies the last occurence of the palette_run_type_flag within the block. When last_palette_last_run_type_flag is not present, it is inferred to be equal to COPY_INDEX_MODE.

- 9.3.4.2.8 Derivation process of ctxInc for the syntax element palette_run_type_flag and last_palette_run_type_flag Inputs to this process are the current sample location ( xC, yC ) specifying the sample relative to the top-left luma sample of the picture, and previously decoded syntax element palette_run_type_flag. Output of this process is the variable ctxInc. The variable ctxInc is derived as follows: # ctxInc = (palette_run_type_flag[xC][y−1] == COPY_ABOVE_MODE) ? 1: 0 (9-2)

## Constraint on palette_predictor_run

- Impose conformace constraints on the parsed syntax palette_predictor_run:

(palette_predictor_run -1) < PredictorPaletteSize - i , where i is the current position before parsing the current palette_predictor_run.

Or

- Impose conformace constraints on the position after adding the parsed syntax palette_predictor_run:

i + (palette_predictor_run -1) < PredictorPaletteSize , where i is the current position before parsing the current palette_predictor_run.

Which can also be expressed as:  ( i+= palette_predictor_run -1 ) should be smaller than PredictorPaletteSize.

## Constraint on number of indices

In the 21-st meeting, coding the NumPaletteIndices is simplified to simply code the (NumPaletteIndices – 1), So NumPaletteIndices = num_palette_indices_idc + 1

Syntax num_palette_indices_idc indicates number of palette indices signalled for the current block is always smaller or equal to current block size. In this case, the syntax description can be as following :

num_palette_indices_idc is an indication of the number of palette indices signalled for the current block. num_palette_indices_idc shall be in the range of 0 to nCbS * nCbS -1, inclusive.When num_palette_indices_idc is not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

- R. Joshi, S. Liu, J. Xu, and Y. Ye (Text editors), “HEVC Screen Content Coding Draft Text 3,” JCTVC-T1005, Geneva, Switzerland, Feb. 2015