| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0093 |
| --- | --- |

| Title: | Non-CE1: On Number of Palette Indices coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Kiran Misra, Jie Zhao, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 7531 kimse@sharplabs.com misrak@sharplabs.com jzhao@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In current SCC draft, num_palette_indices_idc is partitioned into 3 non-contiguous sets and the variable NumPaletteIndices is derived using an equation selected based on the set membership of num_palette_indices_idc. The existing derivation process is asserted to be complex. This contribution proposes the use of a fixed predictor (2* MaxPaletteIndex) and a signaled residual to derive NumPaletteIndices. This approach is asserted to be much simpler. It is reported that the proposed simplification provides a maximum luma bit rate difference of 0.1% for all configurations including AI, RA, LD(B) for lossy and lossless, and for 444 and 420 format.

# Introduction

In current SCC draft, a syntax element num_palette_indices_idc is signalled and a variable NumPaletteIndices, which indicates the number of palette indices for current CU block, is derived from the given num_palette_indices_idc.

As shown in Figure 1, the relation between NumPaletteIndices and num_palette_indices_idc is complexly defined. num_palette_indices_idc is partitioned into 3 non-contiguous sets. For each set a different equation is used for the derivation of NumPaletteIndices. A simplified derivation of NumPaletteIndices and associated signaling is proposed in section 2.

Figure 1 Mapping from Num_palette_indices_idc to NumPaletteIndices (when MaxPaletteIndex is equal to 31)

| Current SCC Draft | Proposed Method |
| --- | --- |
| Receive: num_palette_indices_idc if( num_palette_indices_idc >= ( MaxPaletteIndex – 1) * 32 ) NumPaletteIndices = num_palette_indices_idc + 1 else if(num_palette_indices_idc % 32 = = 31) NumPaletteIndices = MaxPaletteIndex – (num_palette_indices_idc + 1 ) / 32 else NumPaletteIndices = ( num_palette_indices_idc / 32 ) * 31 ) + (num_palette_indices_idc % 32) +MaxPaletteIndex | Receive: num_palette_indices_res_abs If (num_palette_indices_res _abs < 2*MaxPaletteIndex && num_palette_indices_res _abs) Receive: num_palette_indices_res_sign_flag NumPaletteIndices = num_palette_indices_res + (MaxPaletteIndex <<1) |

Figure 2 The number of palette indices coding

# Proposed Method

It is proposed to use a fixed predictor (2* MaxPaletteIndex) and a signaled residual to derive NumPaletteIndices as follows:

NumPaletteIndices = NumPaletteIndicesResidual + 2* MaxPaletteIndex

The residual NumPaletteIndicesResidual is signaled using absolute value and conditional sign signalling. Since NumPaletteIndices is greater than zero the sign information (num_palette_indices_res_sign_flag) only needs to be sent for the following condition.

if(num_palette_indices_res _abs < 2*MaxPaletteIndex && num_palette_indices_res _abs)        (1)

Otherwise sign information can be inferred to be a positive. In Figure 2, proposed method is compared to current SCC Draft. The coding efficiency impact of the proposed change is found to be negligible and is listed in section 3. The SCC draft changes are listed in section 7.

# Experimental Results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM4.0, and the common test conditions specified in [3] are followed. The compression performance is measured using BD rate compared with SCM-4.0 anchor.

Table1. BD rate comparison in 444 lossy (IBC full frame mode)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U |  | R/V |  |
| RGB, text & graphics 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  | 0.0% |  | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  | 0.2% |  | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.1% |  | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics 1080p & 720p | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | -0.1% | 0.1% |  | 0.0% |  | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | -0.1% | 0.0% |  | 0.5% |  | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% |  | 0.1% |  | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.1% |  | 0.0% |

Table2.  BD rate comparison in 444 lossy (IBC 1×4 CTUs mode)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U |  | R/V |  |
| RGB, text & graphics 1080p & 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  | 0.1% |  | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.1% |  | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics 1080p & 720p | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |  | 0.1% |  | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% |  | -0.1% |  | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% |  | -0.3% |  | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% |

Table3.  BD rate comparison in 420 lossy (IBC full frame mode)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U |  | R/V |  |
| YUV, text & graphics 1080p & 720p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  | -0.2% |  | -0.3% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |  | 0.2% |  | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |  | -0.1% |  | 0.0% |

Table4.  BD rate comparison in 444 lossless

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |

Table5.  BD rate comparison in 420 lossless

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |

# Conclusion

It is recommended that the JCT-VC adopt the proposed method of coding number of palette indices in the SCC draft.

# References

[1] M. Karczewicz, W. Pu, V. Seregin, R. Joshi, “Non CE1: Grouping Palette Indices At Front”, JCTVC-T0065, Geneva, CH, February 2015.

[2] R. Joshi, J. Xu, S. Liu, Y. Ye, “High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 3,” JCTVC-T1005, 20th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11: Geneva, CH, 10 February – 17 February 2015

[3] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, JCTVC-T1015, Geneva, CH, February 2015.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes

7.3.8.8 Palette syntax

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| … |  |
| if( MaxPaletteIndex > 0) { |  |
| palette_transpose_flag | ae(v) |
| num_palette_indices_res_abs | ae(v) |
| if( num_palette_indices_res_abs < 2*MaxPaletteIndex && num_palette_indices_res_abs > 0 ) |  |
| num_palette_indices_res_sign_flag | u(1) |
| for( i=0; i < NumPaletteIndices; i++ ) { |  |
| palette_index_idc | ae(v) |
| PaletteIndexIdc[ i ] = palette_index_idc |  |
| } |  |
| last_palette_run_type_flag | ae(v) |
| …. |  |

num_palette_indices_res_abs specifies the absolute value of the residual of the number of palette indices.  When num_palette_indices_res_abs is not present, it is inferred to be equal to 0.

num_palette_indices_res_sign_flag specifies the sign of the residual of the number of palette indices as follows:

- If num_palette_indices_res_sign_flag equal to 0, the corresponding number of palette indices residual has a positive value.

- Otherwise (num_palette_indices_res_sign_flag equal to 1),  the corresponding number of palette indices residual has a negative value.

When num_palette_indices_res_sign_flag is not present, it is inferred to be equal to 0.

The value of residual of the number of palette indices is derived as follows:

NumPaletteIndicesResidual = 
			( 1 − 2 * num_palette_indices_idcres_sign_flag) * num_palette_indices_res_abs

The variable NumPaletteIndices specifies the number of palette indices signalled for the current block and is derived as follows:

NumPaletteIndices = NumPaletteIndicesResidual + 2* MaxPaletteIndex

#### 9.3.3.15 Binarization process for num_palette_indices_idc

…

cRiceParam = 1 + MaxPaletteIndex / 6