| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0094r1 |
| --- | --- |

| Title: | Non-CE1: Palette Run Hiding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Wei Pu, Rajan Joshi, Feng Zou, Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 martak@qti.qualcomm.com Wei.Pu@qti.qualcomm.com rajanj@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

It is observed that in palette mode, the first or the last run in the block is generally longer than the average run length for the block. In this document, it is proposed to use a flag to indicate whether the first or the last run length is not coded in the bitstream. It is reported that for RGB and YUV text & graphics with motion, 0.2% and 0.3% BD-rate reduction are achieved, respectively, for All-Intra 4 CTU configuration, 0.1% and 0.2 BD-rate reduction are achieved, respectively, for All-Intra full frame configuration.

# Introduction

In palette mode, it is observed that the first or the last run is usually longer than the average run length for the block. To exploit this character, a flag is proposed to indicate whether the first or the last run in the block is not signalled to improve coding efficiency.

# Technical description

A new syntax element, palette_run_hide_flag is signalled in the bitstream. It equal to 1 specifies that the first occurrence of the palette run length is not signalled. It equal to 0 specifies that the last occurrence of the palette run length is not signalled. Table I illustrates such modification in palette syntax table.

Table I. Syntax change for palette_run_hide_flag

| … |  |
| --- | --- |
| if( MaxPaletteIndex > 0) { |  |
| palette_transpose_flag | ae(v) |
| palette_run_hide_flag | ae(v) |
| num_palette_indices_idc | ae(v) |
| for( i=0; i < NumPaletteIndices; i++ ) { |  |
| palette_index_idc | ae(v) |
| PaletteIndexIdc[ i ] = palette_index_idc |  |
| } |  |
| last_palette_run_type_flag | ae(v) |
| } |  |
| … |  |

When the first or the last run length is not signalled, it is inferred at the decoder side. Specifically, if  palette_run_hide_flag equals to 0, i.e. the last run length is not signalled, the decoding procedure is the same as the current draft standard [1].

If the palette_run_hide_flag equals to 1, the decoder first assign zero run length to the first run and continues to decode the rest of the runs in the block. After knowing the values of the rest runs, the difference between the number of pixels in the block and the sum of the decoded run lengths is assigned to be the length of the first run.

# Experimental results

Simulation results are carried out using SCC common test condition [2]. Only 4:4:4 all intra lossy results are presented in this section. Complete test results are included in the contribution package.

Table II. Proposed method v.s. SCM4.0 (Full frame intra block copy search)

Table III. Proposed method vs. SCM4.0 (4 CTU intra block copy search)

# References

- R. Joshi, S. Liu, J. Xu, and Y. Ye, “HEVC screen content coding draft text 3,” in JCTVC-T1005, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” in JCTVC-T1015, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).