| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0095r1 |
| --- | --- |

| Title: | Non-CE: Encoder modification for intra prediction mode using adaptive colour transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Chun-Lung Lin, Jih-Sheng Tu, and 3Ching-Chieh Lin 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

In SCM4.0, the intra prediction mode can enable the adaptive colour transform (ACT) to perform in-loop colour space conversion in the prediction residual domain. Because the ACT is adaptively enabled/disabled by a CU-level flag, it increases the encoder complexity to evaluate whether to enable the ACT. This contribution proposes to reduce the encoder complexity by disallowing the ACT-based intra prediction mode in 64x64 CU when the video format is YUV colour space. Besides, it further proposes to disallow the TU splitting for the ACT-based intra prediction in large CUs. Compared to the SCM4.0 anchor, it is reported that the proposed modification can save 5~8% encoding time in AI condition with minor gain change.

# Introduction

HEVC SCC adopts adaptive colour transform (ACT) [1] to convert one color-space to another one wherein the correlations among three colour components are further reduced. In SCM4.0, ACT is performed following the intra prediction mode, inter prediction mode and intra block copy mode. To evaluate whether to enable the ACT for a coding mode, the encoder requires an additional RDO check to evaluate the RD cost of the mode enabling ACT. Therefore, the evaluation of ACT doubles the encoder complexity. To reduce the encoder complexity, this contribution proposes two non-normative changes on intra prediction mode.

# Proposed coding

According to the simulation results in JCTVC-T0080 [2], it is observed that the ACT has no substantial gain in the AI-lossy condition under the YUV video format. Besides, if the largest coding unit (CU) uses the intra prediction mode for screen visual content, it may imply that the content in the area is not complex, and it may be not necessary to enable the ACT to further decorrelate three colour components. Therefore, the first proposed modification in the encoder is to disallow the ACT for intra prediction mode in the largest CU when the video format is based on YUV colour space.

With the help of ACT, the resulting residues in the transform domain may be able to be concentrated into a smaller number of values. Besides, if a large CU uses the intra prediction mode for screen visual content, it may imply that the content in that area is not complex, and it may be not necessary to further evaluate the smaller sizes of TU. Therefore, the second encoder modification proposed in this contribution is to disallow TU partition for ACT-based intra prediction mode in some large CUs.

The combination of the proposed modifications leads to significant encoder speedup with minimal gain change.

The encoder in SCM4.0 uses two functions for RDO checks: one is the function “xCheckRDCostIntra”, and the other is “xCheckRDCostIntraCSC”, where the former is the RDO evaluation for the intra prediction mode disabling ACT, and the latter is the RDO evaluation for the intra prediciton mode enabling ACT. The following summarizes the proposed encoder modifications in SCM4.0:

- Modification 1: Disable “xCheckRDCostIntraCSC” when the size of the current CU is 64x64 and the input video format is based on YUV colour space.

- Modification 2: Disable TU splitting in “xCheckRDCostIntraCSC” when the current CU is one of the sizes of 64x64 to NxN, where N can be 64, 32, 16, 8 or 4.

# Simulation results

The proposed encoder modification is simulated on top of the SCM 4.0 anchor under the common test condition [3]. The following tables are the BD-rate results compared to the anchor under the case of full-frame IBC and 444 chroma format. The simulation results for other cases can be found in the attached spreadsheets. Table 1 shows the results of the proposed encoder with N = 32 on the condition of lossy coding. The results show that it saves 5% encoding time with 0.1% coding gain in AI condition. Table 2 shows the results of the proposed encoder with N = 16 on the condition of lossy coding. The results show that it saves 8% encoding time with 0.1% coding loss in AI condition.

Table 1. Lossy results for the proposed encoder with N = 32 on top of the anchor under the case of full-frame IBC and 444 chroma format

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.2% | -0.1% |
| YUV, Animation, 720p | -0.1% | 0.3% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2. Lossy results for the proposed encoder with N = 16 on top of the anchor under the case of full-frame IBC and 444 chroma format

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.2% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

This contribution proposes to improve the encoding complexity for SCM by disallowing the ACT-based intra prediction mode in 64x64 CU and disallowing the TU splitting for the ACT-based intra prediction mode in large CUs. The results show that 5~8% saving in encoding time can be achieved with minor gain change. It is recommended to adopt this contribution to speed-up the next version of SCM.

# Reference

- R. Joshi, S. Liu, J. Xu, Y. Ye, “HEVC Screen Content Coding Draft Text 3,” 20th JCT-VC meeting, JCTVC-T1005, Geneva, Switzerland, Feb. 2015.

- P. Lai, S. Liu, S. Lei, “AHG6: Coding performance of SCC tools and suggested common test conditions,” 20th JCT-VC meeting, JCTVC-T0080, Geneva, Switzerland, Feb. 2015.

- H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common Test Conditions for Screen Content Coding,” 20th JCT-VC meeting, JCTVC-T1015, Geneva, Switzerland, Feb. 2015.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)