| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0096 |
| --- | --- |

| Title: | CE1 Related: Improved palette encoder |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Feng Zou Vadim Seregin Wei Pu Rajan Joshi 5775 Morehouse Dr. San Diego, CA, USA | Tel: Email: | +1-858-658-5673 martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes encoder only changes for palette mode in the current SCM-4.0 software. The changes include: 1) using squared error to derive the palette and for index assignment; 2) eliminating some of the palette entries if the related pixels can be represented by other palette indices; 3) merging of two consecutive runs and jointly optimizing the run values of index run and copy above run pair. Experimental results show that compared to SCM-4.0 anchor on 444 sequences, the proposed combination reportedly provides the average 1.3% and 1.8% BD-rate savings for the category text & graphics with motion, 1080p & 720p in both RGB and YCbCr color formats respectively in AI with IBC full frame. And 1.5% and 2.2% BD-rate savings for the category text & graphics with motion, 1080p & 720p in both RGB and YCbCr color formats respectively in AI with IBC 4CTU. For 420 sequences, the corresponding BD-rate savings of the proposed method are 1.2% for AI.

# Introduction

In the current SCM-4.0 [1], the palette is derived using SAD between possible palette entry and pixel values. Similarly, index assignment is determined by minimizing the SAD between the derived palette entry and pixel values. During the run derivation in index mode, the maximum possible run is checked.

# Proposed techniques

## Proposed SSE in palette derivation and index assignment

In the proposed scheme, index derivation and assignment are performed using SSE. Specifically, SSE is used to compare palette predictor and derived palette.  And during palette index assignment for each pixel, SSE is used to determine the optimal index which minimizes the SSE.

## Proposed palette index pruning

In the proposed scheme, index pruning is applied after selecting the best scanning direction (horizontal or vertical). Specifically, given the run information for the best scanning order, index pruning is applied. In particular, when pixels with the current palette index can be represented by another index with a smaller RD cost, the current palette index is removed. After palette pruning process, the run values are re-estimated for the updated palette table.

## Proposed merging of two consecutive runs

It is proposed to merge two consecutive runs, coded with index mode, into one index with a run value equal to the sum of two runs or one copy above runor one copy above mode with the same increased run value if the RD cost of such merging is smaller than the two initial RD costs. This process effectively changes the initial index assignment for each pixels involved in the particular run.

## Proposed joint selection of index and copy above runs

In the current SCM-4.0, the index run is calculated as the largest number of consecutive pixels with the same index value. However, it may not be optimal in the RD sense, since the decision on the current run may affect the following run. To achieve better efficiency, it is proposed to adaptively determine the connection point between an index run and a following copy above run, based on the encoder idea proposed in CE1 test C1. Basically, the rate cost of the selected index run and copy above run pairs is checked for the run values which may change the index run or above run binarization. As a result, the best connection point, i.e. new run values for index and copy above runs, is determined by minimizing the rate cost of coding these two consecutive runs.

# Simulation results

The proposed scheme is tested using SCC common test conditions [2] based on SCM-4.0. Table 1 shows the results under the full frame IBC condition, while Table 2 shows the results under the 1×4 CTU IBC condition.

Table 1. Results of the proposed encoder with full frame IBC

Table 2. Results of the proposed encoder with 1x4 CTU IBC

# Reference

[1]. Available at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0

[2]. H. Yu, R. Cohen, K. Rapaka, J. Xu (editors), “Common Test Conditions for Screen Content Coding”, JCTVC-T1015, Geneva, Switzerland.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).