| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0097r2 |
| --- | --- |

| Title: | Non-CE1: On maximum palette predictor size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Ching-Chieh Lin, 3Chun-Lung Lin, and Jih-Sheng Tu 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2JackLin@itri.org.tw 3Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

Although the SCM4.0 sets the maximum palette predictor size as 128, there is no upper limitation for the maximum palette predictor size in the SCC draft text. The unlimited sizes of the palette predictors cause huge memory buffer on the hardware design and huge computational cost due to vast matching operations. Therefore, this contribution presents four optional constraints and a cleanup for the palette predictor size. The contribution also reports the rate-distortion evaluation on different maximum palette predictor sizes, whose numbers are smaller than 128. It is reported that smaller values of the maximum palette predictor size such as 80, 96 and 112 only change 0.0-0.2% coding performance on All-Intra lossy coding and improves the coding time up to 1%.

# Constraint on PaletteMaxPredictorSize

In the current SCC draft [1], there is no upper limitation for the palette predictor size. A rogue encoder can signal a huge value for delta_palette_max_predictor_size to derive a large value for PaletteMaxPredictorSize. This results in a hardware implementation issue for allocating such a big palette predictor buffer at the decoder side. Therefore, the following four optional constrains are proposed to modify the text in Section 7.4.3.2.3 in [1] (changes marked in yellow):

## Optional constraint 1.1

delta_palette_max_predictor_size specifies the difference between the maximum allowed palette predictor size and the maximum allowed palette size. When not present, the value of delta_palette_max_predictor_size is inferred to be 0. The variable PaletteMaxPredictorSize  is derived as follows:

PaletteMaxPredictorSize = palette_max_size + delta_palette_max_predictor_size	(7-1)

The value of PaletteMaxPredictorSize shall be in the range of 0 to 128, inclusive.

## Optional constraint 1.2

delta_palette_max_predictor_size specifies the difference between the maximum allowed palette predictor size and the maximum allowed palette size. When not present, the value of delta_palette_max_predictor_size is inferred to be 0. The variable PaletteMaxPredictorSize  is derived as follows:

PaletteMaxPredictorSize = palette_max_size + delta_palette_max_predictor_size	(7-2)

The value of PaletteMaxPredictorSize shall be in the range of 0 to 96, inclusive.

## Optional constraint 2.1

delta_palette_max_predictor_size specifies the difference between the maximum allowed palette predictor size and the maximum allowed palette size. When not present, the value of delta_palette_max_predictor_size is inferred to be 0. The value of delta_palette_max_predictor_size shall be in the range of 0 to (palette_max_size << 1), inclusive. The variable PaletteMaxPredictorSize  is derived as follows:

PaletteMaxPredictorSize = palette_max_size + delta_palette_max_predictor_size	(7-3)

## Optional constraint 2.2

delta_palette_max_predictor_size specifies the difference between the maximum allowed palette predictor size and the maximum allowed palette size. When not present, the value of delta_palette_max_predictor_size is inferred to be 0. The value of delta_palette_max_predictor_size shall be in the range of 0 to (palette_max_size + palette_max_size >> 1), inclusive. The variable PaletteMaxPredictorSize  is derived as follows:

PaletteMaxPredictorSize = palette_max_size + delta_palette_max_predictor_size	(7-4)

# Cleanup on PredictorPaletteSize

Since max_predictor_palette_size is undefined in the palette mode semantics (see Section 7.4.9.6 in the current SCC draft [1]), a cleanup for the value of PredictorPaletteSize is proposed as bellow to modify the text in Section 7.4.9.6 in [1] (changes marked in yellow):

The value of PredictorPaletteSize shall be in the range of 0 to max_predictor_palette_size PaletteMaxPredictorSize, inclusive.

# Comments on maximum palette size and maximum palette predictor size

In SCM3.0, the maximum palette size is 31 and maximum palette predictor size is 64 [1][2]. In JCT-VC’s 20th meeting, the maximum palette size was changed to 63 and the maximum palette predictor size was changed to 128. However, there is no upper limitation for the maximum palette predictor size in the SCC draft text. The unlimited sizes of the palette predictors will cause huge memory buffer on the hardware design and huge computational cost due to vast matching operations between the current palette table and the palette predictors. There are several ways to solve the raised issues. One is to modify the syntax elements on PaletteMaxPredictorSize or delta_palette_max_predictor_size as the proposed constraints. The second one is to have a slice level signalling to control the maximum palette predictor size based on the qp value. The third one is to use a doubled number of the averaged palette size from the previous slices to set up the maximum palette predictor size.

# Performance evaluation on PaletteMaxPredictorSize

The SCM4.0 sets the maximum palette predictor size as 128. To understand the performance impacts by modifying the maximum size of the palette predictors in the SCM4.0, this contribution further evaluates the smaller numbers including 80, 96 and 112 for the maximum palette predictor size. The evaluation on PaletteMaxPredictorSize is investigated under the common test condition [3]. The following tables report the AI lossy experimental results for full frame IBC search area. Table 1 to Table 3 show the results for the maximum palette predictor size from 80, 96 to 112 respectively. The details for the simulation results could be found in the accompanied spread sheets. It is reported that smaller numbers for the maximum palette predictor size only change 0.0-0.2% coding performance on All-Intra lossy coding and improves the coding time up to 1%.

Table 1: Lossy results for the maximum palette predictor size of 80

Table 2: Lossy results for the maximum palette predictor size of 96

Table 3: Lossy results for the maximum palette predictor size of 112

# Conclusions

Four optional constraints for the upper limitation of the palette predictor size and one text cleanup are proposed. Some comments are also raised following the issues. The contribution also reports the rate-distortion evaluation on different maximum palette predictor sizes, whose numbers are smaller than 128. It is reported that smaller numbers such as 80, 96 and 112 for the maximum palette predictor size only change 0.0-0.2% coding performance on All-Intra lossy coding and improves the coding time up to 1%. It is also asserted that smaller numbers for the maximum palette predictor size can save much more memory buffer.

# References

- R. Joshi, J. Xu, S. Liu, Y. Ye, "High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 3", 20th JCT-VC meeting, JCTVC-T1005, Geneva, Switzerland, Feb. 2015

- SCM-4.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” 20th JCT-VC meeting, JCTVC-T1015, Geneva, Switzerland, Feb. 2015

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).