| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0098 |
| --- | --- |

| Title: | Dynamic Range Adjustment SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Bugdayci Sansli A. K. Ramasubramonian D. Rusanovskyy S. Lee J. R. Sole M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Email: | dbsansli@qca.qualcomm.com dmytror@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a new SEI message for carriage of metadata for use in post processing stages of HEVC-based HDR/WCG content delivery. The name is the proposed SEI message is dynamic range adjustment SEI message.

# Introduction and problem definition

“HDR10” is an end-to-end representation and coding format which allows compression and delivery of High Dynamic Range (HDR) and Wide Color Gamut (WCG) content using the state-of-the art compression standard, HEVC Main 10 Profile, [1][2]. It has been recently adopted by several organizations including the Blu-ray Disc Association (BDA), the High-Definition Multimedia Interface (HDMI) Forum and the Ultra-High Definition (UHD) Alliance.

At the last meeting, MPEG initiated a Call for Evidence (CfE) activity to study further improvement the HDR/WCG content delivery. The HDR10 specified solution was utilized to define an anchor methodology in this CfE [3].. This CfE is not only calling upon new coding tools but also interested in new methodologies and algorithms that would improve the end-to-end performance by modifying or replacing the pre and post processing operations.

Figure 1 illustrates the end-to-end processing chain for the delivery of HDR/WCG content using HEVC Main 10 Profile as specified in MPEG CfE. HEVC Main 10 Profile sufficiently supports the delivery of the metadata related to the pre and post processing operations shown in Figure 1. Operability with the existing codec is an important aspect of the solution format for the easy deployment of HDR/WCG systems. Similarly, improvements of pre/post processing chain can be supported by HEVC through the introduction of new SEI messages for the delivery of related metadata.

Fig. 1. End to end coding and decoding chain defined in CfE by MPEG for the compression of High Dynamic Range / Wide Color Gamut videos.

# Proposal - 1

It’s shown in [4] that changes applied in the pre/post processing stage could bring performance benefit in terms of objective metrics. Hence for the delivery of parameters used in [4], we propose Dynamic Range Adjustment SEI message whose syntax and semantics are shown below.

Syntax:

| dynamic_range_adjustment ( payloadSize ) { | Descriptor |
| --- | --- |
| dra_cancel_flag | u(1) |
| dra_range_id | ue(v) |
| if( !dra_cancel_flag ) { |  |
| dra_persistence_flag | u(1) |
| dra_domain_idc | ue(v) |
| dra_num_comp_minus1 | ue(v) |
| offset_present_flag | u(1) |
| for( c=0; c <= dra_num_comp_minus1; c++) { |  |
| scale[ c ] | u(20) |
| if( offset_present_flag ){ |  |
| offset_sign[ c ] | u(1) |
| offset_magn[ c ] | u(10) |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics:

The dynamic range adjustment SEI message provides information to be used in the post-processing of the reconstructed samples of the output pictures. The process of scaling of the decoded colour components of output picture is an inverse operation of Dynamic Range Adjustment performed by encoder.

dra_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous dynamic range adjustment SEI message in output order that applies to the current layer. dra_cancel_flag equal to 0 indicates that dynamic range adjustment information follows.

dra_range_id contains an identifying number that may be used to identify the purpose of the one or more dynamic range adjustment SEI messages. The value of dra_range_id shall be in the range of 0 to 232 − 2, inclusive.

dra_persistence_flag specifies the persistence of the dynamic range adjustment information SEI message for the current layer. dra_persistence_flag equal to 0 specifies that the dynamic range adjustment applies to the current picture only. Let picA be the current picture. dra_persistence_flag equal to 1 specifies that the dynamic range adjustment information persists for the current layer in output order until either of the following conditions is true:

- A new CLVS of the current layer begins.

- The bitstream ends.

- A picture picB in the current layer in an access unit containing a dynamic range adjustment SEI message with the same value of dra_range_id and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

dra_domain_idc specifies the colour space and the coding domain in which the scale and offset parameters in the SEI message are applied to. The value of dra_domain_idc shall be in the range of 0 to 2, inclusive.

dra_num_comp_minus1 specifies the number of components for which the dynamic range adjustment information will be signalled. The value of dra_num_comp_minus1 shall be in the range of 0 to 2, inclusive.

offset_present_flag equal to 1 indicates that for each scale value that is signalled there is information related to an offset value, present in the bitstream, signalled by the syntax elements offset_sign[c] and offset_magn[c]. offset_present_flag equal to 0 indicates that offset_sign[c] and offset_magn[c] are not present in the bitstream and the offset[c] variable is inferred to be 0.

scale[ c ] is used to specify the scale value of the c-th colour component. The value of scale[ c ] shall be in the range of 0 to 220 − 1, inclusive.

offset_sign[ c ] equal to 0 specifies the sign of the c-th offset value is negative. offset_sign[ c ] equal to 1 specifies that the sign of the c-th offset value is positive.

offset_magn[ c ] specifies the absolute value of the c-th offset value. The values of offset_magn[ c ] and offset_sign[ c ] are used to define the c-th offset offset[ c ].

The variable offset[ c ] is derived as follows:

if( offset_present_flag )
	offset[ c ] = ( (offset_sign[ c ]  = =  1) ? − 1: 1 ) * offset_magn[ c ]
else
	offset[ c ] = 0

The value of offset[ c ] shall be in the range of  −210 + 1 to 210 − 1, inclusive.

The scale values and the offset values of the cIdx-th colour component, Scale[ cIdx ] and Offset[ cIdx ], respectively, are derived as follows:

if( dra_num_comp_minus1  = =  0 ) {
	for( i = 0; i < 3; i++ ) {

Scale[ i ] = scale[ 0 ]
			Offset[ i ] = offset[ 0 ]
		}
	} else ( dra_num_comp_minus1  = =  1 ) {
		Scale[ 0 ] = 1
		Scale[ 1 ] = scale[ 0 ]
		Scale[ 2 ] = scale[ 1 ]
		Offset[ 0 ] = 0
		Offset[ 1 ] = offset[ 0 ]
		Offset[ 2 ] = offset[ 1 ]
	} else {

for( i = 0; i < 3; i++ ) {

Scale[ i ] = scale[ i ]
			Offset[ i ] = offset[ i ]
		}

}
The assignment of the Scale[ c ] and Offset[ c ] value to each colour component is explained under dra_num_comp_minus1. The actual value of the scale is calculated from the decoded scale[c] by dividing scale[c] with 10^5.

Values of dra_num_comp_minus1 from 0 to 2 can used to determine how the signalled scale and offset values are assigned to the components. If dra_num_comp_minus1 is equal to 0, only one pair of scale and offset is signalled and scale and offset values of all colour components are set to the signalled values. If dra_num_comp_minus1 is equal to 1, scale and offset is signalled for only chroma components and luma component has no scaling (i.e. scale is equal to 1 and offset is set to 0 for luma ). If dra_num_comp_minus1 is equal 2, each colour component has a separate scale and offset value and they are signalled separately for each.

# Usage of Color Remapping SEI

Colour_remap_id is used to specify the application space of the SEI message. pre_lut_num_val_minus1, pre_lut_coded_value and pre_lut_target_value are used to signal the pivot points in the piece-wise linear mapping. Colour_remap_video_signal_info_present_flag is set to 0 and the rest of the syntax elements are set so as to keep the signaling overhead minimal.

Advantages of using Colour Remapping SEI message:

- No need to introduce HEVC specification changes, i.e. non-normative.

- Sequence level signaling overhead is slightly higher than the proposed method.

Disadvantages of using Colour Remapping SEI message:

- Significant overhead is introduced when the metadata to be signaled changes at frame-level, i.e. signaling overhead is higher than the proposed method.

# References

- Recommendation ITU-R BT. 2020, “Parameter values for ultra-high definition television systems for production and international programme exchange” (2012).

- SMPTE ST 2084:2014, “High Dynamic Range Electro-Optical Transfer Function of Mastering Reference Displays”, 2014.

- Call for Evidence (CfE) for HDR and WCG Video Coding, MPEG document M36131, Switzerland, Feb.2015

- “Single layer non-normative (category 3a) NCL and CL responses to the Call for Evidence on HDR/WCG”, by Qualcomm, Apple, MovieLabs, NGCodec, MPEG document m36256, Poland, June 2015.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).