| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0101 |
| --- | --- |

| Title: | CE1 Test C.1: Improved palette run-length coding with palette reverse scan order |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yan Ye, Yuwen He 9710 Scranton Rd, #250 San Diego, CA 92121, USA Wei Pu, Rajan Joshi, Marta Karczewicz, Feng Zou, Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications Inc. Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This documents describes the proposed technology and reports the simulation results of CE1 Test C.1. In the proposed method, one flag is signaled at CU level for palette mode to indicate if the palette indices of one palette-coded CU are scanned in reverse scan order or in original scan order. Experimental results show that compared to SCM-4.0 anchor on 444 sequences, the proposed method reportedly provides the average {G/Y, B/Cb, R/Cr} BD-rate savings for AI, RA and LB of {0.7%, 0.9%, 1.0%}, {0.4%, 0.6%, 0.7%} and {0.5%, 0.6%, 0.6%} for the category text & graphics with motion, 1080p & 720p in both RGB and YCbCr color formats respectively. For 420 sequences, the corresponding BD-rate savings of the proposed method are {0.7%, 0.7%, 0.9%}, {0.5%, 0.5%, 0.7%} and {0.3%, 0.6%, 0.5%} for AI, RA and LB, respectively.

# Introduction

At the 18th JCT-VC meeting, CE1 [1] was established to further investigate improvements of the existing palette design. In [2], one palette reverse scan method is proposed and is being studied in CE1 Test C.1.

# Technical Description

The proposed method aims at improving the coding gain provided by the flag last_palette_run_type_flag for palette mode by signaling one additional syntax element palette_reverse_scan_flag at CU level. If the flag is set equal to 1, the palette indices of the current CU are scanned in reverse scan order; otherwise (i.e., the flag is set equal to 0), the palette indices of the current CU are scanned in original scan order. Figure 1 illustrates the reverse scan order when horizontal scan is applied to one palette-coded CU. As it can be seen in Figure 1, the proposed method improves the efficiency of the flag last_palette_run_type_flag by applying the flag to a larger run-length (i.e., 22) using palette reverse scan than the run-length (i.e., 3) using the orignal palette scan order.

(a)                                                                               (b)

Figure 1 Illustration of palette indices scan orders where numbers represent palette indices, patterned squares represent samples that the last_palette_run_type_flag is applied to. (a) Horizontal scan; (b) Horizontal reverse scan

# Simulation Results

The proposed method is implemented based on SCM-4.0 and tested under the common test condition (CTC) as specified in [3]. The coding performance of SCM-4.0 is used as anchors. The encoding and decoding time are measured by using a cluster system with homogeneous cores.

## Lossy coding performance

Table 1 and Error: Reference source not found shows BD-rate performance of the proposed method using full IBC search and local IBC search, respectively, compared to lossy SCM-4.0 anchor on 444 sequences. Table 3 shows BD-rate performance of the proposed method compared to lossy SCM-4.0 anchor on 420 sequences. It can be seen from Table 1 and Error: Reference source not found that for full IBC search, the proposed method reportedly provides average {G/Y, B/Cb, R/Cr} BD-rate savings for AI, RA and LB of {0.7%, 0.9%, 1.0%}, {0.4%, 0.6%, 0.7%} and {0.5%, 0.6%, 0.6%} for the category text & graphics with motion, 1080p & 720p in both RGB and YCbCr color formats of 444 sequences respectively; for local IBC search, the corresponding BD-rate savings are {0.9%, 1.1%, 1.2%}, {0.5%, 0.7%, 0.7%} and {0.4%, 0.5%, 0.6%} for AI, RA and LB respectively. Additionally, it is can be seen from Table 3 that for 420 sequences the proposed method reported provides BD-rate savings of {0.7%, 0.7%, 0.9%}, {0.5%, 0.5%, 0.7%} and {0.3%, 0.6%, 0.5%} for the category text & graphics with motion, 1080p & 720p for AI, RA and LB, respectively.

Table 1 BD-rate performance of the proposed method, compared to lossy SCM-4.0 anchor on 444 sequences using full IBC search

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.7% | -0.7% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.9% | -1.1% | -1.2% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.9% | -1.1% |
| YUV, Animation, 720p | 0.0% | -0.4% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | -0.5% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.7% | -0.9% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.6% | -1.0% |
| YUV, Animation, 720p | -0.1% | -0.6% | -0.4% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | -0.5% |
| RGB, mixed content, 1440p & 1080p | 0.1% | -0.2% | -0.1% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.7% | -0.7% |
| YUV, mixed content, 1440p & 1080p | 0.1% | -0.4% | -0.6% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2 BD-rate performance of the proposed method, compared to lossy SCM-4.0 anchor on 444 sequences using local IBC search

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.8% | -0.9% | -0.9% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.5% | -0.5% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.1% | -1.3% | -1.4% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -1.1% | -1.2% |
| YUV, Animation, 720p | 0.0% | -0.4% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.5% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.6% | -0.9% | -0.9% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.7% | -0.9% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.4% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.2% | -0.1% |
| RGB, Animation, 720p | 0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | -0.8% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.7% | -0.9% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 103% |  |  |

Table 3 BD-rate performance of the proposed method, compared to lossy SCM-4.0 anchor on 420 sequences

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.7% | -0.7% | -0.9% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.5% | -0.6% |
| YUV, Animation, 720p & 768p | -0.3% | -0.5% | -0.6% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.4% | -0.7% |
| YUV, Animation, 720p | -0.1% | -0.4% | -0.6% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.6% | -0.5% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.6% | -0.6% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.2% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 94% |  |  |

## Lossless coding performance

Lossless coding performance and Table 5 show bit-rate performance of the proposed method for 444 and 420 sequences, respectively, compared to lossless SCM-4.0 anchor. As shown in Lossless coding performance, the proposed method provides average bit-rate savings for AI, RA and LB of 0.2%, 0.1% and 0.1% for the category text & graphics with motion, 1080p & 720p in both RGB and YCbCr color formats of 444 sequences respectively. For 420 sequences, the corresponding bit-rate savings are 0.1%, 0.0% and 0.0%.

Table 4 Bit-rate performance of the proposed method, compared to lossless SCM-4.0 anchor on 444 sequences

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.3% | -0.6% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.3% | -0.8% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 90% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | -0.4% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | -0.5% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 93% |  |  |  |
| Dec Time[%] | 91% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.3% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | -0.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 92% |  |  |  |

Table 5 Bit-rate performance of the proposed method, compared to lossless SCM-4.0 anchor on 420 sequences

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 90% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.1% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.1% | -0.1% | 0.6% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |  |
| Dec Time[%] | 95% |  |  |  |

# References

- P. Lai, V. Seregin, X. Xiu, C. Gisquet, Description of Core Experiment 1 (CE1): Palette Mode Improvement, JCTVC-T1101, February 2015, Geneva, Switzerland.

- X. Xiu, Y. Ye, Y. He, W. Pu, R. Joshi, M. Karczewicz, F. Zou, V. Seregin, Non-CE1: Improved Palette Run-Length Coding with Palette Flipping, JCTVC-T0119, February 2015, Geneva, Switzerland.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-T1015, February 2015, Geneva, Switzerland.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).