| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0102_r1 |
| --- | --- |

| Title: | On constrained intra prediction for the unification framework of intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yan Ye Yuwen He 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# In this version of update, accurate encoding and decoding time are provided for 420 CTC sequences.

# Abstract

The unification framework of intra block copy (IBC) and inter signalling was adopted. However, it is asserted that, when constrained intra prediction (CIP) is enabled, the specification text in HEVC screen content coding draft 3 is ambiguous, and reference software SCM-4.0 is broken. In this contribution, it is proposed to disallow the samples of inter CUs, either predicted from temporal reference pictures or the current picture itself, to be used as references for intra prediction, when constrained intra prediction is enabled.

# Problem statements

At the 20th JCT-VC meeting, the signaling unification framework of intra block copy (IBC) mode and inter mode [1] was adopted into the HEVC screen content coding (SCC) draft 3 [2]. However, it was found that both the specification text in [2] and SCM-4.0 are problematic when constrained intra prediction (CIP) is enabled. Specifically, in the current specification text [2], the semantic of the flag constrained_intra_pred_flag is specified as

constrained_intra_pred_flag equal to 0 specifies that intra prediction allows usage of residual data and decoded samples of neighbouring coding blocks coded with or without using a reference picture that is not the current picture. constrained_intra_pred_flag equal to 1 specifies constrained intra prediction, in which case the general intra prediction process only uses residual data and decoded samples from neighbouring coding blocks coded without using a reference picture that is not the current picture.

It can be seen from the above that the current semantic of constrain_intra_pred_flag is not well defined, as the term “general intra prediction process” can be interpreted in two different ways. Firstly, if “general intra prediction process” refers to only intra prediction (Interpretation One), then intra prediction is allowed to use the samples of intra CUs and IBC CUs as references when the CIP is enabled. However, when IBC CUs could be predicted from inter CUs, any error in those inter CUs can first propagate to IBC CUs, and subsequently propagate to intra CUs. . In other words, this violates the CIP design intention, i.e., to suppress potential error propagation from temporal references into intra samples. Alternatively, if “general intra prediction process” refers to both intra prediction and IBC prediction (Interpretation Two), then both intra prediction and IBC prediction are only allowed to use the samples of intra CUs and IBC CUs as references (that is, no temporal reference samples can be used) when the CIP is enabled. Although this method guarantees the CIP functionality, it can significantly reduce the coding efficiency of IBC mode given that IBC CUs are prevented from being predicted form inter reference samples. It should be mentioned that this latter method, i.e., Interpretation Two) is the CIP method used for IBC during the development of HEVC range extension (RExt).

# Technical description

In this contribution, when CIP is enabled, it is proposed to only allow the samples of intra CUs to be used as references for intra prediction. In other words, all the samples of inter CUs, either “true” inter CUs that refer to temporal reference pictures or IBC inter CUs that refer to currPic, are not allowed to be used to predict intra CUs. Additionally, the proposed CIP solution is only applied to P/B-slices that use at least one temporal reference picture. For P/B-slices that have reference picture list(s) containing only currPic, the proposed CIP method is not used, and intra prediction can use reference samples from either intra predicted CUs or IBC predicted CUs.

In the proposed method, as IBC samples are disallowed to be used for intra prediction for P/B-slices with temporal reference pictures, errors due to temporal prediction cannot propagate into intra CUs. Therefore, the CIP functionality is guaranteed. Additionally, different from Interpretation Two, inter samples in already coded areas of the current picture can be used for IBC prediction in the proposed method. This is more beneficial to the coding performance of IBC CUs. It is also asserted that the proposed solution is more consistent with the unification framework given that IBC CUs and inter CUs are now treated “identically” with regards to being used as references for intra prediction (that is, both are disallowed to be used in intra prediction) when the CIP is enabled.

# Simulation results

The proposed method is implemented based on SCM-4.0 and tested following the current common test condition [3] except that the CIP functionality is enabled, i.e., setting ConstrainedIntraPred to 1. Three sets of simulations are compared using the following settings.

- Setting one: this simulation is based on Interpretation One. In addition, TMVP is disabled for IBC CUs to supress error propagation through BV prediction. In this setting, the CIP functionality is broken.

- Setting two: this simulation is based on Interpretation Two. In addition, TMVP is disabled for IBC CUs to supress error propagation through BV prediction. In this setting, the CIP functionality is guaranteed.

- Setting three: this simulation is based on the proposed method. In this setting, the CIP functionality is guaranteed.

The encoding and decoding time of three simulations are obtained from a homogeneous cluster system.

## The performance comparison of setting one v.s. setting three

and Table 5 present BD-rate performance of the proposed method (setting three) for full IBC search and local IBC search, respectively, for 444 sequences, compared to setting two.Table 6 presents the corresponding BD-rate comparison for 420 sequences. As shown in Table 4, Table 5 and Table 6, the proposed method can significantly improve coding efficiency when CIP is enabled. Specifically, for 444 sequences, the average {G/Y, B/Cb, R/Cr} BD-rate savings of the proposed method are of {2.2%, 2.3%, 2.3%} and {3.3%, 3.4%, 3.4%} for RA and LD, respectively, for full IBC search, and {1.4%, 1.5%, 1.4%} and {2.0%, 2.0%, 2.1%} for RA and LD, respectively, for local IBC search. For 420 sequences, the corresponding BD-rate savings are {3.3%, 3.4%, 3.4%} and {4.6%, 4.5%, 4.7%} for RA and LD, respectively. and Error: Reference source not found present BD-rate performance of the proposed method (setting three) for full IBC search and local IBC search, respectively, for 444 sequences, compared to setting one where CIP functionality is broken. Table 3 presents the corresponding BD-rate comparison for 420 sequences. As shown in  and Table 5 present BD-rate performance of the proposed method (setting three) for full IBC search and local IBC search, respectively, for 444 sequences, compared to setting two.Table 6 presents the corresponding BD-rate comparison for 420 sequences. As shown in Table 4, Table 5 and Table 6, the proposed method can significantly improve coding efficiency when CIP is enabled. Specifically, for 444 sequences, the average {G/Y, B/Cb, R/Cr} BD-rate savings of the proposed method are of {2.2%, 2.3%, 2.3%} and {3.3%, 3.4%, 3.4%} for RA and LD, respectively, for full IBC search, and {1.4%, 1.5%, 1.4%} and {2.0%, 2.0%, 2.1%} for RA and LD, respectively, for local IBC search. For 420 sequences, the corresponding BD-rate savings are {3.3%, 3.4%, 3.4%} and {4.6%, 4.5%, 4.7%} for RA and LD, respectively., Error: Reference source not found and Table 3, the proposed method provides the correct CIP functionality and at the same time only causes minor performance loss. Specifically, for 444 sequences, the proposed method causes average {G/Y, B/Cb, R/Cr} BD-rate increase of {0.2%, 0.2%, 0.2%} and {0.1%, 0.1%, 0.1%} for RA and LD, respectively, for full IBC search, and average {G/Y, B/Cb, R/Cr} BD-rate increase of {0.2%, 0.1%, 0.2%} and {0.1%, 0.1%, 0.1%} for RA and LD, respectively, for local IBC search. For 420 sequences, the corresponding BD-rate increases are {0.3%, 0.3%, 0.1%} and {0.2%, 0.0%, 0.1%} for RA and LD, respectively.

Table 1 BD-rate performance of setting three compared to setting one on 444 sequences using full IBC search

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | 0.1% | 0.2% |
| RGB, mixed content, 1440p & 1080p | 0.4% | 0.4% | 0.4% |
| RGB, Animation, 720p | 0.1% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.4% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.2% | 0.3% | 0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.3% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.1% | 0.3% | 0.3% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.1% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.3% | 0.4% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 91% |  |  |

Table 2 BD-rate performance of setting three compared to setting one on 444 sequences using local IBC search

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | 0.1% | 0.2% |
| RGB, mixed content, 1440p & 1080p | 0.4% | 0.4% | 0.3% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.4% | 0.2% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.2% | 0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.2% |
| RGB, Animation, 720p | 0.2% | 0.3% | 0.3% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.1% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.5% | 0.5% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 106% |  |  |

Table 3 BD-rate performance of setting three compared to setting one on 420 sequences

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 0.3% | 0.3% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 0.3% | 0.1% |
| YUV, Animation, 720p | 0.3% | 0.4% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 0.3% | 0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.1% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

## The performance comparison of setting two v.s. setting three

Table 4 and Table 5 present BD-rate performance of the proposed method (setting three) for full IBC search and local IBC search, respectively, for 444 sequences, compared to setting two.Table 6 presents the corresponding BD-rate comparison for 420 sequences. As shown in Table 4, Table 5 and Table 6, the proposed method can significantly improve coding efficiency when CIP is enabled. Specifically, for 444 sequences, the average {G/Y, B/Cb, R/Cr} BD-rate savings of the proposed method are of {2.2%, 2.3%, 2.3%} and {3.3%, 3.4%, 3.4%} for RA and LD, respectively, for full IBC search, and {1.4%, 1.5%, 1.4%} and {2.0%, 2.0%, 2.1%} for RA and LD, respectively, for local IBC search. For 420 sequences, the corresponding BD-rate savings are {3.3%, 3.4%, 3.4%} and {4.6%, 4.5%, 4.7%} for RA and LD, respectively.

Table 4 BD-rate performance of setting three compared to setting two on 444 sequences using full IBC search

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -6.4% | -6.4% | -6.3% |
| RGB, mixed content, 1440p & 1080p | -2.8% | -2.9% | -2.9% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -5.8% | -5.8% | -5.9% |
| YUV, mixed content, 1440p & 1080p | -2.7% | -2.9% | -2.9% |
| YUV, Animation, 720p | -0.1% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -8.1% | -8.4% | -8.4% |
| RGB, mixed content, 1440p & 1080p | -4.9% | -5.0% | -5.1% |
| RGB, Animation, 720p | -0.2% | -0.4% | -0.3% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -7.6% | -7.6% | -7.5% |
| YUV, mixed content, 1440p & 1080p | -5.1% | -5.3% | -5.2% |
| YUV, Animation, 720p | -0.2% | -0.6% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

Table 5 BD-rate performance of setting three compared to setting two on 444 sequences using local IBC search

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.9% | -3.9% | -3.9% |
| RGB, mixed content, 1440p & 1080p | -1.6% | -1.6% | -1.7% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -3.6% | -3.7% | -3.7% |
| YUV, mixed content, 1440p & 1080p | -1.7% | -1.8% | -1.9% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -5.0% | -5.1% | -5.1% |
| RGB, mixed content, 1440p & 1080p | -2.8% | -2.9% | -2.9% |
| RGB, Animation, 720p | -0.3% | -0.3% | -0.4% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -4.8% | -5.0% | -4.8% |
| YUV, mixed content, 1440p & 1080p | -2.8% | -3.0% | -2.9% |
| YUV, Animation, 720p | -0.1% | 0.0% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 103% |  |  |

Table 6 BD-rate performance of setting three compared to setting two on 420 sequences

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -6.8% | -6.7% | -6.8% |
| YUV, mixed content, 1440p & 1080p | -2.9% | -3.3% | -3.1% |
| YUV, Animation, 720p | -0.3% | -0.3% | -0.3% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -8.7% | -8.8% | -9.0% |
| YUV, mixed content, 1440p & 1080p | -4.7% | -4.0% | -4.5% |
| YUV, Animation, 720p | -0.4% | -0.6% | -0.5% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

# Proposed specification changes

##### 7.4.3.3.1 General picture parameter set RBSP semantics

constrained_intra_pred_flag equal to 0 specifies that intra prediction allows usage of residual data and decoded samples of neighbouring coding blocks coded using either intra or inter prediction modeswith or without using a reference picture that is not the current picture. constrained_intra_pred_flag equal to 1 specifies constrained intra prediction, in which case the general intra prediction process only uses residual data and decoded samples from neighbouring coding blocks coded without using intra prediction modea reference picture that is not the current picture.

#### 8.4.4.2 Intra sample prediction

##### 8.4.4.2.1 General intra sample prediction

……

- The availability derivation process for a block in z-scan order as specified in clause Error: Reference source not found is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) as inputs, and the output is assigned to availableN.

–	Each sample p[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, the sample p[ x ][ y ] is marked as "not available for intra prediction":

  - The variable availableN is equal to FALSE.

  - Samples of the block at the neighbouring luma location ( xNbY, yNbY ) are coded from a reference picture which is not the current pictureCuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA, DiffPicOrderCnt (aPic, CurrPic) is not equal to 0 for at least one picture aPic in RefPicList0 and RefPicList1 of the current slice, and constrained_intra_pred_flag is equal to 1.

- Otherwise, the sample p[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xNbCmp, yNbCmp ) is assigned to p[ x ][ y ].

……

##### 8.5.3.2.8 Derivation process for temporal luma motion vector prediction

……

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If slice_temporal_mvp_enabled_flag is equal to 0, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, if the reference picture is the current picture and constrained_intra_pred_flag is equal to 1, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the following ordered steps apply:

……

# References

- C. Pang, Y.-K. Wang, V. Seregin, K. Rapaka, M. Karczewicz, X. Xu, S. Liu, S. Lei, B, Li, J. Xu, Non-CE2: Intra block copy and inter signalling unification, JCTVC-T0227, February 2015, Geneva, Switzerland.

- R. Joshi, S. Liu, J. Xu, Y. Ye, HEVC Screen Content Coding Draft Text 3, JCTVC-T1005, February 2015, Geneva, Switzerland.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-T1015, February 2015, Geneva, Switzerland.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).