| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0103 |
| --- | --- |

| Title: | On chroma derivation of intra block copy for non-444 video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yan Ye Yuwen He 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

In this contribution, two methods are proposed to modify the chroma block vector derivation process in the unification framework of intra block copy in order to improve the coding performance of intra block copy for non-444 videos. In the first method, the current chroma block vector clipping process is modified. In the second method, it is proposed to enable fractional chroma interpolation for intra block copy.

Compared to SCM-4.0 anchor on 420 sequences, the first method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI, RA and LB of {0.3%, 0.6%, 0.7%}, {0.1%, 0.3%, 0.5%} and {0.0%, 0.2%, 0.6%} for the category YUV text & graphics with motion, 1080p & 720p, respectively. For lossless coding, the corresponding bit-rate savings are 0.1%, 0.1% and 0.1% for AI, RA and LB, respectively.

Compared to SCM-4.0 anchor on 420 sequences, the second method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI, RA and LB of {2.5%, 3.1%, 3.0%}, {1.5%, 1.9%, 2.0%} and {0.8%, 1.4%, 1.6%} for the category YUV text & graphics with motion, 1080p & 720p, respectively. For lossless coding, the corresponding bit-rate savings are 0.6%, 0.9% and 1.0% for AI, RA and LB, respectively.

# Introduction

At the 20th JCT-VC meeting, the unification framework of intra block copy (IBC) mode and inter mode [1] was adopted into the HEVC screen content coding  (SCC) draft 3 [2]. In the unification framework, all block vectors (BVs) are restricted to be in integer precision such that a direct sample copy from reference block (i.e., without interpolation filtering) can be used for deriving predicted samples. However, this integer-pixel BV assumption only holds for 444 input video. For non-444 video format, one integer BV may point to a fractional sample for the chroma components. For such case, in the current design, all fractional chroma BVs are clipped to integer precision. Specifically, in the HEVC SCC draft 3 [2], the clipped BV of chroma components are derived as follows:

–	If the picture with index refIdx from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-1)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-2)

–	Otherwise

mvCLX[ 0 ] = ( ( mvLX[ 0 ] >> ( 1 + SubWidthC ) ) * 8	(8-3)

mvCLX[ 1 ] = ( ( mvLX[ 1 ] >> ( 1 + SubHeightC ) ) * 8	(8-4)

Figure 1 shows one example of the BV clipping operations as specifed in [2], where shadowed circles and triangles represent the luma and chroma reference samples used to predict the current CU.

Figure 1 Example of BV clipping operation for chroma components of IBC mode.

# Proposed methods

In this contribution, two methods are proposed to improve the chroma derivation process for IBC-coded CUs for non-444 video format:

- Solution one: improved chroma BV clipping method to fetch better integer chroma reference samples for direct block copy.

- Solution two: enable chroma interpolation filtering.

## Improved BV clipping method for chroma component

As shown in Figure 1, in the existing BV clipping method, any fractional chroma BV  is clipped toward negative infinity; that is, the nearest integer chroma sample with equal or smaller horizontal and vertical coordinates (e.g., the integer chroma samples C0, C1, C2 and C3) are used in intra block copy. However, in this example, the chroma integer samples C3, C4, C5 and C6 acutally provide a set of better reference samples due to the following reasons.

- More chroma samples are from the same block as the luma samples. As shown in Figure 1, only one of the four chroma samples (C3) is from the same block as luma samples using the existing BV clipping method. In contrast, two of the proposed chroma samples (C3, C4) are from the same block as luma samples.

- The proposed set of chroma reference samples are closer than C0, C1, C2 and C3 to the current CU, which tends to be more accurate for IBC prediction.

Therefore, to improve IBC prediction efficiency, an alternative BV clipping method is proposed to clip the fractional BV toward zero, instead of toward negative infinity. The proposed BV clipping method is specified as follows:

–	If the picture with index refIdx from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-5)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-6)

–	Otherwise

mvCLX[ 0 ] =  sign( mvLX[0] ) * ( abs( mvLX[ 0 ] ) >> ( 1 + SubWidthC ) ) * 8	(8-7)

mvCLX[ 1 ] =  sign( mvLX[1] ) * ( abs( mvLX[ 1 ] ) >> ( 1 + SubHeightC ) ) * 8	(8-8)

Figure 2 shows one example of the proposed chroma BV clipping method.

Figure 2 Example of the proposed BV clipping method for chroma components of IBC mode.

## Fractional chroma sample interpolation for IBC mode

In this solution, it is proposed to unify the chroma derivation process of IBC mode with that of motion compensation process by enabling interpolation filtering to generate fractional chroma samples. Correspondingly, the existing BV clipping operation is removed as follows:

–	If the picture with index refIdx from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-9)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-10)

–	Otherwise

mvCLX[ 0 ] = ( ( mvLX[ 0 ] >> ( 1 + SubWidthC ) ) * 8	(8-11)

mvCLX[ 1 ] = ( ( mvLX[ 1 ] >> ( 1 + SubHeightC ) ) * 8	(8-12)

However, after enabling chroma interpolation filtering for IBC mode, fractional chroma samples close to the boundaries of slices/tiles or already decoded area may refer to integer reference samples that are not available because they are not in the current slice/tile or already decoded area. Therefore, additional bit-stream conformance constraints need to be applied in this solution to ensure the availability of chroma reference samples for interpolation filtering process.

8.5.3.2 Derivation process for motion vector components and reference indices

8.5.3.2.1 General

It is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- The variables xRef and yRef are derived as follows:

xRef = ( mvLX[0] % ( 1 << (1+SubWidthC) ) == 0 ) ?  ( xPb + (mvLX[ 0 ]  >>  2) ) :

( xPb + (mvLX[ 0 ]  >>  2) - 1 )   	       (8-XX)

yRef = ( mvLX[1] % (1 << (1+SubHeightC) ) == 0 ) ?  ( yPb + (mvLX[ 1 ]  >>  2) ) :

( yPb + (mvLX[ 1 ]  >>  2) - 1 )   	       (8-XX)

When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2), (yPb + mvLX[ 1 ]  >>  2) )(xRef, yRef) as inputs, the output shall be equal to TRUE.

- The variables xRef and yRef are modified as follows:

xRef = ( mvLX[0] % ( 1 << (1+SubWidthC) ) == 0 ) ?  ( xPb + (mvLX[ 0 ]  >>  2) + nPbW - 1 ) : ( xPb + (mvLX[ 0 ]  >>  2) + nPbW + 1 )   	       (8-XX)

yRef = ( mvLX[1] % (1 << (1+SubHeightC) ) == 0 ) ?  ( yPb + (mvLX[ 1 ]  >>  2) + nPbH - 1 ) : ( yPb + (mvLX[ 1 ]  >>  2) + nPbH + 1 )   	       (8-XX)

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2) + nPbW − 1, yPb + (mvLX[ 1 ]  >>  2) + nPbH – 1 )(xRef, yRef) as inputs, the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ]  >>  2 ) + nPbW + ( (mvLX[0] % (1 << (1+SubWidthC) ) == 0) ? 0 : 2 ) + xB1 is less than or equal to 0.

- The value of ( mvLX[ 1 ]  >>  2 ) + nPbH + ( (mvLX[1] % (1 << (1+SubHeightC) ) == 0) ? 0 : 2 ) + yB1 is less than or equal to 0.

- The following condition shall be true:

( xPb + ( mvLX[ 0 ]  >>  2 ) + nPbSw – 1)xRef  / CtbSizeY − xCurr / CtbSizeY  <=
		yCurr/CtbSizeY − ( yPb + ( mvLX[ 1 ]  >>  2 ) + nPbSh − 1 )yRef / CtbSizeY	(8-13)

# Simulation results

The proposed solutions are implemented based on SCM-4.0 and tested under the common test conditions as specified in [3] for 420 sequences. The coding performance of SCM-4.0 is used as the anchor.

## Coding performance of the proposed BV clipping

Table 1 and Error: Reference source not found present BD-rate and bit-rate performance of the proposed BV clipping method for lossy coding and lossless coding, respectively, compared to SCM-4.0 anchors. For lossy coding, the proposed solution provides average {Y, Cb, Cr} BD-rate savings for AI, RA and LB of {0.3%, 0.6%, 0.7%}, {0.1%, 0.3%, 0.5%} and {0.0%, 0.2%, 0.6%} for the category YUV text & graphics with motion, 1080p & 720p, respectively. For lossless coding, the corresponding bit-rate savings are 0.1%, 0.1% and 0.1% for AI, RA and LB, respectively.

Table 1 BD-rate performance of the proposed BV clipping, compared to SCM-4.0 anchor for 420 lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.6% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.4% | -0.4% |
| YUV, Animation, 720p & 768p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.3% | -0.5% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.4% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.3% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | -0.6% |
| YUV, mixed content, 1440p & 1080p | 0.1% | -0.3% | -0.1% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2 Bit-rate performance of the proposed BV clipping, compared to SCM-4.0 anchor for 420 lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.3% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

## Coding performance of fractional chroma interpolation

Table 3 and Table 4 present BD-rate performance of the fractional chroma interpolation method for lossy coding and lossless coding, respectively, compared to SCM-4.0 anchors. For lossy coding, the proposed solution provides average {Y, Cb, Cr} BD-rate savings for AI, RA and LB of {2.5%, 3.1%, 3.0%}, {1.5%, 1.9%, 2.0%} and {0.8%, 1.4%, 1.6%} for the category YUV text & graphics with motion, 1080p & 720p, respectively. For lossless coding, the corresponding bit-rate savings are 0.6%, 0.9% and 1.0% for AI, RA and LB, respectively.

Table 3 BD-rate performance of fractional chroma interpolation, compared to SCM-4.0 anchor for 420 lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -2.5% | -3.1% | -3.0% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -1.3% | -1.4% |
| YUV, Animation, 720p & 768p | -0.1% | -0.6% | -0.2% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -1.5% | -1.9% | -2.0% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.6% | -0.8% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.8% | -1.4% | -1.6% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -1.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.5% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4 Bit-rate performance of fractional chroma interpolation, compared to SCM-4.0 anchor for 420 lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -0.6% | -0.6% | -2.0% | 0.4% |
| Mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% | -0.1% |
| Animation, 720p & 768p | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 107% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -0.9% | -0.5% | -1.9% | 0.0% |
| Mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | -1.0% | -0.4% | -1.9% | 0.0% |
| Mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

# References

- C. Pang, Y.-K. Wang, V. Seregin, K. Rapaka, M. Karczewicz, X. Xu, S. Liu, S. Lei, B, Li, J. Xu, Non-CE2: Intra block copy and inter signalling unification, JCTVC-T0227, February 2015, Geneva, Switzerland.

- R. Joshi, S. Liu, J. Xu, Y. Ye, HEVC Screen Content Coding Draft Text 3, JCTVC-T1005, February 2015, Geneva, Switzerland.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-T1015, February 2015, Geneva, Switzerland.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).