| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0104 |
| --- | --- |

| Title: | On unification framework of intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yan Ye Yuwen He 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

The unification framework of intra block copy (IBC) and inter signalling was adopted. This contribution proposes several design modifications to the current IBC unification framework. Firstly, it is proposed to add the reference picture currPic that contains already coded samples of the current picture prior to in-loop filtering to both initial reference picture lists L0 and L1 when IBC is enabled. Secondly, it is proposed to signal IBC search range in SPS. Thirdly, it is proposed to disable weighted prediction signalling for the currPic.

# Proposal

At the 20th JCT-VC meeting, the signaling unification framework of intra block copy (IBC) mode and inter mode [1] was adopted into the HEVC screen content coding  (SCC) draft 3 [2]. This contribution proposes several design modifications to the current IBC unification framework.

## Reference picture list initialization when IBC is enabled

In the current IBC unification framework, IBC prediction is enabled by adding the reference picture currPic that contains already coded samples prior to in-loop filtering into the reference picture list L0 of the current picture. Then, the IBC prediction become available if currPic is included in the output reference picture list L0 after reference picture list modification (RPLM). In the current design, currPic is not allowed to appear in the initial reference picture list L1. This problem can prevent IBC from achieving maximum coding gain, as the bi-prediction between currPic and the reference pictures in L0 is not enabled.

In this contribution, it is proposed to include currPic into both initial reference picture lists L0 and L1.

8.3.4 Decoding process for reference picture lists construction

……

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPicTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )	(8-1)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]

if( curr_pic_as_ref_enabled_flag )
		RefPicListTemp1[ rIdx++ ] = currPic
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l1_active_minus1; rIdx++)			(8-2)
	    RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? efPicListTemp1[ list_entry_l1[ rIdx ] ]:   RefPicListTemp1[ rIdx ]

Additionally, to be consistent with the current common test condition (CTC), one encoder-only modification is also used in the proposed method to exclude the reference picture currPic from output reference picture list L1 through RPLM such that currPic is only included in output reference picture list L0 when IBC is enabled. The corresonding software patch is included in the package. Table 1 shows BD-rate performance of the reference picture list initialization method compared to SCM4.0 anchor using full frame IBC search. As it can be seen in Table 1, the corresponding impacts on coding performance are negligible.

Table 1 BD-rate performance of the proposed method, compared to SCM4.0 anchor using full frame IBC search

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

## Signaling IBC search range in SPS

SCM-4.0 supports both full-frame IBC search and local IBC search. In practice, the IBC search range may significantly impact the complexity of HEVC SCC codec. For example, full-frame IBC search will likely need to use off-chip memory to store un-filtered samples of the current picture which is slow in terms of data transferring speed, whereas local IBC search can be implemented using on-chip memory that is much faster than off-chip memory. Because the actual IBC search range that the encoder used may be useful information for the decoder to have, it is proposed to signal the following syntax elements in SPS SCC extension to indicate the maximum absolute values of horizontal and vertical BVs.

| sps_scc_extension( ) { | Descriptor |
| --- | --- |
| curr_pic_as_ref_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| if( palette_mode_enabled_flag ) { |  |
| palette_max_size | ue(v) |
| delta_palette_max_predictor_size | ue(v) |
| } |  |
| motion_vector_resolution_control_idc | u(2) |
| intra_boundary_filtering_disabled_flag | u(1) |
| if(curr_pic_as_ref_enabled_flag) { |  |
| max_block_vector_present_flag | u(1) |
| if( max_block_vector_present_flag ) { |  |
| log2_max_hor_block_vector_minus3 | ue(v) |
| log2_max_ver_block_vector_minus3 | ue(v) |
| } |  |
| } |  |
| } |  |

max_block_vector_present_flag equal to 1 specifies that the syntax elements log2_max_hor_block_vector_minus3 and log2_max_ver_block_vector_minus3 are present. max_block_vector_present_flag equal to 0 specifies that the syntax elements log2_max_hor_block_vector_minus3 and log2_max_ver_block_vector_minus3 are not present. When not present, it is inferred to be 0.

log2_max_hor_block_vector_minus3 plus 3 specifies the base 2 logarithm of the maximum value for horizontal motion vectors for which the reference picture is the coded picture.

log2_max_ver_block_vector_minus3 plus 3 specifies the base 2 logarithm of the maximum value for vertical motion vectors for which the reference picture is the coded picture.

The variables MaxHorizontalBV and MaxVerticalBV are derived as follows:

MaxHorizontalBV = max_block_vector_present_flag ? 1 << ( log2_max_hor_block_vector_minus3 + 3 ) : pic_width_in_luma_samples - MinCbSizeY

MaxVerticalBV = max_block_vector_present_flag ? 1 << ( log2_max_ver_block_vector_minus3 + 3 ) : pic_height_in_luma_sample - MinCbSizeY

Additionally, the following bit-stream conformance constraint is proposed to ensure that the absolute values of horizontal BV and vertical BV should be equal to or less than MaxHorizontalBV and MaxVerticalBV.

8.5.3.2 Derivation process for motion vector components and reference indices

##### 8.5.3.2.1 General

……

It is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- The absolute value of (mvLX[0] >> 2) is less than or equal to MaxHorizontalBV.

- The absolute value of (mvLX[1]>>2) is less than or equal to MaxVerticalBV.

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2), (yPb + mvLX[ 1 ]  >>  2) ) as inputs, the output shall be equal to TRUE.

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2) + nPbW − 1, yPb + (mvLX[ 1 ]  >>  2) + nPbH – 1 ) as inputs, the output shall be equal to TRUE.

……

## Disabled weighted prediction signaling for the pseudo reference picture

In the IBC unification framework, as IBC mode is signaled as normal inter mode, weighted prediction is allowed for currPic. In this contribution, it is proposed to disable WP for currPic by skipping the signaling of weighted prediction parameters, i.e., delta_luma_weight_lx and luma_offset_lx, delta_chroma_weight_lx and delta_chroma_offset_lx for currPic. Instead, they are always set equal to the inferred values (i.e., disabling weighted prediction).

It is asserted the proposed method has the following benefits:

- This allows the derivation process of IBC CUs to be truly accomplished by simply copying the samples of reference blocks.

- In HEVC SCC draft 3 [2], the total number of weighted prediction parameter pairs is restricted to be no larger than 24. Disallowing weighted prediction for currPic allows more weighted prediction parameters to be used for temporal reference pictures. In general bigger illumination changes may happen in the temporal domain rather than in the spatial domain. Therefore reserving weighted prediction parameters to be used only for temporal reference pictures may provide better coding efficiency.

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| if( PicOrderCnt(RefPicList0[i]) != PicOrderCnt(CurrPic) ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| if( PicOrderCnt(RefPicList0[i]) != PicOrderCnt(CurrPic) ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( slice_type = = B ) { |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| if( PicOrderCnt(RefPicList1[i]) != PicOrderCnt(CurrPic) ) |  |
| luma_weight_l1_flag[ I ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| if( PicOrderCnt(RefPicList1[i]) != PicOrderCnt(CurrPic) ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

luma_weight_l0_flag[ i ] equal to 1 specifies that weighting factors for the luma component of list 0 prediction using RefPicList0[ i ] are present. luma_weight_l0_flag[ i ] equal to 0 specifies that these weighting factors are not present. When not present, it is inferred to be equal to 0.

luma_weight_l1_flag[ i ] equal to 1 specifies that weighting factors for the luma component of list 1 prediction using RefPicList1[ i ] are present. luma_weight_l1_flag[ i ] equal to 0 specifies that these weighting factors are not present. When not present, it is inferred to be equal to 0.

# References

- C. Pang, Y.-K. Wang, V. Seregin, K. Rapaka, M. Karczewicz, X. Xu, S. Liu, S. Lei, B, Li, J. Xu, Non-CE2: Intra block copy and inter signalling unification, JCTVC-T0227, February 2015, Geneva, Switzerland.

- R. Joshi, S. Liu, J. Xu, Y. Ye, HEVC Screen Content Coding Draft Text 3, JCTVC-T1005, February 2015, Geneva, Switzerland.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).