| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0105 |
| --- | --- |

| Title: | Non-CE1: Simplification on deriving NumPaletteIndices for palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to modify the derivation of NumPaletteIndices, which specifies the number of palette indices signalled for the current block. Compared to SCM-4.0 anchor, the proposed method reportedly has no impact on the average coding performance. It is asserted that the proposed modification simplifies the decoding of NumPaletteIndices.

# Introduction

In the HEVC screen content coding (SCC) draft 3 [1], syntax element palette_index_idc is grouped together and put in front of other context coded syntax elements; and, syntax element num_palette_indices_idc needs to be signalled and used to derive the number of palette indices signalled in one palette-coded CU, i.e., NumPaletteIndices. To reduce the signaling overhead, the value of NumPaletteIndices is predicted from the maximal palette index in the current CU, i.e., MaxPaletteIndex. Then, the prediction difference, pRes = NumPaletteIndices – MaxPaletteIndex, is signaled in the bitstream through the syntax element num_palette_indices_idc.

In the current palette design, it is allowed to include major colors in the palette table which are not referred to by any samples in the current CU. Because unused major colors are allowed, NumPaletteIndices may be bigger or smaller than MaxPaletteIndex. In other words, the value of pRes could be either positive or negative. Because the signalled value of num_palette_indices_idc is unsigned and needs to be non-negative, the mapping between num_palette_index_idc and pRes becomes complicated in the current design, and the derivation from num_palette_index_idc to NumPaletteIndices is specified with a set of conditionals, as follows:

if( num_palette_indices_idc >= ( MaxPaletteIndex – 1) * 32 )

NumPaletteIndices = num_palette_indices_idc + 1

else if(num_palette_indices_idc % 32 = = 31)

NumPaletteIndices = MaxPaletteIndex – (num_palette_indices_idc + 1 ) / 32 			(7 79)

else

NumPaletteIndices = ( num_palette_indices_idc / 32 ) * 31 ) + (num_palette_indices_idc % 32) + 	MaxPaletteIndex

# Proposed method

In this contribution, one bit-stream conformance constraint is proposed to disallow unused major colors to be signaled in the palette table.

7.4.9.6 Palette mode semantics

The variable CurrentPaletteEntries[ cIdx ][ i ] specifies the i-th element in the current palette for the colour component cIdx and is derived as follows:

numPredictedPaletteEntries = 0
for( i = 0; i < PredictorPaletteSize; i++ ) 
	if( PalettePredictorEntryReuseFlag[ i ] = = 1 ) {
		for( cIdx = 0; cIdx < numComps; cIdx++ ) 
			CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries ] = PredictorPaletteEntries[ cIdx ][ i ]
		numPredictedPaletteEntries++ 
	}
for( cIdx = 0; cIdx < numComps; cIdx++ ) 		(7-1)
	for( i = 0; i < num_signalled_palette_entries; i++ ) 
		CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries + i ] = palette_entry

It is a requirement of bitstream conformance that CurrentPaletteEntries shall not contain any element that is not referred by the samples in the current CU.

With the proposed constraint, NumPaletteIndices is always larger than or equal to MaxPaletteIndex. This removes the need for the complicated mapping process between num_palette_indices_idc and NumPaletteIndices. Specifically, based on the proposed method, the derivation of NumPaletteIndices  can be simplified as follows.

num_palette_indices_idc is an indication of the number of palette indices signalled for the current block.

When num_palette_indices_idc is not present, it is inferred to be equal to 0.

The variable NumPaletteIndices specifies the number of palette indices signalled for the current block and is derived as follows:MaxPaletteIndex + num_palette_indices_idc.

if( num_palette_indices_idc >= ( MaxPaletteIndex – 1) * 32 )
	NumPaletteIndices = num_palette_indices_idc + 1
else if(num_palette_indices_idc % 32 = = 31)
	NumPaletteIndices = MaxPaletteIndex – (num_palette_indices_idc + 1 ) / 32 	(7-2)
else
	NumPaletteIndices = ( num_palette_indices_idc / 32 ) * 31 ) + (num_palette_indices_idc % 32) + 	MaxPaletteIndex

# Simulation results

The proposed method is implemented based on SCM-4.0 and tested under the common test condition (CTC) as specified in [2]. The coding performance of SCM-4.0 is used as anchors. The encoding and decoding time are measured by using a cluster system with heterogeneous cores. But, as the proposed method conceptually only simplifies the derivation of NumPaletteIndices, the impact on encoding and decoding time is expected to be negligible.

Table 1 and Error: Reference source not found shows BD-rate performance of the proposed method using full IBC search and local IBC search, respectively, compared to lossy SCM-4.0 anchor on 444 sequences. Table 3 shows BD-rate performance of the proposed method compared to lossy SCM-4.0 anchor on 420 sequences. Table 4 and Table 5 show bit-rate performance of the proposed method for 444 and 420 sequences, respectively, compared to lossless SCM-4.0 anchor. As it can be seen from the above tables, the proposed simplification does not incur performance loss to any of the coding configurations, and in most cases gives a small gain.

Table 1 BD-rate performance of the proposed method, compared to lossy SCM-4.0 anchor on 444 sequences using full IBC search

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.2% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.2% | 0.0% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.3% |
| YUV, mixed content, 1440p & 1080p | -0.1% | 0.4% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 105% |  |  |

Table 2 BD-rate performance of the proposed method, compared to lossy SCM-4.0 anchor on 444 sequences using local IBC search

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 107% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.2% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.3% | 0.0% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 106% |  |  |

Table 3 BD-rate performance of the proposed method, compared to lossy SCM-4.0 anchor on 420 sequences

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p & 768p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 92% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 92% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.2% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 93% |  |  |

Table 4 Bit-rate performance of the proposed method, compared to lossless SCM-4.0 anchor on 444 sequences

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 96% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |  |
| Dec Time[%] | 93% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  |  |
| Dec Time[%] | 93% |  |  |  |

Table 5 Bit-rate performance of the proposed method, compared to lossless SCM-4.0 anchor on 420 sequences

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 93% |  |  |  |
| Dec Time[%] | 91% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 93% |  |  |  |
| Dec Time[%] | 95% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  |  |
| Dec Time[%] | 94% |  |  |  |

# References

- R. Joshi, S. Liu, J. Xu, Y. Ye, HEVC Screen Content Coding Draft Text 3, JCTVC-T1005, February 2015, Geneva, Switzerland.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-T1015, February 2015, Geneva, Switzerland.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).