| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0106 |
| --- | --- |

| Title: | On signaling adaptive color transform at TU level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

In HEVC screen content coding draft 3, adaptive color transform is enabled/disabled at CU level such that all the residuals of one CU are coded in the same color space. This contribution proposes to improve the performance of adaptive color transform by moving the adaptive color transform flag to TU level.

Compared to SCM-4.0 anchor using full-frame IBC search, the proposed solution provides average {G, B, R} BD-rate reductions of {0.2%, 0.5%, 0.4%}, {0.1%, 0.6%, 0.5%} and {0.3%, 0.4%, 0.2%} for AI, RA and LD respectively in RGB coding, and average {Y, Cb, Cr} BD-rate reductions of {0.6%, 2.0%, 1.9%}, {0.3%, 1.4%, 1.4%} and {0.1%, 0.3%, 0.3%} for AI, RA and LD respectively in YCbCr coding. For lossless coding, the corresponding bit-rate reductions are 0.4%, 1.1% and 1.0% for AI, RA and LD in RGB coding, and 0.0%, 0.0% and 0.0% for AI, RA and LD in YCbCr coding.

# Proposal

In HEVC screen content coding (SCC) draft [1], two coding tools, namely cross-component prediction (CCP) and adaptive color transform (ACT), are employed to exploit the correlation among three color components in the residual domain. Although the decoding processes of both inverse CCP and inverse ACT are specified as TU level operations [1], the two coding tools are enabled/disabled at different levels. Specifically, the CCP tool is enabled/disabled at TU level by sending different scaling parameters (i.e., log2_res_scale_abs_plus1 and res_scale_sign_flag) for each TU. In contrast, the ACT tool is enabled/disabled at CU level by the flag cu_residual_act_flag.

In this contribution, it is proposed to move the ACT flag from CU level to TU level. The asserted advantages of the proposed solution include: 1) TU level ACT signaling can provide finer granularity of switching between different color spaces and therefore improve the coding gain that the ACT tool can achieve; 2) the proposed solution unifies CCP and ACT by enabling/disabling the two coding tools at the same level.

# Encoder speedup

In this contribution, the non-normative encoding methods and speedup logics for CU-level ACT RD test [2] in SCM-4.0 remain mostly unchanged. The proposed encoder conditionally replaces the second CU-level ACT test in SCM-4.0 (i.e., original color space for RGB sequences and transformed color space for YCbCr sequences) with TU-level ACT test to achieve good performance vs complexity tradeoff, according to the following:

- TU level ACT is only tested for small CUs (8x8 and/or 16x16 depending on intra/inter/IBC coding mode).

- For intra CUs, when TU level ACT test is applied, whether to test the second color space of the current TU depends on the RD costs of its parent TU. If the RD cost of the first color space is smaller than that of the second color space for the parent TU, then the second color space is not tested for the current TU.

- TU level ACT is not tested for pictures at some temporal layers (TL) for RA and LD configurations. When TU level ACT is not tested, default CU level ACT test in SCM-4.0 is applied.

It should be mentioned that, regardless of whether CU-level or TU-level ACT test is performed, the selected color space is always signaled at the TU level.

# Simulation results

The proposed methods are implemented based on SCM-4.0 and tested under the common test conditions as specified in [3]. The coding performance of SCM-4.0 is used as the anchor. The encoding and decoding time are measured by using a cluster system with homogeneous cores.

Table 1 and Error: Reference source not found present BD-rate performance of TU level ACT signaling for lossy coding and lossless coding, respectively, compared to SCM-4.0 anchors using full frame IBC search. For lossy coding, the proposed solution provides average {G, B, R} BD-rate reductions of {0.2%, 0.5%, 0.4%}, {0.1%, 0.6%, 0.5%} and {0.3%, 0.4%, 0.2%} for AI, RA and LD respectively in RGB coding, and average {Y, Cb, Cr} BD-rate reductions of {0.6%, 2.0%, 1.9%}, {0.3%, 1.4%, 1.4%} and {0.1%, 0.3%, 0.3%} for AI, RA and LD respectively in YCbCr coding. For lossless coding, the corresponding bit-rate reductions are 0.4%, 1.1% and 1.0% for AI, RA and LD in RGB coding, and 0.0%, 0.0% and 0.0% for AI, RA and LD in YCbCr coding.

Table 3 present BD-rate performance of TU level ACT signaling, compared to lossy SCM-4.0 anchor using local IBC search. As shown in Table 3, the proposed solution provides average {G, B, R} BD-rate reductions of {0.1%, 0.4%, 0.3%}, {0.2%, 0.6%, 0.4%} and {0.2%, 0.4%, 0.2%} for AI, RA and LD respectively in RGB coding, and average {Y, Cb, Cr} BD-rate reductions of {0.6%, 2.0%, 2.0%}, {0.3%, 1.3%, 1.3%} and {0.0%, 0.4%, 0.4%} for AI, RA and LD respectively in YCbCr coding.

Table 1 BD-rate performance of TU level ACT signaling, compared to lossy SCM-4.0 anchor using full IBC search

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.9% | -0.8% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.5% | -0.5% |
| RGB, Animation, 720p | -0.1% | -0.4% | -0.3% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.9% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -1.1% | -1.1% |
| YUV, Animation, 720p | -1.3% | -3.8% | -3.5% |
| YUV, camera captured, 1080p | -0.3% | -1.2% | -1.5% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -1.0% | -0.8% |
| RGB, mixed content, 1440p & 1080p | 0.1% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.2% | -0.8% | -0.5% |
| RGB, camera captured, 1080p | 0.1% | -0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.7% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -1.0% | -1.3% |
| YUV, Animation, 720p | -0.6% | -2.9% | -2.4% |
| YUV, camera captured, 1080p | -0.1% | -1.0% | -1.2% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.4% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.1% |
| RGB, Animation, 720p | -0.4% | -0.7% | -0.3% |
| RGB, camera captured, 1080p | 0.0% | -0.2% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | 0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.9% |
| YUV, Animation, 720p | 0.0% | -0.7% | -0.3% |
| YUV, camera captured, 1080p | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 97% |  |  |

Table 2 Bit-rate performance of TU level ACT signaling, compared to lossless SCM-4.0 anchor using full IBC search

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.8% | -0.9% | -2.2% | -0.3% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.3% | -0.5% | -0.3% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 94% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -1.5% | -1.4% | -3.6% | -0.5% |
| RGB, mixed content, 1440p & 1080p | -1.2% | -1.2% | -1.5% | -0.8% |
| RGB, Animation, 720p | -1.5% | -1.5% | -1.5% | -1.5% |
| RGB, camera captured, 1080p | -0.3% | -0.3% | -0.3% | -0.2% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 94% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -1.2% | -1.1% | -2.7% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -1.0% | -1.0% | -1.5% | -0.5% |
| RGB, Animation, 720p | -1.3% | -1.3% | -1.3% | -1.3% |
| RGB, camera captured, 1080p | -0.3% | -0.2% | -0.3% | -0.2% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | -0.2% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 92% |  |  |  |

Table 3 BD-rate performance of TU level ACT signaling, compared to lossy SCM-4.0 anchor using local IBC search

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.6% | -0.5% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.4% | -0.4% |
| RGB, Animation, 720p | -0.1% | -0.4% | -0.3% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.8% | -0.8% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -1.0% | -1.1% |
| YUV, Animation, 720p | -1.3% | -3.9% | -3.5% |
| YUV, camera captured, 1080p | -0.3% | -1.2% | -1.5% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.8% | -0.6% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.6% | -0.7% |
| RGB, Animation, 720p | -0.2% | -0.9% | -0.5% |
| RGB, camera captured, 1080p | 0.0% | -0.2% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.7% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.9% | -1.1% |
| YUV, Animation, 720p | -0.4% | -2.7% | -2.4% |
| YUV, camera captured, 1080p | -0.2% | -1.0% | -0.9% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.3% | -0.1% |
| RGB, Animation, 720p | -0.3% | -0.7% | -0.4% |
| RGB, camera captured, 1080p | 0.0% | -0.2% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.3% |
| YUV, mixed content, 1440p & 1080p | 0.1% | -0.7% | -0.6% |
| YUV, Animation, 720p | 0.0% | -0.8% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# References

- R. Joshi, S. Liu, J. Xu, Y. Ye, HEVC Screen Content Coding Draft Text 3, JCTVC-T1005, February 2015, Geneva, Switzerland.

- L. Zhang, J. Chen, J. Sole, M. Karczewicz, X. Xiu, Y. He, Y. Ye, SCCE5 Test 3.2.1: In-loop Color Space Transform, JCTVC-R0147, June 2014, Sapporo, Japan.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-T1015, February 2015, Geneva, Switzerland.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).