| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0109r1 |
| --- | --- |

| Title: | Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Chun-Lung Lin, 3Ching-Chieh Lin, Jih-Sheng Tu and William Digonnet 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

In the current palette design, the pixel level palette run value is derived from the combination of the index of the most significant bit (MSB) and the refinement bits, i.e., the syntax elements palette_run_msb_id_plus1 and palette_run_refinement_bits. The first five bins of palette_run_msb_id_plus1 are coded by context mode in the current design, and the other bins are coded by bypass mode. Because larger CUs usually have more palette runs , a 32x32 CU coded by palette mode will result in slower CABAC throughput on the parsing of the whole palette run values than the other CU sizes due to dependency of context-coded bins. Besides, it is observed that the fourth and fifth bin has approximately equal occurring probability on binary numbers (which is either 0 or 1). Therefore, this contribution proposes to simplify the derivation process of palette_run_msb_id_plus1 by using the bypass mode to code the fourth bin and the fifth bin. It is reported the BD-rate increases are 0-0.3% with average smaller than 0.1%.

# Introduction

In SCM 4.0, the palette mode signalls the palette run type flag, run value and escape values for index map coding [1]. There are two run types in the current design: COPY_INDEX_MODE and COPY_ABOVE_MODE. The palette run value is derived by palette_run_msb_id_plus1 and palette_run_refinement_bits. In the current design, the first five bins of the syntax element palette_run_msb_id_plus1 are coded by context bins which may influence the CABAC throughput. The derivation process of the variable ctxInc for palette_run_msb_id_plus1 is as follows:

| binIdx | 0 | 1 | 2 | 3 | 4 | > 4 |
| --- | --- | --- | --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 5 | 6 | 6 | 7 | 7 | bypass |
| palette_run_type_flag = COPY_INDEX_MODE | 0,1,2 | 3 | 3 | 4 | 4 | bypass |

# Proposed method

In order to improve the CABAC throughput, this contribution proposes to replace the context mode for the fourth and fifth bins of palette_run_msb_id_plus1 by the bypass mode. The first three bins are still coded by context mode without any change. Thus the proposed derivation process of the variable ctxInc for palette_run_msb_id_plus1 is as follows:

| binIdx | 0 | 1 | 2 | >2 |
| --- | --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 5 | 6 | 7 | bypass |
| palette_run_type_flag = COPY_INDEX_MODE | 0,1,2 | 3 | 4 | bypass |

# Experimental results

The proposed method is integrated into the SCM 4.0 reference software and tested under the common test condition [2][3]. Table 1 and Table 2 show the lossy and lossless results for the full frame IBC search area. The details for the other experiment results with different test conditions are accompanied with this document. It is observed that the average BD-rate increase is smaller than 0.1%.

Table 1: lossy results for full frame IBC search area

Table 2: lossless results for full frame IBC search area

# Conclusion

This contribution proposes to use bypass mode to code the fourth and fifth bins of the syntax element palette_run_msb_id_plus1. This modification can simplify the derivation process for palette_run_msb_id_plus1 and improve the CABAC throughput. According to the experimental results, there is no significant gain loss for the lossy test condition under the full frame IBC search area. It is recommended to adopt this contribution.

# References

- R. Joshi, J. Xu, S. Liu, Y. Ye, "High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 3", 20th JCT-VC meeting, JCTVC-T1005, Geneva, Switzerland, Feb. 2015

- SCM-4.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” 20th JCT-VC meeting, JCTVC-T1015, Geneva, Switzerland, Feb. 2015

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).