8.5.3.2	Derivation process for motion vector components and reference indices

8.5.3.2.1	General

…….

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1, as well as predFlagL0 and predFlagL1, the following applies:

- If merge_flag[ xPb ][ yPb ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in clause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nCbS, nPbW, nPbH and the partition index partIdx as inputs, and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0 and predFlagL1.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX and refIdxLX, in PRED_LX, and in the syntax elements ref_idx_lX and MvdLX, the following applies:

- The variables refIdxLX and predFlagLX are derived as follows:

- If inter_pred_idc[ xPb ][ yPb ] is equal to PRED_LX or PRED_BI,

refIdxLX = ref_idx_lX[ xPb ][ yPb ]	(8-1)

predFlagLX = 1		(8-2)

- Otherwise, the variables refIdxLX and predFlagLX are specified by:

refIdxLX = −1		(8-3)

predFlagLX = 0		(8-4)

- The variable mvdLX is derived as follows:

mvdLX[ 0 ] = MvdLX[ xPb ][ yPb ][ 0 ]	(8-5)

mvdLX[ 1 ] = MvdLX[ xPb ][ yPb ][ 1 ]	(8-6)

- When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in clause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the variables nPbW, nPbH, refIdxLX and the partition index partIdx as inputs, and the output being mvpLX.

- When predFlagLX is equal to 1 and the picture with index refIdx from reference picture list LX of the slice is not the current picture, the luma motion vector mvLX is derived as follows:

uLX[ 0 ] = ( mvpLX[ 0 ] + mvdLX[ 0 ] + 216 ) % 216	(8-7)

mvLX[ 0 ] = ( uLX[ 0 ]  >=  215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ]	(8-8)

uLX[ 1 ] = ( mvpLX[ 1 ] + mvdLX[ 1 ] + 216 ) % 216	(8-9)

mvLX[ 1 ] = ( uLX[ 1 ]  >=  215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(8-10)

NOTE – The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1, inclusive.

- When predFlagLX is equal to 1 and the reference picture is the current picture, the luma motion vector mvLX is derived as follows:

uLX[ 0 ] = ( ( ( mvpLX[ 0 ] >> 2 ) + mvdLX[ 0 ] )  <<  2 + 216 ) % 216	(8-11)

mvLX[ 0 ] = ( uLX[ 0 ]  >=  215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ] 	(8-12)

uLX[ 1 ] = ( ( mvpLX[ 0 ] >> 2 ) + mvdLX[ 0 ] )  <<  2 + 216 ) % 216	(8-13)

mvLX[ 1 ] = ( uLX[ 1 ]  >=  215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(8-14)

NOTE 2 – The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1, inclusive.

It is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2), (yPb + mvLX[ 1 ]  >>  2) ) as inputs, the output shall be equal to TRUE.

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2) + nPbW − 1, yPb + (mvLX[ 1 ]  >>  2) + nPbH – 1 ) as inputs, the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ]  >>  2 ) + nPbW + xB1 is less than or equal to 0.

- The value of ( mvLX[ 1 ]  >>  2 ) + nPbH + yB1 is less than or equal to 0.

- The following condition shall be true:

( xPb + ( mvLX[ 0 ]  >>  2 ) + nPbSw − 1 ) / CtbSizeY − xCurr / CtbSizeY  <=
		yCurr/CtbSizeY − ( yPb + ( mvLX[ 1 ]  >>  2 ) + nPbSh − 1 ) / CtbSizeY	(8-15)

When ChromaArrayType is not equal to 0 and predFlagLX, with X being 0 or 1, is equal to 1, the derivation process for chroma motion vectors in clause Error: Reference source not found is invoked with mvLX as input, and the output being mvCLX.

……

8.5.3.3	Decoding process for inter prediction samples

8.5.3.3.1	General

……

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCb of chroma prediction samples for the component Cb, where nCbSwC and nCbShC are derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCr of chroma residual samples for the component Cr, where nCbSwC and nCbShC are derived as specified below.

When use_integer_mv_flag is equal to 1 andor the reference index refIdxLX is not equal to currPic, mvLX and mvCLX (where X equals to 0 or 1) are modified as follows:

mvLX = Clip3( − 215, 215 − 1, mvLX << 2 )	(8-16)

mvCLX = Clip3( − 215, 215 − 1, mvCLX << 2 )	(8-17)

The variable nCbSL is set equal to nCbS. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to nCbS / SubWidthC and the variable nCbShC is set equal to nCbS / SubHeightC.

……