| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0111 |
| --- | --- |

| Title: | Non-CE: MV resolution unification for MV derivation in Intra-Block Copy search |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Jih-Sheng Tu, 2Ching-Chieh Lin, 3Chun-Lung Lin and Yao-Jen Chang 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-5639 1sunrisejstu@itri.org.tw 2JackLin@itri.org.tw 3Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

In the current unification scheme of Intra-Block Copy (IBC) and Inter prediction, additional operations for motion vector (MV) derivation process are required due to the inconsistency of MV resolution of IBC. This contribution proposes a MV resolution unification method, in which, the MV resolution of IBC would be set in integer-pixel precision without considering “use_integer_mv_flag”. The proposed scheme removes the redundant operations and further unifies IBC and Inter mode. The experimental results show that the BD-rate changes of the proposed method is -0.1% to 0.1% under lossy condition for 444-format.

# Introduction

In the 20th JCT-VC meeting, it is decided to harmonize the Intra-Block Copy (IBC) mode and Inter prediction mode [1]. The IBC mode is unified with Inter prediction mode using HEVC AMVP and motion vector difference (MVD). And the motion vector (MV) in IBC mode is forced to be quarter-pixel precision, but, the MVD in IBC is stored in integer-pixel precision. Hence, when deriving a MV in IBC mode, the motion vector predictor (MVP) needs to change to integer-pixel precision. After adding the MVD and MVP, the results shifts back to quarter-pixel precision.

# Proposed method

A simplified IBC-Inter mode unification flow is proposed. Comparing to the current design, for an IBC block in the proposed method, the MV is not in quarter-pixel precision but in integer-pixel precision. Therefore, the shift operations can be removed when adding MVP and MVD.

The flowchart of the proposed method is illustrated in Figure 1 and the flowchart of the current design is illustrated in Figure 2. It shows that the proposed method reduces one shift operation and one comparison operation for MV derivation, and simplifies the standard document.

Figure 1. The proposed MV derivation process for MC.

Figure 2. The current MV derivation process for MC.

# Experimental results

The proposed scheme is implemented on SCM-4.0. The modification gets -0.1% to 0.1% gain under 444 colour format and full frame IBC search area. The experimental conditions are subject to the test conditions issued by JCT-VC in the 20th meeting [2]. Table 1 to Table 3 demonstrate the coding performance under full frame IBC against SCM-4.0 anchor. The details for the simulation results could be found in the accompanied spread sheets.

Table 1. Proposed vs. Anchor  (AI-lossy)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | NA |  |  |
| Dec Time[%] | NA |  |  |

Table 2. Proposed vs. Anchor  (RA-lossy)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | NA |  |  |
| Dec Time[%] | NA |  |  |

Table 3. Proposed vs. Anchor  (LB-lossy)

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | NA |  |  |
| Dec Time[%] | NA |  |  |

# Conclusions

This contribution proposes a simplified MV derivation process for IBC and Inter prediction mode unification. The simulation results show negligible coding gain change. Therefore, it is recommended to adopt the proposed modification.

# References

- R. Joshi, S. Liu, J. Xu, Y. Ye, “HEVC Screen Content Coding Draft Text 3,” 20th JCT-VC meeting, JCTVC-T1005, Geneva, CH, Feb. 2014

- H. Yu, R. Cohen, K. Rapaka, J. Xu, " Common Test Conditions for Screen Content Coding," Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-T1015, 20th Meeting: Geneva, CH, 10 Feb – 17 Feb 2015

# Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).