| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0112 |
| --- | --- |

| Title: | Ambient viewing environment SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution proposes an SEI message to describe the ambient viewing environment assumed when mastering the associated video content. The contribution asserts that the ambient viewing environment in which video content is experienced by the viewer can have a substantial effect on perceptual quality and this is widely recognized (e.g., in ITU-R BT.2035 and ITU-R BT.500). For example, viewing an image in the varying environments of a darkened theater, a typical home (at night or during the daytime), or outdoors can have a major effect on the viewing experience. It is reported that adaptive brightness and light sensor technology are becoming common in devices such as mobile phones, tablets, and laptop computers. It is asserted that conveying a representation of the conditions of the nominal viewing environment that was assumed when mastering the video content can enable receiving systems to adapt their local display (e.g., based on a user pre-setting or light sensor input) for optimal display. The proposed message is capable of conveying either of the types of viewing conditions specified in ITU-R BT.2035, and other conditions that can be specified using similar parameters.

# Introduction

## General background

The ambient viewing environment in which video content is experienced by the viewer can have a substantial effect on perceptual quality and this is widely recognized (e.g., in ITU-R BT.2035 and ITU-R BT.500). For example, viewing an image in the varying environments of a darkened theater, a typical home (at night or during the daytime), or outdoors can have a major effect on the viewing experience. Adaptive display brightness and light sensor technology are becoming common in devices such as mobile phones, tablets, and laptop computers.

## Existing specifications of viewing environments

Rec. ITU-R BT.2035 (07/2013), A reference viewing environment for evaluation of HDTV program material or completed programmes, (cf. subclause 1.1) a nominal viewing environment with 10 lux of ambient light luminance having a background chromaticity of D65 (x = 0.3127, y = 0.3290) or D93 (x = 0.2831, y = 0.2971), for use with a display having peak white (value 940 in 10 bit representation) luminance of 100 cd/m2 (100 nits).

Rec. ITU-R BT.500-13 (01/2012), Methodology for the subjective assessment of the quality of television pictures, notes that viewing environments for laboratory use and home use can differ. It identifies a nominal laboratory environment for testing (subclause 2.1.1) with a low room illumination with D65 ambient light chromaticity, a peak white display luminance of 70 cd/m2 (set by PLUGE with reference to Rec. ITU-R BT.814-2 (09/2007), Specifications and alignment procedures for setting of brightness and contrast of displays) and a background behind the picture with a brightness 0.15 times the peak white luminance (i.e., about 10.5 cd/m2), and a nominal home viewing environment (subclause 2.1.2) with 200 lux ambient light level and a display peak white luminance of 200 cd/m2.

Rec. ITU-R BT.2022 (08/2012), General viewing conditions for subjective, assessment of quality of SDTV and HDTV, television pictures on flat panel displays, is another example similar to what is found in Rec. ITU-R BT.500-13 (01/2012). It is roughly similar to what is stated in Rec. ITU-R BT.500, except in regard to peak white brightness. For laboratory environments, it identifies a peak white range of 70–250 cd/m2. For home environments, it identifies a peak white range of 70–500 cd/m2.

Rec. ITU-R BT.710-4 (11/1998), Subjective assessment methods for image quality in high-definition television, recommends that subjective assessments be conducted in a low light environment similar to that described in BT.500 and BT.2022. It identifies a peak white range of 150–250 cd/m2, and is otherwise similar to BT.500 and BT.2022.

Rec. ITU-R BT.1129 (02/1998), Subjective assessment of standard definition digital television (SDTV) systems, identifies both laboratory environment and home environment conditions. These conditions are similar to what is found in BT.500, except it does not seem to identify a colour for the ambient light.

## The problem and concept of the proposal

These specifications thus illustrate that viewing environments differ in practice, even for semi-ideal viewing conditions. Obviously, the viewing conditions that are experienced in practice can differ substantially from those identified in these specifications. Moreover, even when some specification of a reference viewing environment may exist for an application, it is common for the actual viewing environment that is used when mastering image content to differ from the specified reference viewing environment.

The HEVC mastering display colour volume SEI message (and SMPTE ST 2086, on which it was based) specifies a way to convey the properties of a mastering display, which includes specification of display the x and y coordinates of display primaries, the white point, and the nominal minimum and maximum display luminance. However, we have not specified any way to explicitly represent the ambient light characteristics of the viewing environment that was assumed or considered ideal for mastering purposes. We assert that conveying data that represents such viewing environment characteristics would enable receiving systems to adapt their local display (e.g., based on a user pre-setting or light sensor input) for optimal display.

# Proposal

We propose an SEI message that can convey a representation of the conditions of the nominal viewing environment that was assumed when mastering the video content. This can enable receiving systems to adapt their local display (e.g., based on a user pre-setting or light sensor input) for optimal display. The proposed message is capable of conveying either of the types of viewing conditions specified in ITU-R BT.2035, and other conditions that can be specified using similar parameters. The precise syntax and semantics proposed for this SEI message are provided in the following subsections.

## Ambient viewing environment SEI message syntax

| ambient_viewing_environment( payloadSize ) { | Descriptor |
| --- | --- |
| ambient_illuminance | u(32) |
| ambient_light_x | u(16) |
| ambient_light_y | u(16) |
| } |  |

## Ambient viewing environment SEI message semantics

This SEI message identifies the characteristics of the nominal ambient viewing environment for the display of the associated video content. The syntax elements of the ambient viewing environment SEI message may assist the receiving system in adapting the received video content for local display in viewing environments that may be similar or may substantially differ from those assumed or intended when mastering the video content.

This SEI message does not provide information on colour transformations that would be appropriate to preserve creative intent on displays with colour volumes different from that of the described mastering display.

When a ambient viewing environment SEI message is present for any picture of a CLVS of a particular layer and the first picture of the CLVS is an IRAP picture, a ambient viewing environment SEI message shall be present for that IRAP picture. The ambient viewing environment SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. All ambient viewing environment SEI messages that apply to the same CLVS shall have the same content.

ambient_illuminance specifies the environmental illluminance of the ambient viewing environment in units of 0.0001 lux. ambient_illuminance shall not be equal to 0.

ambient_light_x and ambient_light_y specify the normalized x and y chromaticity coordinates, respectively, of the environmental ambient light in the nominal viewing environment in normalized increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). The values of ambient_light_x and ambient_light_y shall be in the range of 0 to 50 000.

NOTE – For example, the conditions identified in Rec. ITU-R BT.2035 can be expressed using ambient_illuminance equal to 100 000 with background chromaticity indicating D65 (ambient_light_x equal to 15 635, ambient_light_y equal to 16 450), or optionally in some regions, background chromaticity indicating D93 (ambient_light_x equal to 14 155, ambient_light_y equal to 14 855).

## Other text impact

The new SEI message should also be integrated into D.2.1 (General SEI message syntax) and Table D.1 (Persistence scope of SEI messages), in a similar manner as with, for example, the mastering display colour volume SEI message.

For Table D.1, the persistence scope should be checked in regard to the scope being CVS versus CLVS. There appears to be an error in this aspect of the prior specification of the mastering display colour volume SEI message. We believe the proper scope for both is the CLVS.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).