| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0115 |
| --- | --- |

| Title: | Video Bitstream Transition SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade K. Wan | Email: | wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution proposes that SEI messages be defined to facilitate the signaling of an upcoming transition from a bitstream encoded with one compression standard to another bitstream encoded with a different compression standard.  It is suggested that this type of signaling is needed in the video layer to facilitate effective support of on-the-fly codec switching.  It is proposed that this functionality be defined to support switching between the ITU-T Recommendation H.262 | ISO/IEC 13818-2, ITU-T Recommendation H.264 | ISO/IEC 14496-10 and ITU-T Recommendation H.265 | ISO/IEC 23008-2 compression standards.

# Background

Traditionally, video has been delivered in a careful manner to avoid codec switches and the possibility that glitches will occur when such transitions occur.  However, there has been renewed interest in supporting switching between codecs to enable the following advantages:

- Reduction of multiple versions of commercials to match programming

  - Currently, operators create and store multiple versions of commercials to be able to match the commercial to the codec being used for a particular channel and avoid issues with switching between codecs.

  - The ability to gracefully handle codec transitions could allow operators to use only one version of a commercial.

  - That is, a MPEG-2 commercial on an existing channel (that has been paired with MPEG-2 programming) can be reused instead of creating an AVC version to pair with AVC programming on another channel.

- Extending adaptive streaming manifests

  - Many current DASH manifests have AVC video up to HD resolutions.

  - An operator may wish to add a higher resolution stream like 4K resolution but would like to use HEVC for 4K resolutions, as using AVC for 4K video may result in bitrates that are too high to be practical.

  - The addition of a HEVC 4K option may be problematic if the client which has started decoding the HEVC 4K program has to adapt to lower bitrates/resolutions and one would like to avoid adding additional lower resolutions in the HEVC format for this situation.

  - The ability to gracefully handle codec transitions could allow operators to avoid adding lower resolutions in the HEVC format and reutilize the existing lower resolution AVC formats if there is a need to adapt to lower bitrates/resolutions.

A significant issue with supporting codec transitions is the difficulty in signaling exactly when and where within the video data the transition from one codec to another occurs.  At the systems layer, there is typically signaling to indicate the video codec (e.g. stream_type in the PMT (Program Map Table) of MPEG-2 transport streams).  This works fine for one-time initialization and is currently the method used to initialize a video decoder and the mechanism to specify the codec (when tuning to another channel).

However, this signaling is problematic for on-the-fly switching purposes.  Systems layer processing is often performed in a different portion of a receiver and thus it may be difficult to synchronize detection of the timing and location of the transition at the systems/transport layer with the exact timing and location necessary for a video layer decoder to implement a switch at the transition point.

A solution would ideally provide both advance warning as well as the exact location of the transition.  This contribution suggests that this problem can be more effectively solved by defining signaling in the video layer as follows:

- A SEI (Supplemental Enhancement Information) message or UD (User Data) can be attached to one or more pictures (in decode order) to provide the decoder the exact number of pictures before the transition so it can be prepared for the transition.

  - Additional information to assist the transition besides the codec can be signalled such as the profile, level and tier of the subsequent bitstream.

  - Note pictures (and not frames) should be specified to avoid any ambiguity when handling interlaced pictures

- An indicator appropriate to the compression standard will be used to signal the end of the last picture before the transition

  - This allows one to define the exact location of the transition and is the proper standard practice for defining the end of one sequence to indicate to the decoder to flush all the decoded pictures before the transition point

    - For ITU-T Recommendation H.262 | ISO/IEC 13818-2, this indicator would be sequence_end_code.

    - For ITU-T Recommendation H.264 | ISO/IEC 14496-10, this indicator would be end_of_sequence.

    - For ITU-T Recommendation H.265 | ISO/IEC 23008-2, this indicator would be end_of_sequence.

  - Note the very first picture after the transition point must be a properly constructed random access point for the new codec to begin decode properly

The following sections provide proposed specification text to enable switching between the ITU-T Recommendation H.265 | ISO/IEC 23008-2, ITU-T Recommendation H.264 | ISO/IEC 14496-10 and 2ITU-T Recommendation H.262 | ISO/IEC 13818-2  compression standards.

Note that similar syntax and parameter values are used across the different specification texts to allow for consistency and to signal subsequent transitions to a different profile, level or tier using the same compression standard.

# Proposed specification text for ITU-T Recommendation H.265 | ISO/IEC 23008-2

Entry added to Table D.1 - Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| video_bitstream_transition | The access unit containing the SEI message |

D.2.x	Video bitstream transition SEI message syntax

| video_bitstream_transition( payloadSize ) { | Descriptor |
| --- | --- |
| remaining_pictures_until_next_bitstream | u(4) |
| codec_of_next_bitstream | u(3) |
| profile_of_next_bitstream | u(8) |
| tier_of_next_bitstream | u(1) |
| level_of_next_bitstream | u(8) |
| } |  |

D.3.x	Video bitstream transition SEI message semantics

The video bitstream transition SEI message provides information on an upcoming transition after the current coded video sequence to a bitstream with data encoded using a different compression standard, profile, level, or tier.  The information on the capabilities needed to decode this bitstream can be used by the decoder to anticipate and prepare itself to properly handle the transition.

If a video bitstream transition SEI message is associated with an access unit that is not the access unit immediately preceding the transition, each access unit until the transition shall also be associated with a video bitstream transition SEI message.

remaining_pictures_until_next_bitstream specifies the number of pictures remaining after the current NAL unit before an End of sequence NAL unit which will indicate the last NAL unit before data encoded with the different compression standard, profile, level, or tier begins.

codec_of_next_bitstream specifies the compression standard of the bitstream after the transition point.  The interpretation of codec_of_next_bitstream is specified in Table D.x.

Table D.x - Interpretation of codec_of_next_bitstream

| codec_of_next_bitstream | Compression standard of the bitstream after the transition point |
| --- | --- |
| 0 | Unspecified |
| 1 | ITU-T Recommendation H.262 ¦ ISO/IEC 13818-2 |
| 2 | ITU-T Recommendation H.264 ¦ ISO/IEC 14496-10 |
| 3 | ITU-T Recommendation H.265 ¦ ISO/IEC 23008-2 |
| 4-7 | Reserved |

profile_of_next_bitstream specifies the profile of the bitstream after the transition point.

- When codec_of_next_bitstream is equal to 0, it is a requirement of bitstream conformance that profile_of_next_bitstream shall be equal to 0.

- When codec_of_next_bitstream is equal to 1, profile_of_next_bitstream specifies the value of profile identification in the bitstream after the transition as specified in ITU-T Recommendation H.262 | ISO/IEC 13818-2.

- When codec_of_next_bitstream is equal to 2, profile_of_next_bitstream specifies the value of profile_idc in the bitstream after the transition as specified in ITU-T Recommendation H.264 | ISO/IEC 14496-10.

- When codec_of_next_bitstream is equal to 3, profile_of_next_bitstream specifies the value of general_profile_idc in the bitstream after the transition as specified in ITU-T Recommendation H.265 | ISO/IEC 23008-2.

- Otherwise, the values of profile_of_next_bitstream are reserved for future use by ITU-T | ISO/IEC.

tier_of_next_bitstream specifies the tier of the bitstream after the transition point.

- When codec_of_next_bitstream is equal to 0, 1 or 2, it is a requirement of bitstream conformance that tier_of_next_bitstream shall be equal to 0.

- When codec_of_next_bitstream is equal to 3, tier_of_next_bitstream specifies the value of general_tier_flag in the bitstream after the transition as specified in ITU-T Recommendation H.265 | ISO/IEC 23008-2.

- Otherwise, the values of tier_of_next_bitstream are reserved for future use by ITU-T | ISO/IEC.

level_of_next_bitstream specifies the level of the bitstream after the transition point.

- When codec_of_next_bitstream is equal to 0, it is a requirement of bitstream conformance that level_of_next_bitstream shall be equal to 0.

- When codec_of_next_bitstream is equal to 1, level_of_next_bitstream specifies the value of level identification in the bitstream after the transition as specified in ITU-T Recommendation H.262 | ISO/IEC 13818-2.

- When codec_of_next_bitstream is equal to 2, level_of_next_bitstream specifies the value of level_idc in the bitstream after the transition as specified in ITU-T Recommendation H.264 | ISO/IEC 14496-10.

- When codec_of_next_bitstream is equal to 3, level_of_next_bitstream specifies the value of general_level_idc in the bitstream after the transition as specified in ITU-T Recommendation H.265 | ISO/IEC 23008-2.

- Otherwise, the values of level_of_next_bitstream are reserved for future use by ITU-T | ISO/IEC.

# Proposed specification text for ITU-T Recommendation H.264 | ISO/IEC 14496-10

D.2.x	Video bitstream transition SEI message syntax

| video_bitstream_transition( payloadSize ) { | Descriptor |
| --- | --- |
| remaining_pictures_until_next_bitstream | u(4) |
| codec_of_next_bitstream | u(3) |
| profile_of_next_bitstream | u(8) |
| tier_of_next_bitstream | u(1) |
| level_of_next_bitstream | u(8) |
| } |  |

D.3.x	Video bitstream transition SEI message semantics

The video bitstream transition SEI message provides information on an upcoming transition after the current coded video sequence to a bitstream with data encoded using a different compression standard, profile, level, or tier.  The information on the capabilities needed to decode this bitstream can be used by the decoder to anticipate and prepare itself to properly handle the transition.

If a video bitstream transition SEI message is associated with an access unit that is not the access unit immediately preceding the transition, each access unit until the transition shall also be associated with a video bitstream transition SEI message.

remaining_pictures_until_next_bitstream specifies the number of pictures remaining after the current NAL unit before an End of sequence NAL unit which will indicate the last NAL unit before data encoded with the different compression standard, profile, level, or tier begins.

codec_of_next_bitstream specifies the compression standard of the bitstream after the transition point.  The interpretation of codec_of_next_bitstream is specified in Table D.x.

Table D.x - Interpretation of codec_of_next_bitstream

| codec_of_next_bitstream | Compression standard of the bitstream after the transition point |
| --- | --- |
| 0 | Unspecified |
| 1 | ITU-T Recommendation H.262 ¦ ISO/IEC 13818-2 |
| 2 | ITU-T Recommendation H.264 ¦ ISO/IEC 14496-10 |
| 3 | ITU-T Recommendation H.265 ¦ ISO/IEC 23008-2 |
| 4-7 | Reserved |

profile_of_next_bitstream specifies the profile of the bitstream after the transition point.

- When codec_of_next_bitstream is equal to 0, it is a requirement of bitstream conformance that profile_of_next_bitstream shall be equal to 0.

- When codec_of_next_bitstream is equal to 1, profile_of_next_bitstream specifies the value of profile identification in the bitstream after the transition as specified in ITU-T Recommendation H.262 | ISO/IEC 13818-2.

- When codec_of_next_bitstream is equal to 2, profile_of_next_bitstream specifies the value of profile_idc in the bitstream after the transition as specified in ITU-T Recommendation H.264 | ISO/IEC 14496-10.

- When codec_of_next_bitstream is equal to 3, profile_of_next_bitstream specifies the value of general_profile_idc in the bitstream after the transition as specified in ITU-T Recommendation H.265 | ISO/IEC 23008-2.

- Otherwise, the values of profile_of_next_bitstream are reserved for future use by ITU-T | ISO/IEC.

tier_of_next_bitstream specifies the tier of the bitstream after the transition point.

- When codec_of_next_bitstream is equal to 0, 1 or 2, it is a requirement of bitstream conformance that tier_of_next_bitstream shall be equal to 0.

- When codec_of_next_bitstream is equal to 3, tier_of_next_bitstream specifies the value of general_tier_flag in the bitstream after the transition as specified in ITU-T Recommendation H.265 | ISO/IEC 23008-2.

- Otherwise, the values of tier_of_next_bitstream are reserved for future use by ITU-T | ISO/IEC.

level_of_next_bitstream specifies the level of the bitstream after the transition point.

- When codec_of_next_bitstream is equal to 0, it is a requirement of bitstream conformance that level_of_next_bitstream shall be equal to 0.

- When codec_of_next_bitstream is equal to 1, level_of_next_bitstream specifies the value of level identification in the bitstream after the transition as specified in ITU-T Recommendation H.262 | ISO/IEC 13818-2.

- When codec_of_next_bitstream is equal to 2, level_of_next_bitstream specifies the value of level_idc in the bitstream after the transition as specified in ITU-T Recommendation H.264 | ISO/IEC 14496-10.

- When codec_of_next_bitstream is equal to 3, level_of_next_bitstream specifies the value of general_level_idc in the bitstream after the transition as specified in ITU-T Recommendation H.265 | ISO/IEC 23008-2.

- Otherwise, the values of level_of_next_bitstream are reserved for future use by ITU-T | ISO/IEC.

# Proposed specification text for ITU-T Recommendation H.262 | ISO/IEC 13818-2

The video bitstream transition information is inserted in the picture layer user data of video bitstreams according to this Specification.  The signalling specified supports an upcoming transition after the current video sequence to a bitstream with data encoded using a different compression standard, profile, level, or tier.  The information on the capabilities needed to decode this bitstream can be used by the decoder to anticipate and prepare itself to properly handle the transition.

If video bitstream transition information is inserted in the picture layer user data of a picture that is not the picture immediately preceding the transition, each picture until the transition shall have video bitstream transition information inserted in the picture layer user data.

| video_bitstream_transition_data() { | No. of bits | Mnemonic |
| --- | --- | --- |
| video_bitstream_transition_data_identifier | 32 | bslbf |
| remaining_pictures_until_next_bitstream | 4 | uimsbf |
| codec_of_next_bitstream | 3 | uimsbf |
| profile_of_next_bitstream | 8 | uimsbf |
| tier_of_next_bitstream | 1 | uimsbf |
| level_of_next_bitstream | 8 | uimsbf |
| } |  |  |

video_bitstream_transition_data_identifier – The bit string 'XXXXXXXX' in hexadecimal. (No preference on exact bit string and request guidance from the group on best method to avoid potential conflicts with other applications of the user_data() mechanism.)  When the user_data bytes of the user_data() syntax structure begin with this bit string, the remaining bytes of the user_data() syntax structure shall conform to the video_bitstream_transition_data() syntax structure.

remaining_pictures_until_next_bitstream specifies the number of pictures remaining after the current picture before a sequence_end_code which will indicate the last picture before data encoded with the different compression standard, profile, level, or tier begins.

codec_of_next_bitstream specifies the compression standard of the bitstream after the transition point.  The interpretation of codec_of_next_bitstream is specified in Table D.x.

Table D.x - Interpretation of codec_of_next_bitstream

| codec_of_next_bitstream | Compression standard of the bitstream after the transition point |
| --- | --- |
| 0 | Unspecified |
| 1 | ITU-T Recommendation H.262 ¦ ISO/IEC 13818-2 |
| 2 | ITU-T Recommendation H.264 ¦ ISO/IEC 14496-10 |
| 3 | ITU-T Recommendation H.265 ¦ ISO/IEC 23008-2 |
| 4-7 | Reserved |

profile_of_next_bitstream specifies the profile of the bitstream after the transition point.

- When codec_of_next_bitstream is equal to 0, it is a requirement of bitstream conformance that profile_of_next_bitstream shall be equal to 0.

- When codec_of_next_bitstream is equal to 1, profile_of_next_bitstream specifies the value of profile identification in the bitstream after the transition as specified in ITU-T Recommendation H.262 | ISO/IEC 13818-2.

- When codec_of_next_bitstream is equal to 2, profile_of_next_bitstream specifies the value of profile_idc in the bitstream after the transition as specified in ITU-T Recommendation ITU-T Recommendation H.264 | ISO/IEC 14496-10.

- When codec_of_next_bitstream is equal to 3, profile_of_next_bitstream specifies the value of general_profile_idc in the bitstream after the transition as specified in ITU-T Recommendation H.265 | ISO/IEC 23008-2.

- Otherwise, the values of profile_of_next_bitstream are reserved for future use by ITU-T | ISO/IEC.

tier_of_next_bitstream specifies the tier of the bitstream after the transition point.

- When codec_of_next_bitstream is equal to 0, 1 or 2, it is a requirement of bitstream conformance that tier_of_next_bitstream shall be equal to 0.

- When codec_of_next_bitstream is equal to 3, tier_of_next_bitstream specifies the value of general_tier_flag in the bitstream after the transition as specified in ITU-T Recommendation H.265 | ISO/IEC 23008-2.

- Otherwise, the values of tier_of_next_bitstream are reserved for future use by ITU-T | ISO/IEC.

level_of_next_bitstream specifies the level of the bitstream after the transition point.

- When codec_of_next_bitstream is equal to 0, it is a requirement of bitstream conformance that level_of_next_bitstream shall be equal to 0.

- When codec_of_next_bitstream is equal to 1, level_of_next_bitstream specifies the value of level identification in the bitstream after the transition as specified in ITU-T Recommendation H.262 | ISO/IEC 13818-2.

- When codec_of_next_bitstream is equal to 2, level_of_next_bitstream specifies the value of level_idc in the bitstream after the transition as specified in ITU-T Recommendation ITU-T Recommendation H.264 | ISO/IEC 14496-10.

- When codec_of_next_bitstream is equal to 3, level_of_next_bitstream specifies the value of general_level_idc in the bitstream after the transition as specified in ITU-T Recommendation H.265 | ISO/IEC 23008-2.

- Otherwise, the values of level_of_next_bitstream are reserved for future use by ITU-T | ISO/IEC.

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).