| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0116 |
| --- | --- |

| Title: | Non-CE1: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tao Lin, Kailun Zhou, Liping Zhao, Xianyi Chen 1239 Siping Ave, Shanghai 200092, China | Tel: Email: | lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

A Palette with Pixel Copy (PPC) tool is added under the existing palette mode and coexists with all of the traditional palette coding tools. A CU is either coded by PPC or traditional palette mode. The palette predictor is shared. The palette in PPC is used only for LEFT copy identical to traditional palette mode. The search range for PPC is either 2CTU (current and left one) or 4CTU (current and left three). The coding performance is evaluated under two configurations of search/reference range: (TC1) FF IBC vs 2CTU PPC and (TC2) constrained 4CTU IBC vs 4CTU PPC. Using SCM40 as anchor, their coding results for Y component of YUV TGM AI lossy coding are reported as -4.2% and -7.7%, respectively for TC1 and TC2. Encoding time ratio is 5-6%.

# Introduction

The PPC technique is a natural extension to traditional palette coding. It provides flexible coding features to fit a variety of picture contents. PPC is more enhancement to than overlapping with traditional palette coding. In particular, by sharing palette predictor with traditional palette coding, PPC makes palette predictor less interrupted by other coding modes, thus improves the coding performance of traditional palette coding.

# Proposed methods

A PPC sub-mode is added under the traditional palette mode, which becomes a sub-mode in parallel with the added PPC sub-mode.

In PPC, a string is either a palette color string or a pixel string.

For a palette color string, a palette index and a run are coded into the bitstream.

For a pixel string, an offset and a length are coded into the bitstream.

A palette color string is always a LEFT copy string.

The search range of pixel string is either 2CTU (current and left one) or 4CTU (current and left three).

An unmatched pixel is coded without quantization directly into the bistream using a special palette index.

No residual is coded.

CABAC throughput is shown in the following table.

| Prediction Mode | PPC |  |
| --- | --- | --- |
| (Note: CU Size = 8x8) | Syntax Elements | Number |
| Mode Flag | cu_transquant_bypass_flag | 1 |
|  | cu_skip_flag | 1 |
|  | palette_mode_flag | 1 |
|  | ppc_submode_flag ppc_scan_mode_flag | 1 |
|  | ppc_palette_color_present_flag | 1 |
|  | ppc_unmatched_pixel_present_flag | 1 |
| Partition |  |  |
| MV or BV | ppc_pixel_string_flag | 64 |
|  | ppc_above_string_flag | 64 |
|  | ppc_offsetX_zero_flag | 64 |
|  | Ppc_offsetY_zero_flag | 64 |
|  | ppc_length_minus1_prefix | 64 |
| Residual (4 4x4 TBs, 3 Color Components) | N/A |  |
| Number of Context-coded Bins |  | 327 |
| Bins per Pixel |  | 5.11 |

# Experimental Results

FF IBC vs 2CTU PPC, lossy

| Anchor_Tested1 | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, TGM, 1080p & 720p | -3.8% | -4.1% | -4.1% | -2.5% | -2.7% | -2.7% | -2.0% | -2.0% | -1.9% |
| RGB, MC, 1440p & 1080p | -1.2% | -1.7% | -1.7% | -0.7% | -1.0% | -1.1% | 0.1% | -0.7% | -0.6% |
| RGB, Ani, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.1% | 0.1% |
| RGB, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -4.2% | -4.1% | -4.1% | -2.4% | -2.3% | -2.5% | -1.6% | -1.7% | -1.8% |
| YUV, MC, 1440p & 1080p | -1.5% | -2.0% | -2.3% | -0.9% | -1.4% | -1.9% | -0.3% | -1.2% | -1.6% |
| YUV, Ani, 720p | 0.0% | 0.0% | -0.1% | -0.1% | -0.3% | 0.1% | 0.0% | 0.2% | 0.0% |
| YUV, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% |
| Enc Time[%] | 105% |  |  | 106% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  | 103% |  |  |

Encoding time ratio is quite accurate and verified by two cross-checkers (Huawei and NCTU).

4CTU IBC vs 4CTU PPC

| Anchor_Tested1 | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, TGM, 1080p & 720p | -7.4% | -8.0% | -7.9% | -5.1% | -5.4% | -5.3% | -3.1% | -3.3% | -3.3% |
| RGB, MC, 1440p & 1080p | -2.4% | -3.2% | -3.2% | -1.2% | -1.9% | -1.9% | -0.4% | -1.2% | -1.1% |
| RGB, Ani, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| RGB, CC, 1080p | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -7.7% | -7.7% | -7.9% | -5.0% | -4.9% | -5.0% | -2.6% | -2.5% | -2.8% |
| YUV, MC, 1440p & 1080p | -3.2% | -4.1% | -4.3% | -1.8% | -2.2% | -2.7% | -0.6% | -1.3% | -1.7% |
| YUV, Ani, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.2% |
| YUV, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 106% |  |  | 104% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  | 100% |  |  |

FF IBC vs 2CTU PPC, lossless

| Anchors_Tested1 | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, TGM, 1080p & 720p | -4.8% | -5.6% | -14.9% | -0.9% | -2.8% | -3.5% | -10.2% | -0.4% | -2.6% | -2.8% | -6.6% | -0.2% |
| RGB, MC, 1440p & 1080p | -0.8% | -0.9% | -1.6% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| RGB, A, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -5.2% | -6.0% | -15.4% | -1.0% | -2.8% | -3.9% | -10.5% | -0.3% | -2.6% | -3.1% | -7.5% | -0.2% |
| YUV, MC, 1440p & 1080p | -0.9% | -1.0% | -1.7% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, A, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 118% |  |  |  | 106% |  |  |  | 104% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 103% |  |  |  | 102% |  |  |  |

To be added.

# Conclusion

In this contribution, PPC coding tool is presented The coding performance improvement of PPC under two different search range is tested. Up to 7.7% average BD-rate reduction in lossy coding is observed in the All-Intra configuration in TC2 (4CTU case). Based on the quite good coding performance improvement, we would recommend the committee to adopt this coding mode in SCC draft standard.

# References

- JCT-VC, “HM-16.4+SCM-4.0,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-S1015, Strasbourg, Oct. 2014.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).