| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0117 |
| --- | --- |

| Title: | Cross-check of JCTVC-U0122 CE1 Test D: Modification of Palette Run Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | PoLin Lai, Jing Ye, Jungsun Kim 2860 Junction Ave, San Jose, CA, USA | Email: | {polin.lai, jing.ye, jungsun.kim } @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents cross-check report for JCTVC-U0122 CE1 Test D: Modification of Palette Run Coding. Software study was conducted and confirms that the proposed palette run-length coding modification is implemented as described in the contribution and Draft Text. The cross-checker’s simulation results match the data provided by the proponent.

# Introduction

JCTVC-U0122 CE1 Test D: Modification of Palette Run Coding proposes the following:

If the current pixel is the first pixel in the line, a flag is signalled into the bitstream, indicating whether it ends at the last pixel in certain line. If so, the number of whole lines (L) the current run spans is coded into the bitstream using coefficient coding function (xWriteCoefRemainExGolomb) with order 0. Instead of directly signalling L, a mapping process is used to map maximal feasible value to zero and shift L up by 1 if it is less than the maxima. Otherwise, it falls back to normal palette run length coding with an independent set of context models.

If the current pixel is not the first pixel in the line, run length L is coded into the bitstream using xWriteCoefRemainExGolomb with order 0. Instead of signalling L directly, if the current run ends at one line below the current line, a mapping procedure is used to reorder the candidate positions in the line according to their horizontal distance between themselves and the current pixel.

# Cross-check

The cross-checker conducted study of the software, and confirms that the proposed palette run-length coding modification implemented as described in the contribution and Draft Text.

The cross-checker’s simulation results match the data provided by the proponent.

The change consists of two parts:

- The syntax to indicate a run starts at the first pixel in a line and ends at the last pixel of the same line or a following line.

- Mapping of the run-length (pixel unit), and mapping of the number of whole lines

To understand the effect of the two parts, the cross-checker also conducted short Intra test (1 second):

- Both parts A and B enabled, 	lossy 444 AI, full-frame IBC, 1 second tests.

- Syntax change only (no part B), 	lossy 444 AI, full-frame IBC, 1 second tests.

The results indicate that each part contributes roughly half of the gain (BD-rate saving).

| Lossy 444, Full-frame IBC, 1 second test | Syntax design Length Mapping |  |  | Syntax design Length Mapping |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | All Intra |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.4% | -0.5% | -0.2% | -0.3% | -0.3% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -0.5% | -0.1% | -0.2% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.4% | -0.4% | -0.1% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

- Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).