| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0118 |
| --- | --- |

| Title: | On intra block copy bitstream constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Krishna Rapaka Rajan Joshi Marta Karczewicz Ye-Kui Wang | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

At the previous meeting, the intra block copy (IBC) design was changed to be aligned with inter mode, with several bitstream constraints added to control the intended intra block behaviour. It is asserted that, however, some of the constraints are not aligned with inter mode. In this contribution, IBC bitstream constraints are proposed to be converted into signalling and decoder side restrictions for robustness. It is proposed to modify the collocate_ref_idx signalling such that it cannot refer to the current picture, allow IBC motion vector to point outside of the picture as in inter mode, infer current picture as a reference when IBC mode is enabled and the number of active pictures in a slice is equal to 1, and convert a not-valid motion vector into a valid reference.

# Problem statement

In the current unified-with-inter IBC design, the following bitstream constraints shall be obeyed:

- The current picture cannot be a collocated picture.

- The IBC predictor shall be a block within the current picture.

- The IBC predictor cannot overlap with the current CU and shall be from the wavefront style reference area. The area is constrained as follows:

( xPb + ( mvLX[ 0 ]  >>  2 ) + nPbSw − 1 ) / CtbSizeY − xCurr / CtbSizeY  <=
		yCurr/CtbSizeY − ( yPb + ( mvLX[ 1 ]  >>  2 ) + nPbSh − 1 ) / CtbSizeY

- If the number of reference indices in RefPicList0 is equal to 1 with IBC enabled, the reference picture shall be the current picture.

The block availability is derived as specified in clause Error: Reference source not found with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2), (yPb + mvLX[ 1 ]  >>  2) ) as inputs.

In the first constraint, collocated_ref_idx, pointing to the current picture, is possible to be signalled and only the value is restricted by a bitstream constraint. The IBC MV is restricted in a way such that the IBC predictor shall be located within the current picture and it is different from the inter mode, where an outside picture predictor is possible. In the fourth constraint, RPLM becomes mandatory to be signalled if the total number of pictures is greater than 1 but only one reference picture is used in the slice when IBC is enabled.

# Proposal

In this contribution, it is suggested to make these constraints stricter to avoid bitstreams by some encoders with the constraints broken.

## Collocated picture constraints

In the current WD, there is a constraint that the current picture cannot be used as a collocated picture. The reason for this constraint is that there would be no motion field yet assigned for the collocated block to be used in the motion prediction if the collocated picture were the current picture, while the value of collocate_ref_idx referring to the current picture can be signalled.

It is proposed to modify the collocated_ref_idx signalling to make such a signalling impossible, by excluding the reference indices pointing to the current picture.

Firstly, the number of indices in RefPicList0 pointing to the current picture is counted as follows:

NumCurrRef0 = 0
for( i = 0; i  <=  num_ref_idx_l0_active_minus1; i++ ) {
	if( RefPicList0[ i ] is the current picture )
		NumCurrRef0++
}

The collocated reference index is signalled only when the number of possible collocated indices is more than 1, e.g., as follows:

| if( slice_temporal_mvp_enabled_flag ) { |  |
| --- | --- |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && (num_ref_idx_l0_active_minus1 − NumCurrRef0 ) > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |

Secondly, the value of the collocated_ref_idx to be signalled in a bitstream is modified as follows, where N is the initial value of the index:

collocated_ref_idx = N
for( i = 0; i < N; i++ ) {
	if( RefPicList0[ i ] is the current picture )
		collocated_ref_idx− −
}

Finally, the parsed collocated_ref_idx is updated to get the intended collocated reference index as follows:

for( i = 0; i  <=  collocated_ref_idx; i++ ) {
	if( RefPicList0[ i ] is the current picture )
		collocated_ref_idx++
}

And the constrained text can be removed from the specification:

It is a requirement of bitstream conformance that the picture referred to by collocated_ref_idx shall be the same for all slices of a coded picture and shall not be the current picture itself.

With the proposed modification, the collocated reference index pointing to the current picture is not possible to be signalled. Additionally, several bits saving can be achieved by reduced collocated_ref_idx value and eliminating the signalling in some cases.

## RPLM signalling modification

In the current specification draft, the RPLM process is invoked when NumPicTotalCurr is greater than 1 and lists_modification_present_flag is equal to 1. When NumPicTotalCurr is greater than 1 and num_ref_idx_l0_active_minus1 is equal to 0 (that is the maximum reference index for reference picture list 0 that may be used to decode the slice is equal to 0) and curr_pic_as_ref_enabled_flag is equal to 1 the list modification information is required to be signalled to make the current picture added into RefPicList0, since the current picture is added a last one into the reference picture list, however this information is redundant as it can be inferred to include the current picture as the first entry in RefPicList0.

It is proposed to include this inference into the RefPicList0 derivation as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++) {		(8-1)
			rIdxTemp = ( num_ref_idx_l0_active_minus1  = =  0  &&  curr_pic_as_ref_enabled_flag ) ?
																			NumPicTotalCurr – 1 : rIdx
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdxTemp ]

}

Additionally, it is proposed to restrict the signalling of RPLM information for RefPicList0 as follows

| ref_pic_lists_modification( ) { | Descriptor |
| --- | --- |
| if( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ !curr_pic_as_ref_enabled_flag ) |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| list_entry_l0[ i ] | u(v) |
| if( slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| list_entry_l1[ i ] | u(v) |
| } |  |
| } |  |

## Extend IBC predictor to outside picture

In the current IBC design, IBC predictor is constrained to be within the picture, this restrictions makes it different from the inter mode. It is proposed to remove predictor constraint for IBC. The text change can be as follows:

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to (Clip3(0, xPb + (mvLX[ 0 ]  >>  2), pic_width_in_luma_samples), Clip3(0,  (yPb + mvLX[ 1 ]  >>  2), pic_height_in_luma_samples) ) as inputs, the output shall be equal to TRUE.

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( Clip3(0, xPb + (mvLX[ 0 ]  >>  2) + nPbW – 1, pic_width_in_luma_samples), Clip3(0, yPb + (mvLX[ 1 ]  >>  2) + nPbH – 1, pic_height_in_luma_samples) ) as inputs, the output shall be equal to TRUE.

## Clipping and padding for IBC reference region

To avoid bitstream restrictions which potentially can be broken by some encoders, it is proposed to clip the IBC reference region to valid area in the decoder when the bitstream references is out-of bound regions.

The decoder clips the reference region

- When the reference region is outside the picture boundary

- When the reference region is outside the parallel processing sample boundary as defined below

( xPb + ( mvLX[ 0 ]  >>  2 ) + nPbSw − 1 ) / CtbSizeY − xCurr / CtbSizeY  <=
		yCurr/CtbSizeY − ( yPb + ( mvLX[ 1 ]  >>  2 ) + nPbSh − 1 ) / CtbSizeY

- When the reference region overlaps with the current coding unit

After clipping, if the reference region overlaps with the region that is not available, all luma samples in the predictor block are set equal to 1  <<  ( BitDepthY − 1 ), and all chroma samples in the predictor block are set equal to 1  <<  ( BitDepthC − 1 ).

Working draft changes

8.5.3.2.1	General

…

Let the positions (xAi, j, yAi, j) represent the bottom-right sample location of the current luma prediction block relative to the top-left luma sample of the current picture at full-sample locations inside the given two-dimensional array when refPicLXL = CurrPic of luma samples. Let CtbYcurr, CtbYPred and CtbXcurr  represent the CTB location corresponding to yCurr, yAi, j,, xCurr .

yAi, j = Clip3( nPbH – 1, (CtbYcurr + 1)  <<  CtbLog2SizeY – 1, yIntL + j )

xAi, j = Clip3( nPbW – 1, ((CtbYcurr – CtbYPred + 1) )  <<  CtbLog2SizeY) + xCurrctb – 1,  xIntL + i )

mvLX[ 0 ] = xCurr –  xAi, j – nPbW

mvLX[ 1 ] = yCurr –  yAi, j – nPbH

It is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2), (yPb + mvLX[ 1 ]  >>  2) ) as inputs, if the output is equal to FALSE, all luma samples in the predictor block are set equal to 1  <<  ( BitDepthY − 1 ), and all chroma samples in the predictor block are set equal to 1  <<  ( BitDepthC − 1 ).  the output shall be equal to TRUE.

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2) + nPbW − 1, yPb + (mvLX[ 1 ]  >>  2) + nPbH – 1 ) as inputs, and if the output is equal to FALSE, all luma samples in the predictor block are set equal to 1  <<  ( BitDepthY − 1 ), and all chroma samples in the predictor block are set equal to 1  <<  ( BitDepthC − 1 ).  the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ]  >>  2 ) + nPbW + xB1 is less than or equal to 0.

- The value of ( mvLX[ 1 ]  >>  2 ) + nPbH + yB1 is less than or equal to 0.

- The following condition shall be true:

( xPb + ( mvLX[ 0 ]  >>  2 ) + nPbSw − 1 ) / CtbSizeY − xCurr / CtbSizeY  <=
		yCurr/CtbSizeY − ( yPb + ( mvLX[ 1 ]  >>  2 ) + nPbSh − 1 ) / CtbSizeY

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).