| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0119 |
| --- | --- |

| Title: | On slice segment freeze signalling for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thorsten Laude Leibniz Universitaet Hannover Insitut fuer Informationsverarbeitung Appelstrasse 9a 30167 Hannover Germany | Tel: Email: | +4951176219588 laude@tnt.uni-hannover.de |
| Source: | Leibniz Universitaet Hannover, Institut fuer Informationsverarbeitung |  |  |

_____________________________

# Abstract

This document proposes a segment freeze process that is enabled  by high-level syntax elements. It is stated that the principle of the proposed freeze process consists of the freezing of a slice segment in the current picture for subsequently coded pictures. Furthermore, it is described that no syntax is coded in subsequent pictures to reconstruct the samples of the part of the picture corresponding to the frozen slice segment. Instead, it is asserted that the frozen slice segment is copied to the subsequent picture. Additionally, a termination method for the freeze process is proposed to end the freezing of a slice segment.

# Introduction

The absence of changes between consecutive pictures is a common characteristic in many screen content applications. For instance, considering a remote computing scenario, it is conceivable that the screen content might remain unchanged for several seconds or even minutes. For a typical picture rate of 25 pictures per second this results in 1500 unchanged pictures per minute. The current HEVC screen content coding (SCC) draft [1] provides the mean of changing the picture rate by signaling VUI or VPS syntax elements. However, this mean is unsuitable to solve the given problem due to the low delay requirement imposed upon many screen content applications which prevents the analysis of subsequent pictures. Thus, at the time instance when a picture is coded, it is unclear whether the content of the picture will be static in subsequent pictures. Additionally, variable picture rates are only applicable to cope with unchanged content if the entire picture is unchanged. Taking into account that typically only relatively small parts of the screen content change when a change occurs (e.g. a moving cursor) it is evident that variable picture rates are not beneficial for this scenario although the major part of the picture may be unchanged.

Another approach to code unchanged screen content is the usage of existing coding tools (e.g. the skip mode) in combination with the high compression ratio of CABAC for repetitive bins which was asserted to be around 1:45 during the last JCT-VC meeting [2]. Unfortunately, CABAC is not available for high level syntax (HLS). However, considering the low overall bit rates for some screen content applications, the signaling overhead for HLS is not negligible.

Furthermore, assuming that a picture may be partitioned into slice segments, it is likely that the part of the screen content which is covered by a particular slice segment is unchanged for many pictures while other slice segments in these might very well change. For this scenario a slice segment freeze process is proposed.

# Slice segment freeze process

The main idea of the proposed slice segment freeze process is the signalling for a slice segment of the current picture that this slice segment will be frozen for subsequently coded pictures. Henceforth, all data corresponding to this slice segment will be copied to subsequent pictures. Thus, no signalling is required for the corresponding part of subsequent pictures (no slice segment header, no block level syntax) since all information is copied from the frozen slice segment.

Figure 1 illustrates an example for the proposed slice segment freeze process. For this example it is assumed that four pictures are coded at time instances t to t+3. Each of the pictures is partitioned into five slice segments denoted as “Slice 1” to “Slice 5”. Additionally, it is described in which picture a slice segment has originally been coded (e.g. “Slice 1 (t)” means that the first slice segment has been coded at time instance t). The slice segment freeze process is highlighted by arrows. For instance, the slice segment freeze process is used for the first three slice segments of the picture coded at time instance t. In consequence, these three slices are copied to the picture coded at time instance t+1. In the following, the slice segment “Slice 2” is further copied to the picture coded at time instance t+2. Similar, the slice segment freeze process is applied for the fourth and fifth slice segment of the picture at time instance t+2.

Figure 1: example for the slice segment freeze process

When the slice segment freeze process is used for a slice segment, it is indeterminate for how many subsequent pictures the slice segment will be frozen. Therefore, it is proposed to freeze the slice segment until a new slice segment or several new slice segments are coded which cover the part of the picture which were previously covered by the frozen slice segment. Due to this slice segment freeze process termination method the freeze process does not require any analysis of future pictures and thus is beneficial for applications with low delay constraints.

In addition to the slice segment freeze process, it is proposed to impose constraints on the partitioning of subsequent pictures if the slice segment freeze process is used for a slice segment in the current picture. As one constraint it is proposed to specify that the remaining part of the subsequent picture, which cannot be filled by copied frozen slice segments, must be completely filled by other slice segments. As another constraint, it is proposed to specify for the case that the slice segment freeze process termination method is used, that the entire part of the picture, which was previously filled by a frozen slice segment, must be filled by newly coded slice segments.

# Text specification

It is proposed to signal the usage of the slice segment freeze process as part of the slice segment header following Table 1. Changes relative to the HEVC SCC draft 3 [1] are highlighted in yellow. The syntax element slice_segment_freeze_flag specifies the usage of the described slice segment freeze process. slice_segment_freeze_flag equal to 1 specifies that the slice segment will be frozen. If not present, slice_segment_freeze_flag is inferred to be equal to 0.

Table 1: signalling of the slice segment freeze process in the slice segment header

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| […] |  |
| slice_segment_freeze _flag | u(1) |
| byte_alignment( ) |  |
| } |  |

Furthermore, it is proposed to add syntax elements on a higher level to enable and disable the slice segment freeze process. Table 2 presents the proposed sequence parameter set syntax. freeze_process_enabled_flag equal to 1 specifies that the slice segment freeze process may be used for slice segments. freeze_process_enabled_flag equal to 0 specifies that the slice segment freeze process is not used. If not present, freeze_process_enabled_flag is inferred to be equal to 0.

Table 2: signalling of the enabling of the slice segment freeze process in the sequence parameter set

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| freeze_process_enabled_flag | u(1) |
| […] |  |
| } |  |

If the slice segment freeze mode is used for a slice segment, it is proposed to invoke the following decoding process for every colour component to reconstruct the samples of the copied slice segment.

Inputs to this process are:

–	a location ( xCurr, yCurr ) specifying the top-left sample of the frozen slice segment relative to the top-left sample of the current picture component,

–	an array recSamplesFrozenSlice specifying the reconstructed samples of the frozen slice segment,

–	a variable cIdx specifying the colour component.

Output of this process is an array recSamples specifying the reconstructed samples of the copied slice segment.

Depending on the value of the colour component cIdx, the following assignments are made:

–	If cIdx is equal to 0, recSamplesFrozenSlice corresponds to the reconstructed picture sample array after deblocking recPictureL of the picture to which the frozen slice corresponds and recSamples corresponds to the reconstructed picture sample array after deblocking recPictureL.

–	Otherwise, if cIdx is equal to 1, recSamplesFrozenSlice corresponds to the reconstructed picture sample array after deblocking recPictureCb of the picture to which the frozen slice corresponds and recSamples corresponds to the reconstructed picture sample array after deblocking recPictureCb.

–	Otherwise (cIdx is equal to 2), recSamplesFrozenSlice corresponds to the reconstructed picture sample array after deblocking recPictureCr of the picture to which the frozen slice corresponds and recSamples corresponds to the reconstructed picture sample array after deblocking recPictureCr.

The reconstructed samples array recSamples is derived as follows:

recSamples[ xCurr + i ][ yCurr + j ] = recSamplesFrozenSlice[ xCurr + i ][ yCurr + j ]

with ( xCurr + i, yCurr + j ) covering all locations in the frozen slice segment.

# Recommendation

This contribution proposes a slice segment freeze process that is enabled by new high-level syntax elements. With this new method, the static part in screen content can be more efficiently coded with uniform quality and extreme low bitrates while it remains static across pictures. It is recommended to adopt the proposal in the HEVC SCC extension.

# References

[1]	R. Joshi, S. Liu, J. Xu, and Y. Ye, JCT-VC T1005: High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 3. 20th Meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Geneva, CH. 2015.

[2]	G. J. Sullivan and J.-R. Ohm, Meeting report of the 20th meeting of the Joint Collaborative Team on Video Coding. 2015.

# Patent rights declaration(s)

Leibniz Universitaet Hannover, Institut fuer Informationsverarbeitung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).