| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0122 |
| --- | --- |

| Title: | CE1 Test D: Modification of Palette Run Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Wei Pu, Rajan Joshi, Feng Zou, Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 martak@qti.qualcomm.com Wei.Pu@qti.qualcomm.com rajanj@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In Geneva meeting, CE1 Test D on modification of palette run coding is established. In this test, improvement on palette run length coding depending on the current pixel’s position is tested. It is reported that for text & graphics with motion, 1080p & 720p, -0.4% and -0.5% BD-rate are achieved for RGB and YUV test sequences, respectively with full frame intra block copy search range. For 4 CTU intra block copy search range test, -0.5% and -0.6% BD-rate are achieved.

# Introduction

In palette mode, the palette run length syntax element is coded using multiple CABAC contexts. The contexts mainly depend on two classes of information. First, it depends on the run type, i.e. whether the current run is copy above run or index copy run. Second, if the run to be coded is index run, the contexts further depend on its corresponding index values.

However, it is noticed the palette run also depends on the current pixel’s position in the block. This contribution reports the test results of palette run coding depending on positions.

# Technical description

In this test, position information is exploited.

If the current pixel is the first pixel in the line, a flag is signalled into the bitstream, indicating whether it ends at the last pixel in certain line. If so, the number of whole lines (L) the current run spans is coded into the bitstream using coefficient coding function (xWriteCoefRemainExGolomb) with order 0. Instead of directly signalling L, a mapping process is used to map maximal feasible value to zero and shift L up by 1 if it is less than the maxima. Otherwise, it falls back to normal palette run length coding with an independent set of context models.

If the current pixel is not the first pixel in the line, run length L is coded into the bitstream using xWriteCoefRemainExGolomb with order 0. Instead of signalling L directly, if the current run ends at one line below the current line, a mapping procedure is used to reorder the candidate positions in the line according to their horizontal distance between themselves and the current pixel. An example of this procedure is illustrated in Fig. 1.

Fig. 1. Position reordering according to their horizontal distance between current pixel (C).

# Experimental results

Simulation results are carried out using SCC common test condition [2]. Only all intra lossy results are presented in this section. Complete test results are included in the contribution package.

Table I. Proposed method v.s. SCM4.0, 4:4:4 format, 4 CTU intra block copy search range

Table II. Proposed method v.s. SCM4.0, 4:4:4 format, full frame intra block copy search range

Table III. Proposed method v.s. SCM4.0, non 4:4:4 format

# References

- R. Joshi, S. Liu, J. Xu, and Y. Ye, “HEVC screen content coding draft text 3,” in JCTVC-T1005, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” in JCTVC-T1015, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015.

- M. Karczewicz, R. Joshi, W. Pu, V. Seregin, and F. Zou, “Non-CE1: Modification of palette run coding,” in JCTVC-T0133, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).