| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PO, 19 – 26 June 2015 | Document: JCTVC-U0134r1 |
| --- | --- |

| Title: | Upsampling and downsampling filters for HDR content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Dmytro Rusanovskyy Joel Sole Rojals Marta Karczewicz Sungwon Lee Done Bugdayci Sansli Jianle Chen 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

The proposal documents several chroma upsampling and downsampling filters that are shown to provide objective gains in coding HDR content. Different pairs of upsampling and downsampling filters are reported to be used with different sequences. Objective gains of 1.9% in deltaE BD-rate is reported for the HDR test sequences. It is proposed to allow signalling of the best upsampling and downsampling filters in SEI messages to enable decoders to choose the filter to decode the bitstreams that would provide the best objective performance.

In revision r1 of this document, references to other JCT-VC proposals that are related to upsampling and downsampling filters proposed for HDR content are added.

# Introduction

The MPEG HDR/WCG CfE [1] describes an end-to-end coding chain for HDR video content. Video is captured in the linear domain, in 4:4:4 RGB colour space with floating point accuracy. This data is then processed (after applying OETF, colour space conversion to YUV and quantizatioin to 10 bits) to obtain 10-bit 4:2:0 YUV version which is then coded using an HEVC Main 10 profile encoder or others that may be developed. Figure 1 describes the end-to-end coding chain as described in the CfE.

Figure 1: End-end-end HDR coding chain

This document concentrates on the colour space down-conversion and up-conversion, also referred to as downsampling and upsampling, respectively. This is done in the YUV colour space in the non-linear domain. Similarly, colour space up-conversion is also necessary to reconstruct the HDR content at the decoder side. Downsampling and upsampling filters are typically employed in these two steps, respectively, and the application of these filters helps in avoid aliasing.

The current CfE uses short filters that are described in Table 1 and Table 2 below.

Table 1: Default downsampling filters used in the HDR/WCG CfE

| Phase | -1 | 0 | 1 |
| --- | --- | --- | --- |
| Vertical | 0 | 4 | 4 |
| Horizontal | 1 | 6 | 1 |

Table 2: Default upsampling filters used in the HDR/WCG CfE

| Phase | -2 | -1 | 0 | 1 |
| --- | --- | --- | --- | --- |
| Vertical | -4 | 36 | 36 | -4 |
| Horizontal | -2 | 16 | 54 | -4 |
|  | -4 | 54 | 16 | -2 |

The chroma samples locations for 4:2:0 format are chroma_sample_loc_type_top_field equal to 0.

The document JCTVC-U0041 [2] describes several methods to improve the upsampling and downsampling filter techniques currently used in the CfE, and also provides comparison of the objective metrics for uncompressed HDR content with respect to the anchor. In this document, we propose several downsampling and upsampling filters that have shown objective quality improvement for coding HDR content. Several pairs of upsampling and downsampling filters were tested for different sequences, and the best filter pair is reported. It is argued that an encoder can choose the best filter pair (downsampling and upsampling), and send this information about to the decoder side using side-information, thus enabling the decoder to use to realize objective gains.

# Proposed method

The set of filters used to test were obtained using several parameters for the cosine-windowed sinc filter and Lanczos filters. The different filters used are described in Table 3 and Table 4. All the filters have co-sited chroma resampling locations (chroma_sample_loc_type_top_field equal to 2), and same upsampling/downsampling filters are used in the horizontal and the vertical directions.

Table 3: Downsampling filters used

| DS Filter ID/Phase | -6 | -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DS_A | -3 | -4 | -2 | 3 | 11 | 17 | 20 | 17 | 11 | 3 | -2 | -4 | -3 |
| DS_B |  |  | -4 | -7 | 11 | 38 | 52 | 38 | 11 | -7 | -4 |  |  |
| DS_C |  |  | 0 | -7 | 0 | 39 | 64 | 39 | 0 | -7 | 0 |  |  |
| DS_D |  | 1 | 0 | -9 | 0 | 39 | 66 | 39 | 0 | -9 | 0 | 1 |  |
| DS_E |  | 0 | -3 | -7 | 8 | 38 | 56 | 38 | 8 | -7 | -3 | 0 |  |
| DS_F | 0 | 3 | 0 | -10 | 0 | 39 | 64 | 39 | 0 | -10 | 0 | 3 | 0 |

Table 4: Upsampling filters used

| US Filter ID/Phase | -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| US_A |  |  | 2 | -8 | 38 | 38 | -8 | 2 |  |  |
| US_B | 4 | -5 | 8 | -13 | 38 | 38 | -13 | 8 | -5 | 4 |
| US_C |  | -1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 |  |

## Case for adaptive filter coefficients

The filters described above were used to test the HDR test sequences as per the test conditions described in the CfE guidelines. The simulations are performed using HM version 16.2 and HDR Tools version r589. Only the upsampling and the downsampling filters were changed, and rest of the coding chain was kept as described in the CfE. The OETF used was the PQ transfer function. It is observed that varying the filter coefficients used for performing the upsampling and downsampling can provide BD-rate gains (on an average, 1.9% for deltaE metric) for the sequences. Table 5 provides the BD-rate performance for the best combination of the filters for the HDR test sequences. For one sequence, one pair of downsampling-upsampling filter pair was chosen.

Table 5: Objective metric gains using different filter pairs for the HDR sequences

| Sequence | tPSNRxyz-X | tPSNRxyz-Y | tPSNRxyz-Z | tPSNR-XYZ | PSNR_ deltaE | mfPSNR | DS filter used | US filter used |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEater | -0.71% | -0.18% | -1.82% | -0.93% | -1.09% | -1.51% | DS_D | US_A |
| Tibul | -1.06% | -0.29% | -3.06% | -1.20% | -2.55% | -2.03% | DS_A | US_C |
| Market | -0.11% | 0.00% | -0.71% | -0.34% | -1.86% | -0.43% | DS_E | US_B |
| AutoWelding | 0.31% | 0.39% | -1.08% | -0.24% | -1.22% | -0.95% | DS_D | US_B |
| BikeSparklers | -0.14% | 0.09% | -0.92% | -0.37% | -2.14% | -0.84% | DS_C | US_B |
| ShowGirl | 0.00% | 0.16% | -0.52% | -0.14% | -2.18% | -0.19% | DS_B | US_B |
| MagicHour | -0.01% | 0.22% | -0.98% | -0.51% | -1.67% | -0.73% | DS_D | US_B |
| WarmNight | -0.04% | 0.21% | -0.69% | -0.26% | -1.53% | 0.25% | DS_C | US_B |
| BalloonFestival | -1.32% | -0.24% | -2.45% | -1.68% | -2.89% | -1.18% | DS_F | US_A |
| Average | -0.34% | 0.04% | -1.36% | -0.63% | -1.90% | -0.85% |  |  |

# SEI message

The above results indicate that varying the chroma upsampling and the downsampling filters for various sequences can provide objective gains to the deltaE metric. An encoder can send the filter coefficients, and these could be used by the decoder to achieve better performance than choosing a default filter.

The HEVC specification defines the chroma resampling filter hint SEI message that contains signalling of upsampling and downsampling filters. These filters are defined to be such that when multiple upsampling and downsampling operations are done to the decoded pictures, the degradation in quality is minimized. The purpose of the filters described in this proposal is to provide the best performance when decoding the pictures. Since the chroma resampling filter hint SEI is defined with a different purpose, it may not be acceptable to use the same SEI. Moreover, the chroma resampling filter hint SEI message has persistence scope of the entire sequence; such a restriction is not needed for the purpose described in this document, and it may be better to allow the filters to be varied even within the sequence. Therefore, a new SEI message to signal the best upsampling filter is proposed. The signalling of the SEI message could be similar to that described in the chroma resampling filter hint SEI message, where the filter coefficients are explicitly signalled. As it is possible that the decoder side may choose an upsampling filter of its own, it is also proposed that the downsampling filter that are used by the encoder be signalled, so that the decoder side, if it chooses so, can make a decision on which filter would be best utilized by it.

# References

- Call for Evidence (CfE) for HDR and WCG Video Coding, MPEG document M36131, Switzerland, Feb.2015

- “On 4:4:4 to 4:2:0 conversion – Performance of downsampling and upsampling filters, and MinMax and Closed Loop Filtering,” A. M. Tourapis, Y. Su, D. Singer, J. Sole, D. Rusanovskyy, S. Lee, D. Bugdayci, A. Ramasubramonian, M. Karczewicz, C. Fogg, A. Duenas, F. Bossen, JCTVC-U0041, 19 – 26 June, 2015, Warsaw, PL.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).