| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0137 |
| --- | --- |

| Title: | On the use of AVC and HEVC "unspecified" NAL unit types in file formats and systems |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Qualcomm Incorporated David Singer Apple Inc. Gary J. Sullivan Microsoft Corp. | Tel: Email: | yekuiw@qti.qualcomm.com singer@apple.com garysull@microsoft.com |
| Source: | Qualcomm Incorporated, Apple Inc., and Microsoft Corporation |  |  |

_____________________________

# Abstract

This contribution contains the same content as in the MPEG input document m 36547. It is submitted for information to the JCT-VC.

This contribution discusses the handling of data associated with the "unspecified" AVC and HEVC NAL unit type codes, particularly in ‘transport environments’ such as the file formats of AVC, HEVC and their extensions, and in RTP.

The "unspecified" NAL unit types refer to those NAL unit types that are identified as "unspecified" in the AVC and HEVC specifications (i.e., type codes 0 and 24..31 for AVC and type codes 48..63 in HEVC in a recent edition of each). The type codes of these NAL unit types are, for example, used for the aggregator and the extractor structures that were introduced during the development of the SVC file format, and later included in the MVC file format, the (draft) MVC+D file format, the HEVC file format, and the (draft) layered HEVC file format. These (draft) standards are all documented in the latest integrated ISO/IEC 14496-15 draft specification in the MPEG output document N 15182.

This contribution reports that, unfortunately, some uses of these type codes may not conform to the formatting requirements that are specified for the proper formatting of NAL units in the AVC and HEVC video coding specifications (in regard to start code emulation, emulation prevention patterns, and trailing zeros). Thus, from the perspective of the video coding specifications, such strings of data associated with these type codes should not be considered part of an ordinary video elementary stream in the sense of conformance to the AVC and HEVC specification of the NAL unit format, and this may cause interoperability problems if such data is fed to an ordinary video decoder that has not been designed specifically for operation within the system environment in which this data is used. If feasible, a preferable solution would be for system specifications (esp. future ones) to be designed to obey the constraints specified in the video coding standards for NAL unit formatting.

# Problem

In the AVC and HEVC video coding specifications, several "unspecified" NAL unit types are allocated for use by external systems specifications such as file formats, MPEG-2 TS carriage formats, and RTP payload formats for AVC and HEVC (including their extensions). For example, AVC contains a NOTE that says, in part, that "NAL unit types 0 and 24..31 may be used as determined by the application" and HEVC similarly says "NAL unit types in the range of UNSPEC48..UNSPEC63 may be used as determined by the application".

Indeed, these NAL unit types have been used in some systems specifications, e.g., the aggregator and the extractor structures in the (draft) file formats of SVC, MVC, MVC+D, HEVC, and layered HEVC. However, it appears that these structures were not seen as relevant outside their particular environment (e.g., the file format) when the systems specification was developed, and hence the design rules for the payload formatting of NAL units were not thought relevant, and were not followed.

In particular, start-code emulation was not considered a problem in systems that do not use the byte stream format, and hence start-code emulation avoidance was not specified. Consequently, if such structures ‘leak out’ of the system/file-format context (e.g. are re-formatted into the byte stream format or otherwise passed to a video elementary stream decoder) and they happen to contain start code emulations or other patterns that are forbidden by the video coding standards, problems may arise:

- If the payload data contains a byte-aligned string that is equal to 0x000001, it could be interpreted as a start code that beings another NAL unit of data in decoders that operate using start code detection. For byte-alignment recovery, the strings 0x000000 and 0x000002 are also forbidden, as they can appear to be start codes at bit-shifted positions.

- If the external environment removes what appears to be a start-code emulation prevention byte (a byte equal to 0x03 that follow two bytes that are both equal to 0), the body of the payload content will be corrupted; this should not be a problem if this data is then merely discarded.

- The process that removes emulation prevention bytes from the incoming data could detect a format violation and initiate data corruption recovery operations if it sees a forbidden pattern such as a 0x000003 that is followed by some byte that's not in the range of 0x00 to 0x03, since that is clearly forbidden. Even though such a pattern isn't a start code, it isn't clear what the decoder should do when such a pattern is encountered, and a strict decoder or software process that checks the validity of its input data may consider the presence of forbidden data strings within NAL units to be an error.

- The NAL units may disobey some other rules (e.g. they may end with a zero byte) and again this may raise errors.

- Some decoders are designed such that they accept only the start-code based byte stream format as input, and require their input to be repackaged into that format if the incoming data arrives in some other form. The subsequent process will search within that data stream for start codes to identify the location of the NAL units within the data. If there is a start code pattern where there shouldn't be one, the decoder will detect extra NAL units and interpret whatever byte follows the start code prefix as containing a NAL unit type. That could result in the interpretation of junk data as a slice, or parameter set, or whatever other data is indicated by that type code.

Several of these issues may raise errors, with unknown effect. It is also possible that, upon detecting an obviously corrupted bitstream, a decoder may do something like ignore all subsequent data until it gets an IRAP picture to recover from the corruption.

Therefore, while it may be acceptable for systems specifications to contain such structures (using the "unspecified" NAL unit types without considering start code emulation prevention), they should require (using the "shall" language) the parser or depacketizer to never pass such structures outside of their context or otherwise pass such structures to decoders that are not designed with an awareness of the framing and formatting of these data segments.

However, in the families of file format for AVC and HEVC, this "shall" requirement has not been completely specified; there are places where these NAL units are permitted in streams that can be interpreted by software that does not detect and process/remove them.

# Discussions on ISO/IEC 14496-15

We suggest discussing how to specify this "shall" requirement in the families of file format for AVC and HEVC. The following aspects should be discussed and concluded:

- In the draft layered HEVC file format, this requirement should be specified, not just for the two "unspecified" NAL unit types used for aggregators and extractors, but for all "unspecified" NAL unit types, noting that in the future a data structure using an "unspecified" NAL unit types may follow the start code emulation prevention process and thus can be called real NAL units.

- For the file formats for SVC, MVC, and MVC+D, since the SVC and MVC file formats have been finalized already, a similar change, if to be made, has to be made as a corrigendum item.

- For the (plain) AVC file format, aggregators and extractors are not supported, but if the change is made there, then it applies to the entire AVC family of file formats.

- Similar as in 3) for the (plain) HEVC file format.

# On other systems specifications

The associated requirements should also be specified and clarified in the MPEG-2 TS carriage formats and the RTP payload formats for AVC and HEVC, and any other video coding specifications that use NAL-unit structured video bitstreams.

MPEG-2 Systems (PS and TS) operate using a byte stream format, where start code emulation prevention seems needed in any case. Accidental start code emulations within MPEG-2 systems streams could cause emulation of the systems-level start codes as well as emulation of video start codes, which could be a particularly serious problem in that environment.

It is noted that the associated requirements have been specified in the HEVC RTP payload format, but only for the used "unspecified" NAL unit types", not for all "unspecified" NAL unit types.

It is suggested that relevant experts should check and discuss in relevant groups what the requirements should be for all "unspecified" NAL unit types in the various environments.

# Other specifications

It has been noted that some may be using these NAL unit types in unconstrained environments or for bitstream extensions (e.g., using these NAL unit types for defining new types of slices). Clearly we have a possibility of conflict here. They may be ‘safe’ when confined, as intended, to a particular systems/transport environment (as a stream cannot be simultaneously a file format stream and an RTP stream), but they may not be safe when used for bitstream extensions intended for use in arbitrary system environments – for example, because such extended bitstreams could potentially be migrated to system environments that contain a conflicting use of the same NAL unit types.

We also point out the normative statements already declared in AVC ("NAL units that use nal_unit_type equal to 0 or in the range of 24..31, inclusive, shall not affect the decoding process specified in this Recommendation | International Standard” and HEVC ("NAL units that have nal_unit_type in the range of UNSPEC48..UNSPEC63, inclusive, for which semantics are not specified, shall not affect the decoding process specified in this Specification").

# On the HEVC specification

It is suggested that it would be useful to add some notes, e.g., to warn that the requirement as discussed above should be made in all external systems specifications.

We should also clarify the intention: that such data is not to be used by ordinary decoders designed according to the AVC/HEVC specification, and may not be suitable for use as a bitstream extension scheme.