| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0146 |
| --- | --- |

| Title: | CE1 Related: Combination of JCTVC-U0101 reverse scan and JCTVC-U0096 palette encoder improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Feng Zou Marta Karczewicz Wei Pu Rajan Joshi (Qualcomm) Xiaoyu Xiu Yan Ye Yuwen He (InterDigital) 5775 Morehouse Dr. San Diego, CA, USA 9710 Scranton Rd, San Diego, CA, USA | Tel: Email: | +1-858-658-5673 vseregin@qti.qualcomm.com Xiaoyu.Xiu@InterDigital.com |
| Source: | Qualcomm Inc. InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

This document proposes the combination of JCTVC-U0096 palette encoder improvement and JCTVC-U0101 palette reverse scan. The simulation results indicate that the performance of these two proposals are additive. Experimental results show that compared to SCM-4.0 anchor on 444 sequences, the proposed combination reportedly provides the average 1.5% and 2.1% BD-rate savings in AI  full frame IBC for the category text & graphics with motion, 1080p & 720p in both RGB and YCbCr color formats respectively. And 1.8% and 2.7% BD-rate savings in AI 4CTU IBC for the category text & graphics with motion, 1080p & 720p in both RGB and YCbCr color formats respectively. For 420 sequences, the corresponding BD-rate savings of the proposed method is 1.3% BD-rate savings for YUV, text & graphics with motion, 1080p & 720p.

# Introduction

In JCTVC-U0096[1],  palette encoder improvements are proposed including: 1) using squared error to derive the palette and for index assignment; 2) eliminating some of the palette entries if the related pixels can be represented by other palette indices; 3) merging of two consecutive runs and jointly optimizing the run values of index run and copy above run pair.

In JCTVC-U0101[2], one flag is signaled at CU level for palette mode to indicate if the palette indices of one palette-coded CU are scanned in reverse scan order or in original scan order

# Proposed techniques

The combination of JCTVC-U0096 and JCTVC-U0101 is proposed.

# Simulation results

The proposed scheme is tested using SCC common test conditions [3] based on SCM-4.0 [4]. Table 1 shows the results under the full frame IBC condition, while Table 2 shows the results under the 1×4 CTU IBC condition.

Table 1. Results of the proposed encoder with full frame IBC

Table 2. Results of the proposed encoder with 1x4 CTU IBC

# Reference

[1]. M. Karczewicz, F. Zou, V. Seregin, W. Pu and R. Joshi, “CE1 Related: Improved palette encoder”, JCTVC-U0096, Warsaw, Poland.

[2]. X. Xiu, Y. Ye and Y. He, “CE1 Test C.1: Improved palette run-length coding with palette reverse scan order”, JCTVC-U0101, Warsaw, Poland.

[3]. H. Yu, R. Cohen, K. Rapaka, J. Xu (editors), “Common Test Conditions for Screen Content Coding”, JCTVC-T1015, Geneva, Switzerland.

[4]. Available at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).