| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0148 |
| --- | --- |

| Title: | Non-CE1: Context model unification for palette run type flags in HEVC SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Wei Wang, Fanyi Duanmu, Haoping Yu 2330 Central Expy Santa Clara, CA 95050 USA | Tel: Email: | +1-408-330-5129 m.xu@huawei.com |
| Source: | Huawei USA R&D |  |  |

_____________________________

# Abstract

This contribution unifies the Sbac context model for syntax elements last_palette_run_type_flag and palette_run_type_flag in the palette mode in HEVC SCC extension. This method simplifies the context modeling as well as the overall codec complexity. No noticeable performance loss has been observed.

# Description

Color palette mode in HEVC SCC WD [1] and reference software SCM 4.0 is shown effective for typical screen content [2]. For each CU, it generally involves two main parts, i.e., color table processing and index map coding. In the current design of index map coding, index runs are coded in INDEX_MODE and COPY_ABOVE modes. With index grouping [3], the mode of the last run and other runs are represented by two individual syntax elements, i.e., last_palette_run_type_flag and palette_run_type_flag, and are associated with different context models.

Alternatively, this contribution proposes to unify the Sbac context model for these syntax elements. The context model of last_palette_run_type_flag is removed and the context model for palette_run_type_flag is used for the symbol last_palette_run_type_flag.

This unification simplifies the context modeling for palette mode, and does not introduce any new syntax element. Intuitively, the context model of palette_run_type_flag is a good estimation of the model of last_palette_run_type_flag.

# Simulations

Experiments are performed on top of the SCM 4.0 with settings using common test condition [4]. Results for 4:4:4 and 4:2:0 lossy and lossless configurations are summarized in Table 1 through Table 5.

Table 1  4:4:4 lossy coding efficiency of proposed method versus SCM 4.0, with FF IBC

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.3% | 0.0% | 0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.2% | 0.0% | 0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 2  4:4:4 lossy coding efficiency of proposed method versus SCM 4.0, with 1x4 IBC

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |

Table 3  4:4:4 lossless coding efficiency of proposed method versus SCM 4.0

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 100% |  |  |  | 101% |  |  |  |

Table 4  4:2:0 lossy coding efficiency of proposed method versus SCM 4.0

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.4% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | 0.4% |
| YUV, Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% | 0.0% | 0.3% | -0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |

Table 5  4:2:0 lossless coding efficiency of proposed method versus SCM 4.0

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 720p & 768p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  | 100% |  |  |  |

# Recommendation

It is recommended to adopt this unification into HEVC SCC WD and SCM 5.0, given that performance is well maintained without loss (instead, marginal gain is observed in some categories), and the context modeling is simplified.

# References

[1] R. Joshi, S. Liu, Y. Ye, “HEVC Screen Content Coding Draft Text 3”, Doc. JCT-VC T1005, Geneva, CH, Feb. 2015.

[2] M. Xu, W. Wang, Z. Ma, H. Yu, “AHG6: Information on the usage of IBC, Palette, and Intra Prediction in SCC”, Doc. JCT-VC T0194, Geneva, CH, Feb. 2015.

[3] M. Karczwicz, W. Pu, R. Joshi, V. Seregin, “Non CE1: Grouping Palette Indices At Front”, Doc. JCT-VC T0065, Geneva, CH, Feb. 2015.

[4] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC T1015, Geneva, CH, Feb. 2015.

# Patent rights declarations(s)

Huawei USA R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).