| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0149 |
| --- | --- |

| Title: | Non-CE1: Index Map Splitting (IMS) Mode for Palette Coding in HEVC SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Fanyi Duanmu, Wei Wang, Shervin Minaee, Haoping Yu 2330 Central Expy Santa Clara, CA95050 USA | Tel: Email: | +1-408-330-5129 m.xu@huawei.com |
| Source: | Huawei USA R&D |  |  |

_____________________________

# Abstract

This contribution presents an index map coding scheme by splitting the index map into four equal-sized square sub index maps (SIMs). These SIMs share the same palette table derived from the current CU and coded using existing index map coding approach (i.e., index mode and copy-above mode, respectively) with adaptive scanning order. This scheme suggests improving the system performance by simultaneously processing the SIMs in parallel. Currently this index map splitting mode is applied on palette coded CUs with dimension ranging from 32x32 down to 8x8. Experimental results demonstrate a negligible BD-Rate performance loss (i,e., lossy BD-Rate loss for 1080p text graphics content encoded using All intra configuration) but potential 4x processing efficiency improvement for index map encoding and decoding.

# Proposed Method

Color palette mode in HEVC SCC WD [1] and reference software SCM 4.0 is shown effective for typical screen content [2]. It applies to all CUs in a recursive quad-tree fashion. In practical codec system, palette and index map encoding and decoding impose a tight clock-cycle budget constraint. Therefore, this contribution proposes an index map splitting coding scheme to enable parallel processing.

More specifically, the proposed index map splitting mode includes

- splitting the index map into4 equal-sized square sub index maps (shown in Figure 1);

- maintaining the existing coding method for each sub index map (e.g., adaptive scan direction, index/copy above mode, etc.);

- adding an additional flag for each palette coded CU to indicate whether to partition current index map into smaller blocks.

Figure 1: Index Map splitting as 4 square paralleled Sub Index Map Unit

# Simulations

Experiments are performed on top of the SCM 4.0 with default settings using common test condition [3]. Results for 4:4:4 and 4:2:0 lossy and lossless are summarized below in Table 1 to Table 5.

Table 1 4:4:4 lossy Coding Efficiency of Proposed IMS vs. SCM 4.0 (IBC-FF)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% | -0.1% | -0.2% | -0.2% | 0.0% | 0.3% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.3% | 0.0% | -0.2% | 0.5% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 102% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |

Table 2 4:4:4 lossy Coding Efficiency of Proposed IMS vs. SCM 4.0 (IBC-4CTU)

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  | 99% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  | 101% |  |  |  |

Table 3  4:4:4 lossless Coding Efficiency of Proposed IMS w/ context model change vs. SCM 4.0

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p |  |  |  |
| RGB, text & graphics with motion,720p |  |  |  |
| RGB, mixed content, 1440p |  |  |  |
| RGB, mixed content, 1080p |  |  |  |
| RGB, Animation, 720p |  |  |  |
| RGB, camera captured, 1080p |  |  |  |
| YUV, text & graphics with motion, 1080p |  |  |  |
| YUV, text & graphics with motion,720p |  |  |  |
| YUV, mixed content, 1440p |  |  |  |
| YUV, mixed content, 1080p |  |  |  |
| YUV, Animation, 720p |  |  |  |
| YUV, camera captured, 1080p |  |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4  4:2:0 lossy Coding Efficiency of Proposed IMS w/ context model change vs. SCM 4.0

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p |  |  |  |
| RGB, text & graphics with motion,720p |  |  |  |
| RGB, mixed content, 1440p |  |  |  |
| RGB, mixed content, 1080p |  |  |  |
| RGB, Animation, 720p |  |  |  |
| RGB, camera captured, 1080p |  |  |  |
| YUV, text & graphics with motion, 1080p |  |  |  |
| YUV, text & graphics with motion,720p |  |  |  |
| YUV, mixed content, 1440p |  |  |  |
| YUV, mixed content, 1080p |  |  |  |
| YUV, Animation, 720p |  |  |  |
| YUV, camera captured, 1080p |  |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 5  4:2:0 lossless Coding Efficiency of Proposed IMS w/ context model change vs. SCM 4.0

# Recommendation

It is recommended to further test this feature in CE to enable the parallel sub index map processing for palette mode so as to improve the system-processing throughput.

Modified HEVC SCC WD is attached.

# Patent rights declaration(s)

Huawei USA R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] R. Joshi, S. Liu, Y. Ye, “HEVC Screen Content Coding Draft Text 3”, Doc. JCT-VC T1005, Geneva, CH, Feb. 2015.

[2] M. Xu, W. Wang, Z. Ma, H. Yu, “AHG6: Information on the usage of IBC, Palette, and Intra Prediction in SCC”, Doc. JCT-VC T0194, Geneva, CH, Feb. 2015.

[3] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC T1015, Geneva, CH, Feb. 2015.