| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0156 |
| --- | --- |

| Title: | On 4:2:0 chroma positioning for Rec. ITU-R BT.2020 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information and proposed liaison communication |  |  |
| Author(s) or Contact(s): | Chad Fogg, MovieLabs Gary Sullivan, Microsoft Alexis Tourapis, Apple | Tel: Email: | +1 425 703 5308 chad.fogg@gmail.com, garysull@microsoft.com, atourapis@apple.com |
| Source: | MovieLabs, Microsoft, and Apple |  |  |

_____________________________

This contributions reports that Rec. ITU-R BT.2020 appears to prescribe a different chroma positioning for 4:2:0 sampling than what has been previously assumed in video coding work. Based on the chroma sampling location type numbering scheme found in AVC and HEVC, it is reported that, rather than using location type #0 (which is considered the default in AVC and HEVC and is prescribed in MPEG-2) or type #1 (which is considered the default in H.261, H.263, and JFIF), it appears that (in its Table 5) BT.2020 specifies the use of a 4:2:0 chroma sampling lattice with chroma sampling location type #2.

This interpretation is confirmed by a white paper that was just published by the Blu-ray Disc Forum: "White Paper Blu-ray Disc Read-Only Format – Coding constraints on HEVC video streams for BD-ROM Version 3.0", June 2015 (clause 2.7.3 Location of chroma samples), available at http://www.blu-raydisc.com/en/Technical/TechnicalWhitePapers/General.aspx.

Two actions are suggested: 1) Sending a liaison letter to ITU-R to ask them to confirm that this is the intended interpretation and to ask why it was decided to differ from the recent past practice in this regard, and 2) Studying the potential impact of this on the colour gamut scalability design in the SHVC scalable extensions of HEVC.

Rec. ITU-T T.871 | ISO/IEC 10918-5