- Palette with Pixel Copy Decoder IC Implementation Reference Design

Preliminary Specification

1、General Decoder Spec

- Real time HEVC Palette with Pixel Copy(PPC) decoder，support 4096x2160@30 fps, working frequency 333MHz.

2、Block Diagram and Decoding flow of PPC Decoder

2.1 Block Diagram of PPC Decoder

As illustrated in Figure 1, the PPC Decoder has 2 pipeline stages: PPC Entropy Decoding Unit and PPC Reconstruction Unit.

The PPC Entropy Decoding Unit includes PPC Entropy controller which controls the CABAC decode engine to decode the bitstream and PPC CU Info SRAM which is used to store the CU’s syntax element. The PPC CU Info SRAM is reused with Intra/Inter/IBC Residual SRAM.

The PPC Reconstruction Unit includes PPC REC Controller, RefYUV SRAM Address Generator, Current RecYUV SRAM Address Generator and 2 SRAM to store Reference YUV and Current Reconstructed YUV. The RECYUV SRAM is reused with Intra/Inter/IBC which is the input SRAM for the following pipeline module Deblock Filter and SAO (Sample Adaptive Offset). RefYUV SRAM is also reused with IBC reconstruction unit.

Pipeline sram

Pipeline sram

PPC Reconstruction Unit

PPC Entropy Decoding Unit

Figure 1 Block Diagram of PPC Decoder

2.2 PPC decoding flow：

The PPC decoding process is split into PPC Entropy Decoding flow and PPC reconstruction flow, as illustrated in Figure 2a and Figure 2b.

Figure 2a. Entropy Decoding Flow                                        Figure 2b. REC Flow

The PPC coded CU is limited to 8x8, 16x16 and 32x32, both the entropy decoder and reconstruction is CU based.

2.2.1 PPC Entropy decoder

For a PPC coded CU, the PPC entropy controller control the CABAC decode engine to decode all related syntax elements. And then store those syntax elements into SRAM for future REC flow.

Key steps of PPC Entropy Decode Flow are specified as following:

- Decode PPC CU header info.

- If PLT (Palette) mode is present, then decode PLT reuse flag, PLT size, and PLT entries.

- Parse PLT string mode (PLTmode).

- If current string is pixel copy string, then parse string length and offsetX/offsetY.

- If current string is not pixel copy string, then parse PLT index. If this is unmatched pixel, then parse pixel value (YUV) from the bitstream, else get pixel value from PLT array and then parse index run.

- Pack PPC Pixel Info (PLTMode, OffsetX, OffsetY, PLTPixel and length) and store it into sram.

- Repeat step 3 to step 6, until entire CU is done.

The basic data element stored in PPC Info Sram is defined as table 1.

Table 1 Definition of PPC Info Sram

| bits | 35 | 34 | …… | 0 |
| --- | --- | --- | --- | --- |
|  | PPC Pixel Info |  |  |  |

IsPLTMode = PPCPixelInfo[35].

If IsPLTMode=1, then Length= PPCPixelInfo[33:24],PixelYUV= PPCPixelInfo[23:0];

Otherwise (IsPLTMode=0), then Length= PPCPixelInfo[25:16],OffsetY= PPCPixelInfo[15:9], OffsetX= PPCPixelInfo[8:0];

2.2.2 PPC YUV Reconstruction

2.2.2.1 The reconstruction flow

When input PPC Info sram and output REC YUV sram are ready for use, the PPC YUV reconstruction flow starts to work. The REC flow is specified as following steps:

- Fetch Pixel Info from sram

- If current string is PLT Mode, then current and next Length-1 REC Pixels value can be derived directly from Pixel Info at the same cycle.

- If current string is Pixel Copy Mode, then generate reference sram address, and fetch reference YUV from sram.

- Pack reconstructed pixel yuv with 4x4 tile format, generate reference YUV and reconstruction YUV sram address and store packed pixel into both ref and rec YUV sram.

- Repeat step 1 to 4, until whole CU is done.

2.2.2.2 Datapath and Throughput analysis for reconstruction flow

Figure 3a. Datapath for PLT Mode

Figure 3b. Datapath for PixelCopy Mode

The datapath for PLT mode is illustrated in figure 3a. The input data is read from Cu Info Sram, and pack Input pixel data into RecYUV register, after every 4 or 4x4 pixels are packed, then store it into both RefYUV and RecYUV sram. From the definition of Pixel Info Sram, the Length of valid pixel info range from 1 to 1024, so minimum throughput is 1 pixel per cycle and maxim throughput is 16 pixels per cycle in this mode (consider we just deal with 1 PixelInfo each cycle).

The datapath for PixelCopy mode is illustrated in figure 3b. To get Rec YUV, we need fetch ref yuv pixel from RefYUV sram firstly, and after every 4 or 4x4 pixel are packed into RecYUV register, then store them back into RefYUV and RecYUV sram. Since the ref YUV is stored in 4x4 tile format, so the minimum ref pixel number is 1, and maxim ref pixel number is 16. Hence the minimum throughput is 0.8 pixel per cycle, maximum throughput is 8 pixels per cycle.

2.2.2.3 Memory definition for reconstruction flow

To balance the throughput between horizontal and vertical, we arrange the pixel stored in the reference sram in 4x4 tile format. And this is consistent with output Rec YUV sram.

The definition of reference YUV sram is illustrated as Table 3.

Table 3a. pixel order in 4x4 tile format and block order in 32x32 DPU

| block[127:0]={pixel15[7:0],……,pixel1[7:0],pixel0[7:0]} |  |  |  | block order in 32x32(DPU, Decoder Processing Unit) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | blk0 | blk1 | …… | blk7 |
| 4 | 5 | 6 | 7 | blk8 | blk9 | …… | blk15 |
| 8 | 9 | 10 | 11 | …… | …… | …… | …… |
| 12 | 13 | 14 | 15 | blk48 | blk49 | …… | blk55 |
| pixel order in 4x4 block |  |  |  | blk56 | blk57 | …… | blk63 |

Table 3b. sram definition in 32x32 DPU

| sram definition for a DPU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| address | 0 | 1 | 2 | 3 | …… | 62 | 63 |
| data | blk0 | blk1 | blk2 | blk3 | …… | blk62 | blk63 |

Table 3.c sram definition of reference YUV (4 64x64 CTU)

| SRAM | CTU | SRAM | CTU | SRAM | CTU | SRAM | CTU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | DPU 0 (CTU 0) | 64 | DPU 1 (CTU 0) | 128 | DPU 2 (CTU 0) | 192 | DPU 3 (CTU 0) |
| …… |  | …… |  | …… |  | …… |  |
| 63 |  | 127 |  | 191 |  | 255 |  |
| 256 | DPU 0 (CTU 1) | 320 | DPU 1 (CTU 1) | 384 | DPU 2 (CTU 1) | 448 | DPU 3 (CTU 1) |
| …… |  | …… |  | …… |  | …… |  |
| 319 |  | 383 |  | 447 |  | 511 |  |
| 512 | DPU 0 (CTU 2) | 576 | DPU 1 (CTU 2) | 640 | DPU 2 (CTU 2) | 704 | DPU 3 (CTU 2) |
| …… |  | …… |  | …… |  | …… |  |
| 575 |  | 639 |  | 703 |  | 767 |  |
| 768 | DPU 0 (CTU 3) | 832 | DPU 1 (CTU 3) | 896 | DPU 2 (CTU 3) | 960 | DPU 3 (CTU 3) |
| …… |  | …… |  | …… |  | …… |  |
| 831 |  | 895 |  | 959 |  | 1023 |  |

3、Hardware implementation results

3.1 Results

The PPC Reconstruction module is written with verilog HDL and synthesized with 65nm lib, total equivalent gate count is 6.2K.

3.2 Part of RTL source code: RTL code of SRAM Write Address Generator

function [5:0] WriteAddrGen;

//description: generate sram write address for REC YUV

input addr_gen_en;

input [4:0] pos_x; //in unit of 1, Position in current CU

input [4:0] pos_y; //in unit of 1, Position in current CU

input [3:0] blk_x_in_ctu; //in unit of 4, Start Position in LCU

input [3:0] blk_y_in_ctu; //in unit of 4, Start Position in LCU

input [2:0] log2_lcu_size;

reg[5:0] rec_sram_ad;

begin

//step 1:

if(addr_gen_en) begin

if( log2_lcu_size == 3'd4 ) //CTU is 16x16

rec_sram_ad = ((blk_y_in_ctu[2:0] + pos_y[4:2])<<2) + (blk_x_in_ctu[2:0] + pos_x[4:2]);

else //CTU is 32x32 or 64x64

rec_sram_ad = ((blk_y_in_ctu[2:0] + pos_y[4:2])<<3) + (blk_x_in_ctu[2:0] + pos_x[4:2]);

end

else

rec_sram_ad = 6'd0;

//step 2:

WriteAddrGen = rec_sram_ad;

end

endfunction