| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0173 |
| --- | --- |

| Title: | Crosscheck of Non-CE1: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding (JCTVC-U0116) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, 1Chun-Chi Chen, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of enhancement to palette coding by Palette with Pixel Copy (PPC) coding (JCTVC-U0116). The software package provided by the proponent was tested under the screen content common test conditions specified in JCTVC-T1015 [1]. All BD-rate numbers match those provided by the proponents.

# Experimental Results

Table 1. BD-rate reduction and processing time ratio in lossy coding.

| full frame intra block copy search | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.8% | -4.1% | -4.1% | -2.5% | -2.7% | -2.7% | -2.0% | -2.0% | -1.9% |
| RGB, mixed content, 1440p & 1080p | -1.2% | -1.7% | -1.7% | -0.7% | -1.0% | -1.1% | 0.1% | -0.7% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -4.2% | -4.1% | -4.1% | -2.4% | -2.3% | -2.5% | -1.6% | -1.7% | -1.8% |
| YUV, mixed content, 1440p & 1080p | -1.5% | -2.0% | -2.3% | -0.9% | -1.4% | -1.9% | -0.3% | -1.2% | -1.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% | -0.1% | -0.3% | 0.1% | 0.0% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% |
| Enc Time[%] | 106% |  |  | 103% |  |  | 111% |  |  |
| Dec Time[%] | 101% |  |  | 105% |  |  | 104% |  |  |

| constrained intra block copy search (1x4 CTUs) | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -7.4% | -8.0% | -7.9% | -5.1% | -5.4% | -5.3% | -3.1% | -3.3% | -3.3% |
| RGB, mixed content, 1440p & 1080p | -2.4% | -3.2% | -3.2% | -1.2% | -1.9% | -1.9% | -0.4% | -1.2% | -1.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -7.7% | -7.7% | -7.9% | -5.0% | -4.9% | -5.0% | -2.6% | -2.5% | -2.8% |
| YUV, mixed content, 1440p & 1080p | -3.2% | -4.1% | -4.3% | -1.8% | -2.2% | -2.7% | -0.6% | -1.3% | -1.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 105% |  |  | 104% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 106% |  |  | 100% |  |  |

| full frame intra block copy search | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -1.1% | -1.1% | -1.1% | -0.7% | -0.6% | -0.7% | -0.5% | -0.6% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.5% | -0.7% | -0.2% | -0.4% | -0.3% | -0.2% | -0.6% | -0.9% |
| YUV, Animation, 720p | -0.2% | -0.3% | -0.3% | -0.1% | -0.2% | -0.3% | 0.0% | -0.2% | 0.2% |
| Enc Time[%] | 117% |  |  | 109% |  |  | 104% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  | 107% |  |  |

Table 2. BD-rate reduction and processing time ratio in lossless coding.

| full frame intra block copy search | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -4.8% | -5.6% | -14.9% | -0.9% | -2.8% | -3.5% | -10.2% | -0.4% | -2.6% | -2.8% | -6.6% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.8% | -0.9% | -1.6% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -5.2% | -6.0% | -15.4% | -1.0% | -2.8% | -3.9% | -10.5% | -0.3% | -2.6% | -3.1% | -7.5% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.9% | -1.0% | -1.7% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 116% |  |  |  | 98% |  |  |  | 105% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  | 102% |  |  |  |

| full frame intra block copy search | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| Text & graphics with motion, 720p | -1.1% | -1.3% | -3.4% | -0.1% | -0.5% | -0.7% | -1.4% | 0.0% | -0.4% | -0.4% | -1.1% | 0.0% |
| Mixed content, 480p | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 133% |  |  |  | 107% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 103% |  |  |  | 105% |  |  |  |

# References

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common Test Conditions for Screen Content Coding,” JCTVC-T1015, Geneva, Feb. 2015.