| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0175r1 |
| --- | --- |

| Title: | BoG report on worst case memory bandwidth assessment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Krishna Rapaka | Email: | krapaka@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document provides a report of the BoG on worst case memory bandwidth assessment.

# Introduction

A BoG was held on Saturday June 20th, 2015, 4:00 pm to 5.10 pm to discuss worst case memory bandwidth assessment methods related to CE2 Tests and chroma interpolation methods in JCTVC-U0080, JCTVC-U0103 and JCTVC-U0077. The mandates of the BoG were:

- Discuss and detail worst case memory bandwidth computation methodology.

- Prepare worst case memory BW per pixel and per picture for above methods and compare with HEVC v1 worst case.

A further BoG activity was held on Tuesday June 23th, 2015, 5:35 pm to 6.30 pm to discuss worst case memory bandwidth assessment methods related to CE2 Tests under 4:2:0 format and related combination of 5.2/5.3 CE2 Tests with chroma interpolation methods in JCTVC-U0080, JCTVC-U0103 and JCTVC-U0077.

# List of Proposals

| Doc No. | Title | Participants | Type |
| --- | --- | --- | --- |
| JCTVC-U0034 | CE2: Test 3 on intra block copy constraints on filtering (single-pass encoding decisions) | J. Lainema, M. M. Hannuksela (Nokia) | Proposal |
| JCTVC-U0053 | CE2 Test 2: Intra block copy constraints on filtering | G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon) | Proposal |
| JCTVC-U0078 | CE2 : Test 5 on intra block copy constraints on prediction | K. Rapaka, V. Seregin, R. Joshi, T. Hsieh, M. Karczewicz (Qualcomm) | Proposal |
| JCTVC-U0077 | On chroma motion vector derivation for intra block copy | X. Xu, S. Liu, S. Lei (MediaTek) | Proposal |
| JCTVC-U0080 | Intra block copy chroma interpolation for Non-444 | K. Rapaka, V. Seregin, R. Joshi, M. Karczewicz (Qualcomm) | Proposal |
| JCTVC-U0103 | On chroma derivation of intra block copy for non-444 video | X. Xiu, Y. Ye, Y. He (InterDigital) | Proposal |

# BoG Discussion and Suggestions

## Worst case memory bandwidth evaluation method

Both memory read and write are taken into account in the bandwidth calculation. For memory read, bandwidth is calculated as number of reference pixels fetched from the memory based on some memory patterns such as 4x2 etc. It is assumed that memory can be accessed and read based on the memory patterns, the chroma storage is not interleaved and only integer number of blocks can be fetched.

Let’s denote a block size to be compensated as M×N, memory pattern as m×n, and L is the interpolation filter length (L =8 for Luma, L=4 for chroma).

For each pixel within M×N block the number of pixels per fetched from the memory can be calculated as follows.

For memory write, both the filtered and unfiltered reconstructed pixels are written to memory for SCM 4.0.

In 4:2:0 chroma format, the chroma is considered to be interleaved as recommended in JCTVC-C007 and JCTVC-L0440. This methodology as described in these documents were used for AHGs in HEVC v1 and SHVC memory BW analysis. For each pixel within M×N luma block the number of pixels fetched from the memory for chroma is calculated as follows.

  - Memory analysis test conditions

### CE2 Test methods

The following analysis were performed in the BoG

- Analysis of increase in the memory bandwidth due to intra block copy.

| memory pattern | SCM4.0 Vs HEVC |
| --- | --- |
| 4x1 | 112.83% |
| 8x1 | 109.76% |
| 4x2 | 112.00% |
| 8x2 | 109.09% |
| 4x4 | 109.52% |

** These numbers assume that additional write of unfiltered IBC samples is fully aligned.

- Analysis of increase in the memory bandwidth due to intra block copy in 4:2:0.

| memory pattern | SCM4.0 Vs HEVC |
| --- | --- |
| 4x1 | 110.43% |
| 8x1 | 108.33% |
| 4x2 | 109.68% |
| 8x2 | 107.69% |
| 4x4 | 107.50% |

- Worst case per pixel memory assessment of CE2 Test under 4:4:4 chroma format and compare against HEVC v1.

| Memory Patterns | CE2 Tests (per pixel memory access) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test 2 | Test3 | Test5.1 | Test 5.2 | CE2Test5.3 | CE2 Test 5.4 |
| 4x1 | 23.375 | 23.375 | 23.375 | 21.75 | 21.75 | 23.375 |
| 8x1 | 30.750 | 30.750 | 30.750 | 28.25 | 28.25 | 30.750 |
| 4x2 | 25.000 | 25.000 | 25.000 | 23 | 23 | 25.000 |
| 8x2 | 33.000 | 33.000 | 33.000 | 30 | 30 | 33.000 |
| 4x4 | 31.500 | 31.500 | 31.500 | 28 | 28 | 31.500 |
|  | CE2 Tests Vs HEVC V1 |  |  |  |  |  |
|  | Test 2 vs HEVC v1 | Test 3 vs HEVC v1 | Test 5.1 vs HEVC v1 | Test 5.2 vs HEVC v1 | Test 5.3 vs HEVC v1 | Test 5.4 vs HEVC v1 |
| 4x1 | 100.00% | 100.00% | 100.00% | 93.05% | 93.05% | 100.00% |
| 8x1 | 100.00% | 100.00% | 100.00% | 91.87% | 91.87% | 100.00% |
| 4x2 | 100.00% | 100.00% | 100.00% | 92.00% | 92.00% | 100.00% |
| 8x2 | 100.00% | 100.00% | 100.00% | 90.91% | 90.91% | 100.00% |
| 4x4 | 100.00% | 100.00% | 100.00% | 88.89% | 88.89% | 100.00% |

** These numbers assume that 4x1, 4x2, 8x1, 8x2 and 4x4 memory patterns are used.

- Worst case per pixel memory assessment of CE2 Test under 4:2:0 chroma format and compare against HEVC v1.

### Chroma interpolation test methods

- Increase in the bandwidth due to chroma interpolation when only IBC is used and no temporal referencing.

| Memory Patterns | SCM 4.0 –IBC only (4:2:0) (per pixel memory access) | JCTVC-U0080/U0103/U077 - IBC only (4:2:0) (per pixel memory access) |
| --- | --- | --- |
| 4x1 | 7.5 | 9.5 |
| 8x1 | 9 | 12.25 |
| 4x2 | 9 | 10.75 |
| 8x2 | 11 | 14 |
| 4x4 | 10.5 | 13.5 |

- Worst case per pixel memory assessment of chroma interpolation methods (JCTVC-U0080/00103/U0077) under 4:2:0 chroma format and compared against SCM 4.0- 4:2:0.

| Memory Patterns | SCM 4.0 :- 4:2:0 (per pixel memory access) | JCTVC-U0080/U0103/U0077 (per pixel memory access) |
| --- | --- | --- |
| 4x1 | 15.875 | 15.875 |
| 8x1 | 19.500 | 19.500 |
| 4x2 | 17.000 | 17.000 |
| 8x2 | 21.000 | 21.000 |
| 4x4 | 21.500 | 21.500 |

- Compare worst case SCM 4.0 – 4:4:4 chroma methods vs chroma interpolation methods.

| Memory Patterns | SCM 4.0 - 4:4:4 (per pixel memory access) | SCM 4.0- 4:2:0 IBC (per pixel memory access) |
| --- | --- | --- |
| 4x1 | 26.375 | 15.875 |
| 8x1 | 33.75 | 19.500 |
| 4x2 | 28 | 17.000 |
| 8x2 | 36 | 21.000 |
| 4x4 | 34.5 | 21.500 |

The current specification defines the maximum picture size for a level number based on the luma samples only. Hence it is asserted that 4:4:4 is always the worst case and 4:2:0 does not violate the worst case condition. For example, for the same level corresponding to a 4:4:4 format picture, the same number of luma samples in 4:2:0 format picture would be present.

- Combination pare worst case SCM 4.0 – 4:4:4 chroma methods vs chroma interpolation methods.

### Suggested text for chroma interpolation method (as described in U0080)

The changes are marked in yellow.

8.5.3.2	Derivation process for motion vector components and reference indices

8.5.3.2.1	General

The variable offsetX and offsetY are derived as follows:

- if ChromaArrayType is not equal to 0,

offsetX = mvCLX[ 0 ] & 0x7 ? 2 : 0

offsetY = mvCLX[ 1 ] & 0x7 ? 2 : 0

- else

offsetX = 0

offsetY = 0

It is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2)  −  offsetX, (yPb + mvLX[ 1 ]  >>  2)  −  offsetY ) as inputs, the output shall be equal to TRUE.

- When the derivation process for z-scan order block availability as specified in clause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + (mvLX[ 0 ]  >>  2) + nPbW − 1  +  offsetX , yPb + (mvLX[ 1 ]  >>  2) + nPbH – 1 )  +  offsetY) as inputs, the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ]  >>  2 ) + nPbW + xB1  +  offsetX is less than or equal to 0.

- The value of ( mvLX[ 1 ]  >>  2 ) + nPbH + yB1  +  offsetY is less than or equal to 0.

- The following condition shall be true:

( xPb + ( mvLX[ 0 ]  >>  2 ) + nPbSw − 1 +  offsetX ) / CtbSizeY − xCurr / CtbSizeY  <=
		yCurr/CtbSizeY − ( yPb + ( mvLX[ 1 ]  >>  2 ) + nPbSh − 1 +  offsetY ) / CtbSizeY

##### 8.5.3.2.10 Derivation process for chroma motion vectors

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

……

For the derivation of the chroma motion vector mvCLX, the following applies:

–	If the picture with index refIdx from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-1)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-2)

–	Otherwise,

mvCLX[ 0 ] = ( ( mvLX[ 0 ] >> ( 1 + SubWidthC ) ) * 8	(8-xxx)

mvCLX[ 1 ] = ( ( mvLX[ 1 ] >> ( 1 + SubHeightC ) ) * 8	(8-xxx)

Further BoG Discussion (Tuesday 5.40 p.m):

It was asked if additional configurations of memory patterns (e.g different burst modes) needs to be included. After discussion it was concluded that same patterns (4x2 burstsize 1 and 8x2 burstsize 1) as used in HEVC v1 and SHVC methods analysis be considered.

After analysis of the memory numbers in this document, BoG concludes that the worst case memory BW for following cases does not cross the HEVC v1 worst case BW limit:

- 4 : 4 : 4 + CE2 Test 2,3, 5.1,5.2,5.3,5.4

- 4: 2: 0 + CE2 Test 2,3, 5.1,5.2,5.3,5.4

- 4:2 :0 + chroma interpolation methods in (U0077,U0080,U0103)

- 4:2 :0 + chroma interpolation methods in (U0077,U0080,U0103) + CE2 Tests 5.2, 5.3