| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0176 |
| --- | --- |

| Title: | Non-CE1: Joint proposal of JCTVC-U0169 and JCTVC-U0090 Item 3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou Vadim Seregin Marta Karczewicz Wei Pu Rajan Joshi Jing Ye, Jungsun Kim, Shan Liu, PoLin Lai, Weijia Zhu, Kai Zhang, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei 5775 Morehouse Dr. San Diego, CA, USA 2860 Junction Ave San Jose, CA, USA | Tel: Email: | +1-858-658-5673 fzou@qti.qualcomm.com {jing.ye, jungsun.kim, shan.liu, polin.lai, weijia.zhu, kai.zhang, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Qualcomm Inc. MediaTek Inc. |  |  |

_____________________________

# Abstract

This document presents a joint method for rice parameter derivation of numIndices in current palette coding. The results were tested on top of SCM4.0 + JCTVC-U0096 + JCTVC-U0086 Part1. Simulation results show that the proposed simplification has BD-rate savings of 0.1%, 0.1% for TGM RGB and YUV respectively in full frame IBC AI test condition. And 0.1%, 0.1% for TGM RGB and YUV respectively in 4CTU IBC AI test condition.

# Introduction

In the Warsaw meeting, JCTVC-U0096 (improved palette encoder) and JCTVC-U0086 (simplification of cRiceParam derivation for num_palette_indices_idc) are adopted.

# Proposed simplification

In the current SCM4.0 and specification, the rice parameter is derived as follows:

The variables cRiceParam is derived as follows:

cRiceParam = 2 + MaxPaletteIndex / 6

To simplify the derivation to avoid division operation at the decoder, the following rice parameter derication is proposed:

The variables cRiceParam is derived as follows:

cRiceParam = 3 + (MaxPaletteIndex >> 3)

# Simulation results

The proposed scheme is tested using SCC common test conditions [1] based on SCM-4.0[2] + JCTVC-U0096 + JCTVC-U0086 Part 1. Table 1 shows the results under the full frame IBC condition, while Table 2 shows the results under the 1×4 CTU IBC condition.

Table 1. Results of the proposed encoder with full frame IBC

Table 2. Results of the proposed encoder with 1x4 CTU IBC

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mediatek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Specification

9.3.3.15	 Binarization process for num_palette_indices_idc

Input to this process is a request for a binarization for the syntax element num_palette_indices_idc, MaxPaletteIndex, and nCbS.

Output of this process is the binarization of the syntax element.

The variables cRiceParam is derived as follows:

cRiceParam = 3 + (MaxPaletteIndex >>3)                                                                     	(9 30)

The variable cMax is derived from cRiceParam as:

cMax = 4  <<  cRiceParam  	                                                                                            (9 31)

# References

[1]. H. Yu, R. Cohen, K. Rapaka, J. Xu (editors), “Common Test Conditions for Screen Content Coding”, JCTVC-T1015, Geneva, Switzerland.Available at

[2]. https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0