| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0181 |
| --- | --- |

| Title: | On storage of filtered and unfiltered current decoded pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Shan Liu Chenghao Liu Shawmin Lei Ye-Kui Wang Krishna Rapaka Vadim Seregin | Email: | xiaozhong.xu@mediatek.com shan.liu@mediatek.com chenghao.liu@mediatek.com shawmin.lei@mediatek.com yekuiw@qti.qualcomm.com krapaka@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | MediaTek Inc., Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is a follow-up of JCTVC-U0100 after its review and revisit at this meeting, for further revisit of the topic (storage of filtered and unfiltered current decoded pictures).

The attachment contains a text implementation based on JCTVC-U038 for the following changes:

- When intra block copy (IBC) is enabled (i.e., curr_pic_as_ref_enabled_flag is equal to 1), both filtered and unfiltered versions of the current decoded picture are stored in the DPB at the same time, thus the memory requirement for decoded pictures remains the same regardless of whether IBC is enabled.

- Some asserted editorial improvements, including:

  - Move of DPB operations (storage, removal) from clause 8 to Annex C (and F), where such DPB operations are conventionally specified.

  - Add of a clarification that the storage of the decoded current picture should start after decoding the first block.

- A discussion of an alternative approach to maintain the same memory requirement for decoded pictures regardless of whether IBC is enabled, by not storing both filtered and unfiltered versions of the current decoded picture in the DPB at the same time, but changing the value of maxDpbPicBuf from 6 to either 5 or 6 − curr_pic_as_ref_enabled_flag.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).