### 7.3.2.2.3  Sequence parameter set screen content coding extension syntax

| sps_scc_extension( ) { | Descriptor |
| --- | --- |
| curr_pic_as_ref_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| if( palette_mode_enabled_flag ) { |  |
| palette_max_size | ue(v) |
| delta_palette_max_predictor_size | ue(v) |
| sps_ppc_reference_range_left_ctus | u(2) |
| } |  |
| motion_vector_resolution_control_idc | u(2) |
| intra_boundary_filtering_disabled_flag | u(1) |
| } |  |

### 7.3.8.8  Palette syntax

| palette_coding(x0, y0, nCbS) { | Descriptor |
| --- | --- |
| ppc_submode_flag | ae(v) |
| if(ppc_submode_flag) |  |
| ppc_palette_color_present_flag | ae(v) |
| if(ppc_palette_color_present_flag ¦ ¦ ! ppc_submode_flag){ |  |
| palettePredictionFinished = 0 |  |
| NumPredictedPaletteEntries = 0 |  |
| for(i = 0; i < PredictorPaletteSize && !palettePredictionFinished && NumPredictedPaletteEntries < palette_max_size; i++ ) { |  |
| palette_predictor_run | ue(v) |
| if( palette_predictor_run != 1) { |  |
| if( palette_predictor_run > 1 ) |  |
| i += palette_predictor_run − 1 |  |
| PalettePredictorEntryReuseFlag[ i ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |
| If( NumPredictedPaletteEntries < palette_max_size ) |  |
| num_signalled_palette_entries | ue(v) |
| numComps = ( ChromaArrayType = = 0 ) ? 1 : 3 |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| palette_entry | ae(v) |
| } |  |
| if(!ppc_submode_flag){ |  |
| if( CurrentPaletteSize != 0 ) |  |
| palette_escape_val_present_flag | ae(v) |
| if( palette_escape_val_present_flag ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_palette_abs | ae(v) |
| if( cu_qp_delta_palette_abs ) |  |
| cu_qp_delta_palette_sign_flag | ae(v) |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded ) { |  |
| cu_chroma_qp_palette_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0 ) |  |
| cu_chroma_qp_palette_offset_idx | ae(v) |
| } |  |
| } |  |
| } else { |  |
| palette_escape_val_present_flag | ae(v) |
| } |  |
| if( MaxPaletteIndex > 0) { |  |
| palette_transpose_flag | ae(v) |
| num_palette_indices_idc | ae(v) |
| for( i=0; i < NumPaletteIndices; i++ ) { |  |
| palette_index_idc | ae(v) |
| PaletteIndexIdc[ i ] = palette_index_idc |  |
| } |  |
| last_palette_run_type_flag | ae(v) |
| } else { |  |
| if(ppc_submode_flag) |  |
| palette_transpose_flag | ae(v) |
| } |  |
| CurrNumIndices = 0 |  |
| PaletteScanPos = 0 |  |
| while( PaletteScanPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ PaletteScanPos ][ 0 ] |  |
| yC = y0 + travScan[ PaletteScanPos ][ 1 ] |  |
| if(ppc_submode_flag){ |  |
| ppc_coding(x0, y0, nCbS ) |  |
| } else { |  |
| if( PaletteScanPos > 0) { |  |
| xcPrev = x0 + travScan[ PaletteScanPos − 1 ][ 0 ] |  |
| ………… |  |
| runPos++ |  |
| PaletteScanPos++ |  |
| } |  |
| } |  |
| } |  |
| if(!ppc_submode_flag &&palette_escape_val_present_flag ) { |  |
| ………… |  |
| } |  |
| } |  |
| ppc_coding ( x0, y0, nCbS ) { | Descriptor |
| if(ppc_palette_color_present_flag ¦ ¦ palette_escape_val_present_flag) |  |
| ppc_pixel_string_flag[ xC ][ yC ] | ae(v) |
| if(ppc_pixel_string_flag){ |  |
| ppc_offset_is_above_flag | ae(v) |
| if(!ppc_offset_is_above_flag){ |  |
| ppc_offsetY_zero_flag | ae(v) |
| if(!ppc_offsetY_zero_flag){ |  |
| if(!palette_transpose_flag && ((yC & 0x1) = =1)) |  |
| ppc_offsetX_sign | u(1) |
| ppc_offsetX_abs_minus1 | ae(v) |
| } else { |  |
| ppc_offsetY_sign | u(1) |
| ppc_offsetY_abs_minus1 | ae(v) |
| if(ppc_offsetY_sign) |  |
| if(palette_transpose_flag && ((xC & 0x1) = =1)) |  |
| ppc_offsetX_abs | ae(v) |
| else |  |
| ppc_offsetX_abs_minus1 | ae(v) |
| else{ |  |
| ppc_offsetX_zero_flag | ae(v) |
| if (ppc_offsetX_zero_flag) { |  |
| ppc_offsetX_sign | u(1) |
| ppc_offsetX_abs_minus1 | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| maxPaletteRun = nCbS * nCbS –PaletteScanPos– 1 |  |
| if( maxPaletteRun > 0 ) { |  |
| palette_run_msb_id_plus1 | ae(v) |
| if( palette_run_msb_id_plus1 > 1 ) |  |
| palette_run_refinement_bits | ae(v) |
| } |  |
| pixelCopyPos = 0 |  |
| while (pixelCopyPos <= paletteRun) { |  |
| xR = x0 + travScan[ PaletteScanPos ][ 0 ] |  |
| yR = y0 + travScan[ PaletteScanPos ][ 1 ] |  |
| ppcParaFlag [xR][yR] = 1 |  |
| ppcParaVal[0] [xR][yR] = offsetX |  |
| ppcParaVal[1] [xR][yR] = offsetY |  |
| pixelCopyPos ++ |  |
| PaletteScanPos ++ |  |
| } |  |
| } |  |
| else{ |  |
| ppcParaFlag [xC][yC] = 0 |  |
| if(palette_escape_val_present_flag && CurrPaletteIndex = = MaxPaletteIndex){ |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ){ |  |
| palette_escape_val | ae(v) |
| ppcParaVal [cIdx] [xC][yC] = palette_escape_val |  |
| } |  |
| PaletteScanPos++ |  |
| } |  |
| else{ |  |
| maxPaletteRun = nCbS * nCbS –PaletteScanPos– 1 |  |
| if( maxPaletteRun > 0 ) { |  |
| palette_run_msb_id_plus1 | ae(v) |
| if( palette_run_msb_id_plus1 > 1 ) |  |
| palette_run_refinement_bits | ae(v) |
| } |  |
| runPos = 0 |  |
| while (runPos <= paletteRun) { |  |
| xR = x0 + travScan[ PaletteScanPos ][ 0 ] |  |
| yR = y0 + travScan[ PaletteScanPos ][ 1 ] |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| ppcParaVal [cIdx] [xR][yR] = CurrentPaletteEntries[cIdx][CurrPaletteIndex] |  |
| runPos ++ |  |
| PaletteScanPos ++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

### 7.4.3.2.1  General sequence parameter set RBSP semantic

sps_ppc_reference_range_left_ctus specifies the left reference range of Palette with Pixel Copy (PPC) sub-mode of palette mode in units of CTUs and shall be in the range of 0 to 3, inclusive.

- 7.4.9.6  Palette mode semantics

……

ppc_submode_flag equal to 1 specifies that the current coding unit is coded by Palette with Pixel Copy (PPC) sub-mode. ppc_submode_flag equal to 0 specifies that the cu is coded by non-PPC palette mode. In PPC sub-mode, a string is either a palette color string or a pixel string.

ppc_palette_color_present_flag equal to 1 specifies that the current coding unit has at least one palette color string. ppc_palette_color_present_flag equal to 0 specifies that there are no palette color string in the current coding unit. When ppc_palette_color_present_flag is not present, it is inferred to be equal to 0.

ppc_pixel_string_flag equal to 1 specifies a pixel string in the current coding unit. ppc_pixel_string_flag equal to 0 specifies either a palette color string or an unmatched pixel, i.e. an escape coded pixel. When ppc_pixel_string_flag is not present, it is inferred to be equal to 1. When ppc_pixel_string_flag equal to 0, the same procedure to derive CurrPaletteIndex when palette_run_type_flag[ xC ][ yC ] equal to COPY_INDEX_MODE is applied to derive CurrPaletteIndex.

ppc_offset_is_above_flag equal to 1 specifies (offsetX, offsetY)=(0,1) when palette_transpose_flag equal to 0 or (offsetX, offsetY)=(1,0) when palette_transpose_flag equal to 1, where (offsetX, offsetY) specifies the offset between the location (xC, yC) of a current sample and the location (xC − offsetX, yC − offsetY) of its reference sample.

ppc_offsetY_zero_flag specifies whether the value of an offsetY is equal to 0. ppc_offsetY_zero_flag equal to 0 specifies the value of an offsetY is 0. ppc_offsetY_zero_flag equal to 1 specifies the value of an offsetY is not 0.

ppc_offsetX_sign specifies the sign of an offsetX as follows:

- If ppc_offsetX_sign is equal to 0, the corresponding offsetX has a positive value.

- Otherwise (ppc_offsetX_sign is equal to 1), the corresponding offsetX has a negative value.

- When not present, the value of ppc_offsetX_sign is inferred to be equal to 0.

ppc_offsetY_sign specifies the sign of an offsetY as follows:

- If ppc_offsetY_sign is equal to 0, the corresponding offsetY has a positive value.

- Otherwise (ppc_offsetY_sign is equal to 1), the corresponding offsetY has a negative value.

- When not present, the value of ppc_offsetY_sign is inferred to be equal to 0.

ppc_offsetY_abs_minus1+1 specifies the absolute value of an offsetY.

ppc_offsetX_abs_minus1 +1 specifies the absolute value of an offsetX.

ppc_offsetX_abs specifies the absolute value of an offsetX.

ppc_offsetX_zero_flag specifies whether the value of an offsetX is equal to 0. ppc_offsetY_zero_flag equal to 0 specifies the value of an offsetX is 0.

The variable ppcParaFlag[xC][yC] specifies the copy modes in PPC sub-mode. ppcParaFlag[xC][yC] equal to 0 specifies either a palette color string or an unmatched pixel. ppcParaFlag[xC][yC] equal to 1 specifies a pixel string. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the current cu.

When ppcParaFlag[ xC ][ yC ] equal to 1, the variable ppcParaVal[0][ xC ][ yC ] specifies an offsetX that specifies the horizontal offset between the location ( xC , yC ) of current pixel to be decoded/constructed and the corresponding reference pixel location in the reconstructed picture buffer.

When ppcParaFlag[ xC ][ yC ] equal to 1, the variable ppcParaVal[1][ xC ][ yC ] specifies an offsetY that specifies the vertical offset between the location ( xC , yC ) of current pixel to be decoded/constructed and the corresponding reference pixel location in the reconstructed picture buffer.

When ppcParaFlag[ xC ][ yC ] equal to 1, the variable offsetX and offsetY are derived as follows:

if(ppc_offset_is_above_flag){

if(palette_transpose_flag = = 0){

offsetX = 0

offsetY = 1

}

else {

offsetX = 1

offsetY = 0

}

}

else	{

offsetY = ppc_offsetY_zero_flag?( (1–2*ppc_offsetY_sign)*( ppc_offsetY_abs_minus1+1)):0

if(offsetY<0 && palette_transpose_flag && ((xC & 0x1) = =1)) {

offsetX = ppc_offsetX_zero_flag?( (1–2*ppc_offsetX_sign)*(ppc_offsetX_abs):0

}

else {

offsetX = ppc_offsetX_zero_flag?( (1–2*ppc_offsetX_sign)*( ppc_offsetX_abs_minus1+1)):0

}

}

Decoding process for coding units coded in Palette with Pixel Copy mode ()

Inputs to this process are:

–	a location ( xCb, yCb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable cIdx specifying the colour component of the current block,

- two variables nCbSX and nCbSY specifying the width and height of the current block, respectively,

- the reconstructed picture sample buffer S (SL, SCb, SCr when cIdx=0,1,2, respectively) of the current picture,

Depending on the value of cIdx, the variables nSubWidth and nSubHeight are derived as follows:

–	If cIdx is equal to 0, nSubWidth is set to 1 and nSubHeight is set to 1,

–	Otherwise, nSubWidth is set to SubWidthC and nSubHeight is set to SubHeightC.

Output of this process is part of the reconstructed picture sample buffer S[xCb/nSubWidth + x][yCb/nSubHeight + y], with x = 0..nCbSX − 1,  y = 0..nCbSY − 1, specifying reconstructed sample values for the current block.

When nSubWidth = nSubHeight = 1, the reconstructed picture sample buffer S[xCb + x][yCb + y] at location ( xCb, yCb ) is derived as follows:

For PaletteScanPos = 0……nCbSX* nCbSY − 1,

xR = xCb + travScan[PaletteScanPos*nSubWidth][palette_transpose_flag],

yR = yCb + travScan[PaletteScanPos][!palette_transpose_flag],

if ppcParaFlag [xR][ yR] is equal to 0, the following applies:

S[xR][yR] = ppcParaVal[ cIdx ] [ xR ][ yR ],

if ppcParaFlag [xR][ yR] is equal to 1, the following applies:

S[xR][yR] = S[xR−ppcParaVal[ 0 ][ xR ][ yR ]][yR−ppcParaVal[ 1 ][ xR ][ yR ]].

    - Binarization process

- 9.3.2.1 General

Table 9-4 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| palette_coding( ) | ppc_submode_flag | Table 1 | 0 | 1 | 2 |
|  | ppc_palette_color_present_flag | Table 2 | 0 | 1 | 2 |
|  | ppc_pixel_string_flag | Table 3 | 0 | 1 | 2 |
|  | ppc_offset_is_above_flag | Table 4 | 0 | 1 | 2 |
|  | ppc_offsetY_zero_flag | Table 5 | 0 | 1 | 2 |
|  | ppc_offsetX_zero_flag | Table 6 | 0 | 1 | 2 |

- Values of variable initValue for ppc_submode_flag

| Initialization variable | ppc_submode_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

- Values of variable initValue for ppc_palette_color_present_flag

| Initialization variable | ppc_palette_color_present_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 139 | 139 | 139 |

- Values of variable initValue for ppc_pixel_string_flag

| Initialization variable | ppc_pixel_string_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

- Values of variable initValue for ppc_offset_is_above_flag

| Initialization variable | ppc_offset_is_above_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

- Values of variable initValue for ppc_offsetY_zero_flag

| Initialization variable | ppc_offsetY_zero_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

- Values of variable initValue for ppc_offsetX_zero_flag

| Initialization variable | ppc_offsetX_zero_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

#### 9.3.3.1 General

| Table 9-42 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| palette_coding() | ppc_submode_flag | FL | cMax = 1 |
|  | ppc_palette_color_present_flag | FL | cMax = 1 |
|  | ppc_pixel_string_flag | FL | cMax = 1 |
|  | ppc_offset_is_above_flag | FL | cMax = 1 |
|  | ppc_offsetY_zero_flag | FL | cMax = 1 |
|  | ppc_offsetX_zero_flag | FL | cMax = 1 |
|  | ppc_offsetY_sign | FL | cMax = 1 |
|  | ppc_offsetY_abs_minus1 | 9.3.1.17 |  |
|  | ppc_offsetX_sign | FL | cMax = 1 |
|  | ppc_offsetX_abs_minus1 | 9.3.1.18 |  |
|  | ppc_offsetX_abs | 9.3.1.19 |  |
|  | palette_escape_val (when ppc_submode_flag = = 1) | FL | cMax = cIdx = = 0 ? ( (1<<BitDepthY) − 1 ) : ( (1<<BitDepthC) − 1 ) |

#### Truncated 2-th order Exp-Golomb binarization process

Inputs to this process is a request for a truncated 2-th order Exp-Golomb (TEG2) binarization for a syntax element with value symbolVal and cMax.

Output of this process is the TEG2 binarization associating each value symbolVal with a corresponding bin string.

Process TB(symbolVal, CMax) specifies the truncated Binary(TB) binarization process for a syntax element with value symbolVal and CMax. See 9.3.3.6 for more details of Truncated Binary(TB) binarization process.

Process FL(CMax) specifies the Fixed-length(FL) binarization process for a syntax element with value CMax. See 9.3.3.5 for more details of Fixed-length(FL) binarization process.

uiCount = 2

truncatedValue = 1 << uiCount

while( symbolVal >= (1<<uiCount)){

put( 1 )

symbolVal = symbolVal -( 1 << uiCount  )

uiCount ++

truncatedValue +=1  <<  uiCount

}

if(cMax >=truncatedValue )

put( 0 )

curValue = truncatedValue -(1<<uiCount)

if(cMax <truncatedValue )

TB(symbolVal, cMax-curValue+1)

else

FL(symbolVal,1<<uiCount-1)

#### Binarization process for ppc_offsetY_abs_minus1

Input to this process is a request for a binarization for the syntax element ppc_offsetY_abs_minus1, OffsetYPMax, OffsetYNMax, CuStartYInPic.

Output of this process is the binarization of the syntax element.

The variable CuStartYInPic specify the vertical offset of the current coding unit relative to the top-left pixel of the picture.

The variable OffsetYPMax specifies the maxmum offsetY value of the current pixel to be decoded/constructed in the current coding unit when ppc_offsetY_sign=0 and is derived as follows:

LcuStartYInPic = CuStartYInPic & (CtbSizeY-1)

OffsetYPMax = CuStartYInPic + travScan[PaletteScanPos][1]- LcuStartYInPic

The variable OffsetYNMax specifies the maxmum offsetY value of the current pixel to be decoded/constructed in the current coding unit when ppc_offsetY_sign=1 and is derived as follows:

OffsetYNMax = LcuStartYInPic + CtbSizeY –CuStartYInPic - travScan[PaletteScanPos][1]

The binarization for the syntax element ppc_offsetY_abs_minus1 is specified in Table 9-3.

| Table 9-3– Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| palette_coding() | ppc_offsetY_abs_minus1 | TEG2 | cMax = OffsetYPMax, ppc_offsetY_sign=0 |
|  |  | TEG2 | cMax = OffsetYNMax, ppc_offsetY_sign=1 |

#### Binarization process for ppc_offsetX_abs_minus1

Input to this process is a request for a binarization for the syntax element ppc_offsetX_abs_minus1, OffsetXPMax, OffsetXNMax, CuStartXInPic, SearchLeftCTUs.

Output of this process is the binarization of the syntax element.

The variable CuStartXInPic specify the horizontal offset of the current coding unit relative to the top-left pixel of the picture.

The variable SearchLeftCTUs specify the left search CTUs of the current coding unit and is derived as follows:

CurrLeftCTUs = CuStartXInPic/CtbSizeY

SearchLeftCTUs = Min(CurrLeftCTUs, sps_ppc_reference_range_left_ctus)

The variable OffsetXPMax specifies the maxmum offsetX value of the current pixel to be decoded/constructed in the current coding unit when ppc_offsetX_sign=0 and is derived as follows:

LcuStartXInPic = CuStartXInPic &(CtbSizeY-1)

OffsetXPMax = CuStartXInPic + travScan[PaletteScanPos][0]- LcuStartXInPic+ SearchLeftCTUs * CtbSizeY

The variable OffsetXNMax specifies the maxmum offsetX value of the current pixel to be decoded/constructed in the current coding unit when ppc_offsetX_sign=1 and is derived as follows:

OffsetXNMax =  LcuStartXInPic + CtbSizeY - CuStartXInPic - travScan[PaletteScanPos][0]

The binarization for the syntax element ppc_offsetX_abs_minus1 is specified in Table 9-4.

| Table 9-4– Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| palette_coding() | ppc_offsetX_abs_minus1 | TEG2 | cMax = OffsetXPMax, ppc_offsetX_sign=0 |
|  |  | TEG2 | cMax = OffsetXNMax, ppc_offsetX_sign=1 |

#### Binarization process for ppc_offsetX_abs

Input to this process is a request for a binarization for the syntax element ppc_offsetX_abs, OffsetXPMax.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element ppc_offsetX_abs is specified in Table 9-5.

| Table 9-5– Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| palette_coding() | ppc_offsetX_abs | TEG2 | cMax = OffsetXPMax , ppc_offsetX_sign=0, palette_transpose_flag=1 |

#### General

Table 9-47– Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| ppc_submode_flag | 0 | na | na | na | na | na |
| ppc_palette_color_present_flag | 0 | na | na | na | na | na |
| ppc_pixel_string_flag | 0 | na | na | na | na | na |
| ppc_offset_is_above_flag | 0 | na | na | na | na | na |
| ppc_offsetY_zero_flag | 0 | na | na | na | na | na |
| ppc_offsetX_zero_flag | 0 | na | na | na | na | na |
| ppc_offsetY_sign | bypass | na | na | na | na | na |
| ppc_offsetY_abs_minus1 | bypass | na | na | na | na | na |
| ppc_offsetX_sign | bypass | na | na | na | na | na |
| ppc_offsetX_abs_minus1 | bypass | na | na | na | na | na |
| ppc_offsetX_abs | bypass | na | na | na | na | na |